# Optimizing Genesis Native Library Applications

This document will help you estimate the expected performance of Genesis on a particular application, and help you optimize your code so that you reach the expected performance. Although most of the general advice given here is likely to remain valid, particular details of individual functions may change with each release of the Genesis software, since improvements may be made, and/or new functions added. You should always look for the most recent version of this document.

The first step in writing an efficient application is to pick the functions that can do the job as fast as possible. To do this you need to know the execution time of individual functions. This is not as simple as it sounds, because the execution time of each processing function varies with the size and data type of the images processed, and can be affected by many other parameters. The execution time of some functions is even dependent on the actual content of the image. To address these issues, several types of information are provided:

- Actual benchmarks for each function with common data types.

- Simple rules for estimating the performance of cases not listed.

- A more-detailed discussion of functions whose performance is more difficult to estimate.

There are also some general points, described below, that can be applied to all processing functions. These will help you avoid some of the basic mistakes which can make your code inefficient.

## Overheads

All functions have a fixed overhead, which means that they become less efficient when operating on small images. The overhead is not exactly the same for all functions, but it is typically about 0.5 ms for each function call. This means that you cannot process an image in less time, no matter how small the image is. (Note that this overhead applies to processing functions; simple control functions have less overhead, and are discussed later.) This overhead has several implications:

When adjusting a known benchmark to a different image size, you cannot simply scale it according to the number of pixels in the image. You should first subtract the overhead, then scale the benchmark, then add the overhead again. For example, if a function with a 0.5 ms overhead takes a total of 2.5 ms for a 512x512 image, it will take 1.0 ms for a 256x256 image, and 8.5 ms for a 1024x1024 image.

When your application only requires that one or more regions of interest (ROIs) be processed, it might actually turn out to be more efficient to process the whole image than to define and process several ROIs separately. If your images themselves are quite small, you should consider packing several into a larger buffer and processing them all at once.

Simple asynchronous control functions (such as imBufPutField()), and other functions that don’t operate on images, have a lower overhead. It is typically 0.2 ms for these functions. However, synchronous functions (such as imBufGetField() or imBufChild()) are slower, and take about 0.5 ms.

You should try to avoid synchronous functions as much as possible within time-critical loops. Allocate all buffers outside of loops unless you have no choice. When using synchronous functions to read back processing results, use as few calls as possible. Sometimes you have the option of reading back results individually using one function call each, or reading a whole group of results at once. The latter method will be more efficient, and is particularly important for blob analysis and pattern matching where many results are usually produced.

## Estimating missing benchmarks

When the particular case you need is supported by the library but not listed in the benchmark table, you can usually estimate its performance using a few simple rules. This document does not describe how to estimate the performance of functions that are not in the standard library, but that you might be interested in implementing as a custom function if the performance justifies it. (To implement custom functions on the C80 you need the Genesis Native Library Developer’s Toolkit (DTK), as well as the appropriate code development tools from Texas Instruments. This is a fairly complex task, but can often provide a significant performance increase if several standard library functions can be combined into a single custom function.)

### I/O bound functions

A function is I/O bound when performance is limited by the speed at which it can access data in memory. The processors will be idle some of the time while they are waiting for data to be transferred on-chip. It is useful here to know that processing functions using the PPs always work by transferring the image from external memory into on-chip memory, a block at a time. Each block is processed in on-chip memory, then the results are transferred back to external memory. Processing and I/O can be overlapped, so the PPs are not kept waiting for data to process, unless they process faster than data can be transferred (then the operation is said to be I/O bound).

If a function is described as I/O bound in the benchmark table, or if the I/O figure given is close to 300 MB/s which indicates that the function is I/O bound, it is simple to estimate benchmarks for cases not listed explicitly. You should simply consider the total number of bytes of I/O per pixel (counting all source and destination buffers), and scale the benchmark accordingly. For the most accurate estimate you should also consider the function overhead. For example, consider the I/O bound operations of imIntDyadic(). Given the 8-bit case, let’s try to estimate the performance for the 16-bit case. The total I/O in the 8-bit case is 3 bytes/pixel, and it is 6 bytes/pixel in the 16-bit case (exactly twice as much). The function overhead can be calculated as 0.4 ms from the first two columns. The calculated performance for the 16-bit case is therefore (3.0–0.4)×2 + 0.4 = 5.6 ms, which is in agreement with the measured value. If, say, the two source buffers were 8-bit and the destination were 16-bit, the total I/O would be 4 bytes/pixel and you would expect the performance to lie somewhere between the listed 8- and 16-bit cases. In fact it should be (3.0–0.4)×4/3 + 0.4 = 3.9 ms.

### Compute bound functions

Most compute bound functions are listed explicitly in the benchmark table, because you would need to know how the operation is coded in PP assembly language in order to estimate the performance. However, this doesn’t apply to some functions. In particular, neighbourhood operations which support different sized kernels (such as convolution or morphology) usually take a certain amount of time for each kernel element. Therefore you should look up the benchmark for a kernel similar in size to yours, then scale the benchmark according to the number of kernel values. For example, the C80 processing rate for a general 5x5 16-bit convolution (25 kernel values) is quoted as 7.65 MPix/s. The estimated performance for a 7x7 convolution (49 kernel values) would therefore be 7.65×25/49 = 3.90 MPix/s, which agrees very well with the measured value. This rule normally works well, but beware of particular kernels that might be carried out with optimized PP code and give quite different performance from the general case. For example, kernels whose values are all the same (or where only the centre value is different) are faster on the C80 than kernels with completely arbitrary values. The situation with the NOA is similar but the rules are different. Pixel type (8- or 16-bit), coefficient type (8- or 16-bit), and kernel symmetry (horizontal and/or vertical) all affect performance considerably on any given size of kernel. This is discussed in more detail later.

## Parallelism

There are times when you could execute several operations in parallel, by sending the commands to different threads, but this will not always reduce the total execution time. When you send commands to different processing nodes, they will truly run in parallel and they will have no impact on each other as long as each node processes buffers allocated in its own memory. However, when only one node is involved, things are more complicated.

First, to benefit from parallelism, each function must use different resources. For example, there is no point in executing two functions at the same time if they both use the PPs. If both try to allocate all the PPs (which is the default behaviour) they will simply execute serially, since one function will have to wait for the other to finish completely before it is granted access to the PPs. If you use imThrControl() to limit each thread to, say, two PPs, both functions will run at the same time but each at only half speed (so there is no net gain in performance). Note that this is true whether the functions are compute bound (so each runs at half speed because it only has half of the processors) or I/O bound (where each runs at half speed because it only gets access to memory half of the time).

If two functions use different resources (for example the PPs and MP, or PPs and NOA) there will still be no advantage to executing them in parallel if both are I/O bound, since the available memory bandwidth is the limiting factor. However, if one or both functions are not I/O bound, there should be an advantage to running then in parallel. One common case involves processing an image while copying the previously-processed image to the display. When this is done serially (i.e. processing and copy commands are send to the same thread), the transfer time is simply added to the processing time. When this is done is parallel (i.e. the copy command is sent to another thread, with the proper synchronization), the transfer begins as soon as processing completes, but it does not prevent processing of the next image from starting. This way some or all of the transfer time is hidden (depending on how I/O intensive the processing function is). For more details on how to implement this, look at the Native Library examples.

It is also important to realize that using too many threads, and especially the extra synchronization functions that more threads usually implies, can make your application less efficient. There are certainly big performance gains to be had sometimes by using extra threads to exploit parallelism in your application, but you should generally try to solve the problem using the smallest number of threads that gives you the parallelism you need.

## Benchmarks

Below is a table of actual benchmarks for 512x512 images, using a 50 MHz C80 with NOA. To keep the table to a reasonable size, some cases have been omitted if they can easily be estimated from other cases using the rules given elsewhere in this document. If a function has options that are not mentioned in the table (e.g. replace vs. transparent overscan), assume that the option makes no significant difference to the timing.

The table gives actual execution time (in milliseconds), processing rate (in MPixels/second), and the memory bandwidth required (in MBytes/second). The latter figure is useful when considering the impact of concurrent operations such as grabbing and transfers to display. If the operation is I/O bound, or nearly so, it will be slowed down by other operations that also need access to memory. You should assume that the sustainable bandwidth on processing memory (SDRAM) is 300 MB/s (the peak rate is 400 MB/s). Hence if grabbing and other operations require, say, 30 MB/s, they will slow down an I/O bound processing function by about 10% if executed in parallel. The effect on a compute bound function should be negligible. The quoted I/O rates are approximate values taking into account any extra I/O that might be performed internally to hold intermediate results etc.

Function overheads are included in the measured execution times, so you must take them into account when scaling the numbers to a different image size (see the section on overheads for details). Overheads are not included in the processing or I/O rates, so you can more easily scale these to different image sizes. The difference between the measured execution time and the expected time calculated from the processing rate and image size is the actual fixed overhead for that function.

Note that some of the numbers may change in future releases as improvements are made. You should always look for the most recent version of these benchmarks. Entries marked with an asterisk (*) are further explained after the table. Entries marked with † are data dependent, and performance will vary from image to image.

| Function and options | Time (ms) with overhead |  | Rate (MPix/s) without overhead |  | I/O (MB/s) without overhead |  |
| --- | --- | --- | --- | --- | --- | --- |
| imBinConvert() 8-bit to/from binary 16-bit to/from binary 32-bit to/from binary | 1.4 2.3 4.4 |  | 260 133 65 |  | 290 280 270 |  |
| imBinCountDifference()* Difference count Event count (Src2 = 0) | 0.9 0.7 |  | 600 1100 |  | 150 140 |  |
| imBinMorphic()* IM_ERODE/IM_DILATE IM_3X3_RECT_1, 1 iteration IM_3X3_RECT_1, 2 iterations IM_3X3_RECT_1, 3 iterations All operations except IM_MATCH General 3x3 General 5x5 General 7x7 General 9x9 General 11x11 IM_MATCH General 3x3 General 5x5 General 16x16 General 32x32 | #### C80 1.0 1.3 1.7 2.2 4.1 6.8 10.4 14.7 17.8 25.8 70.7 288 | #### NOA 1.3 1.4 1.5 1.3 1.4 1.5 1.9 2.5 3.1 3.2 5.5 15.8 | #### C80 470 290 210 170 76 43 27.6 19.2 15.2 10.4 3.78 0.92 | #### NOA 500 500 500 500 500 500 320 215 143 141 75.6 23.8 | #### C80 120 72 53 42 19 11 7 5 17 12 4 2 | #### NOA 125 125 125 125 125 125 81 54 160 160 85 51 |
| imBinTriadic() No inputs One input Two inputs Three inputs | 0.6 0.8 1.0 1.2 |  | 980 580 420 320 |  | 120 140 160 160 |  |
| imBlobCalculate()*† 8-bit, 9 blobs, total area 5% of image Area only Area + binary COG Area + grey COG 8-bit, 100 blobs, total area 25% Area only Area + binary COG Area + grey COG | ~2.7 ~3.1 ~3.8 ~9 ~12 ~17 |  |  |  |  |  |
| imBufClear() Binary 8-bit 16-bit 32-bit | 0.6 1.0 1.7 3.1 |  | 950 370 190 95 |  | 120 370 380 380 |  |
| imBufCopy() IM_PROC to IM_PROC (on same node) Binary 8-bit 16-bit 32-bit IM_PROC to IM_DISP (VIA driven)* 8-bit, PCI write 8-bit, PCI read 8-bit, VM IM_PROC to IM_HOST (VIA driven)* 8-bit IM_HOST to IM_PROC (VIA driven)* 8-bit | 0.9 2.0 3.5 6.4 3.6 4.6 3.1 3.6 4.1 |  | 580 170 88 44 86 66 118 86 74 |  | 150 340 350 350 86 66 118 86 74 |  |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| imBufGet()* 8-bit | 29.0 |  | 9.2 |  | 9.2 |  |
| imBufPut()* 8-bit | 5.1 |  | 61 |  | 61 |  |
| imBufPack()† 8-bit, none tagged 8-bit, all tagged 8-bit, 35% tagged (circular mask) 16-bit, 35% tagged (circular mask) 32-bit, 35% tagged (circular mask) | 0.6 2.7 1.4 2.2 3.5 |  | 1500 110 270 150 85 |  | 190 240 230 230 240 |  |
| imFloatConvert() 8-bit to float 16-bit to float 32-bit to float float to 32-bit | 4.6 12.1 7.1 7.1 |  | 67.0 22.7 38.4 38.4 |  | 330 140 310 310 |  |
| imFloatDyadic() IM_ADD, IM_SUB IM_SUB_ABS IM_MIN, IM_MAX IM_MULT IM_DIV IM_SQUARE_ADD IM_ATAN2 | 12.4 15.7 27.5 10.5 40.7 19.0 900 |  | 21.7 17.1 9.6 25.8 6.5 14.0 0.29 |  | 260 200 110 310 78 170 3.5 |  |
| imFloatMac1() | 11.4 |  | 23.7 |  | 190 |  |
| imFloatMac2() | 19.0 |  | 14.0 |  | 170 |  |
| imFloatMonadic() IM_ADD, IM_SUB IM_SUB_ABS, IM_SUB_NEG IM_MIN, IM_MAX IM_MULT, IM_DIV IM_DIV_INTO | 7.1 12.3 24.8 7.1 38.0 |  | 38.4 21.8 10.7 38.4 7.0 |  | 310 170 85 310 56 |  |
| imFloatUnary() IM_NEG IM_ABS IM_SQUARE IM_SQRT IM_CUBE IM_LOG, IM_EXP, IM_SIN, IM_COS, IM_TAN, IM_ATAN IM_CBRT | 7.1 12.3 7.1 53.7 12.9 700 – 900 1700 |  | 38.4 21.8 38.4 4.9 20.7 ~0.3 0.15 |  | 310 170 310 39 170 ~2.5 1.2 |  |
| imGenWarpLutMatrix() IM_CTL_PRECISION = 0 IM_CTL_PRECISION > 0 | 63.5 74.0 |  | 4.2 3.6 |  | 17 14 |  |
| imIntBinarize() IM_IN_RANGE/ IM_OUT_RANGE 8-bit 16-bit 32-bit All other conditions 8-bit 16-bit 32-bit | 2.1 3.9 7.4 1.9 2.7 4.3 |  | 148 75 37 173 114 67 |  | 300 220 190 340 340 340 |  |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| imIntClip() Mode = IM_THRESH_CONST+IM_PASS_PIXEL Cond = IM_IN_RANGE or IM_OUT_RANGE 8-bit 16-bit 32-bit All other non-binary cases are I/O-bound 2 bytes of I/O per pixel 3 bytes of I/O per pixel 4 bytes of I/O per pixel 5 bytes of I/O per pixel 6 bytes of I/O per pixel 7 bytes of I/O per pixel 8 bytes of I/O per pixel etc. | 2.9 5.3 10.2 2.0 2.7 3.5 4.3 5.0 5.8 6.6 |  | 105 53 27 170 113 85 68 57 48 42 |  | 210 210 210 340 340 340 340 340 340 340 |  |
| imIntConnectMap() 8-bit to 8-bit 8-bit to 16-bit | 7.0 7.3 |  | 40 38 |  | 80 110 |  |
| imIntConvert() 8-bit to 16-bit 8-bit to 32-bit 16-bit to 32-bit 16-bit to 8-bit (truncate) Unsigned 16-bit to 8-bit (IM_CLIP) Signed 16-bit to 8-bit (IM_CLIP) Signed 16-bit to 8-bit (IM_ABS_CLIP) 32-bit to 8-bit (IM_CLIP) 32-bit to 16-bit (IM_CLIP) | 3.0 4.3 5.0 3.0 3.2 5.8 4.5 6.0 6.0 |  | 100 64 55 94 84 48 63 43 39 |  | 300 320 330 280 250 140 190 210 250 |  |
| imIntConvertColor() 8-bit IM_RGB_TO_HSL 8-bit IM_HSL_TO_RGB 8-bit IM_RGB_TO_H 8-bit IM_RGB_TO_L 8-bit IM_RGB_TO_I 8-bit IM_L_TO_RGB 8-bit IM_MATRIX, 3x1 no clip 8-bit IM_MATRIX, 3x1 unsigned clip 8-bit IM_MATRIX, 3x1 signed clip 8-bit IM_MATRIX, 3x3 no clip 8-bit IM_MATRIX, 3x3 unsigned clip 8-bit IM_MATRIX, 3x3 signed clip | 36.4 44.6 30.0 6.5 5.7 5.9 5.3 7.7 9.0 14.5 22.3 26.3 |  | 7.2 5.9 8.9 43 61 57 49 34 29 18.1 11.8 10.0 |  | 43 35 35 170 240 230 200 140 120 110 71 60 |  |

|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| --- | --- | --- | --- | --- | --- | --- |
| imIntConvolve()* 8-bit (see detailed description later) IM_SMOOTH IM_SHARPEN IM_SHARPEN2 IM_HORIZ_EDGE IM_VERT_EDGE IM_SOBEL_EDGE IM_PREWITT_EDGE IM_LAPLACIAN_EDGE IM_LAPLACIAN_EDGE2 IM_ROBERTS_EDGE General kernel 3x3 5x5 7x7 9x9 11x11 Symmetric kernel 5x5 7x7 9x9 11x11 All 1’s kernel (16-bit output) 5x5 11x11 Effect of normalization options: No Shift (16-bit output) Clip/Absolute imIntConvolve()* 16-bit (see detailed description later) IM_SMOOTH IM_SHARPEN IM_SHARPEN2 IM_HORIZ_EDGE IM_VERT_EDGE IM_SOBEL_EDGE IM_PREWITT_EDGE IM_LAPLACIAN_EDGE IM_LAPLACIAN_EDGE2 IM_ROBERTS_EDGE General kernel 3x3 5x5 7x7 9x9 11x11 Symmetric kernel 5x5 7x7 9x9 11x11 All 1’s kernel 5x5 11x11 Effect of normalization options: Shift Clip/Absolute | #### C80 9.7 8.6 9.4 7.3 4.7 11.3 11.3 8.6 9.5 3.0 9.7 26.0 44.9 69.4 99.2 26.0 44.9 69.4 99.2 5.7 8.5 –2.1 +0.7 C80 13.3 8.8 16.1 7.5 7.6 11.6 11.6 8.8 16.1 5.4 13.3 35.2 68.3 112.8 168.6 35.2 68.3 112.8 168.6 11.2 18.3 +0.0 +4.1 | ##### NOA 2.3 2.3 2.3 2.3 2.3 11.3 11.3 2.3 2.3 3.0 2.3 4.0 5.5 8.2 11.8 3.2 4.9 5.8 6.8 3.2 6.8 +0.0 +0.0 ##### NOA 4.2 4.2 4.2 4.2 4.2 11.6 11.6 4.2 4.2 5.4 4.2 9.0 17.0 27.6 40.8 4.8 6.3 9.4 13.0 4.8 9.0 +0.0 ##### +0.0 | ###### C80 28.8 32.3 29.8 38.3 62.2 24.2 24.2 32.3 29.7 101 28.8 10.4 5.96 3.84 2.68 10.4 5.96 3.84 2.68 52 35 C80 20.9 31.6 16.9 37.2 36.5 23.6 23.6 31.6 16.9 52.4 20.9 7.65 3.91 2.36 1.57 7.65 3.91 2.36 1.57 24.7 15.1 | ###### NOA 160 160 160 160 160 24.2 24.2 160 160 101 160 80 55 35 24 107 62 52 44 107 44 NOA 74 74 74 74 74 23.6 23.6 74 74 52.4 74 32 16.1 9.8 6.6 65 47 30 22 65 32 | C80 58 65 60 77 120 48 48 65 60 200 58 21 12 7.7 5.4 21 12 7.7 5.4 156 105 C80 84 130 68 150 140 94 94 130 68 210 84 31 16 9.4 6.3 31 16 9.4 6.3 99 60 | NOA 320 320 320 320 320 48 48 320 320 200 320 160 110 70 48 215 125 105 87 315 130 NOA 300 300 300 300 300 94 94 300 300 210 300 125 65 39 26 260 190 120 87 260 130 |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| imIntCorrelate() 8x8 model, STEP = 1 8x8 model, STEP = 2 16x16 model, STEP = 1 16x16 model, STEP = 2 | 305 227 1090 307 |  | 0.86 0.24 1.16 0.85 |  | 2.6 0.7 3.5 2.6 |  |
| imIntCountDifference() 8-bit 16-bit 32-bit | 2.0 3.7 6.9 |  | 160 79 40 |  | 320 320 320 |  |
| imIntDistance() IM_CITY_BLOCK 8-bit 8-bit to 16-bit All other cases | 12.4 14.6 19.2 |  | 21.9 18.6 13.9 |  | 88 130 98 |  |
| imIntDyadic() IM_DIV 8-bit 16-bit 32-bit IM_DIV_FRAC 8-bit to 16-bit 16-bit to 32-bit IM_MULT_MSB 32-bit All other operations (I/O bound) 8-bit 16-bit 32-bit | 18.6 29.1 47.3 32.3 52.7 13.0 3.0 5.6 10.7 |  | 14.4 9.1 5.6 8.2 5.0 21 102 51 26 |  | 43 55 67 33 40 250 310 310 310 |  |
| imIntErodeDilate()* Unsigned 8-bit Kernel IM_3X3_RECT_0 Kernel all zero 5x5 7x7 9x9 11x11 General kernel (not all zero) 3x3 5x5 7x7 9x9 11x11 Unsigned 16-bit Kernel IM_3X3_RECT_0 Kernel all zero 5x5 7x7 9x9 11x11 General kernel (not all zero) 3x3 5x5 7x7 9x9 11x11 Overhead for signed data (all cases) | #### C80 4.7 23.7 44.7 69.4 105 19.2 50.7 97.8 161 240 7.9 47.3 88.1 141 207 19.5 51.0 98.3 161 240 +0.9 | #### NOA 2.2 2.5 3.5 4.8 6.7 2.2 4.7 8.7 14.0 20.6 4.0 4.6 6.1 7.5 8.9 4.1 4.8 8.8 14.2 20.8 +0.0 | C80 62 11.4 6.0 3.8 2.5 14.1 5.2 2.7 1.6 1.1 35 5.5 3.0 1.9 1.3 13.9 5.2 2.7 1.6 1.1 | NOA 160 132 91 62 44 160 64 32 19.6 13.2 75 64 48 38 32 75 62 32 19.3 13.0 | C80 120 23 12 7.7 5.1 28 11 5.4 3.3 2.2 140 23 12 8 5 56 21 11 6.5 4.4 | NOA 320 265 180 125 87 320 130 64 39 26 300 260 190 150 130 300 250 130 77 52 |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| imIntFindExtreme() IM_MIN or IM_MAX (only one) Unsigned 8-bit Unsigned 16-bit 32-bit IM_MIN and IM_MAX (both) Unsigned 8-bit Unsigned 16-bit 32-bit | 1.2 2.0 3.4 1.8 3.1 4.5 |  | 310 160 86 190 96 64 |  | 310 320 340 190 190 250 |  |
| imIntFFT() 32-bit fixed-point complex 2-d (forward or reverse) 1-d (horizontal pass only on 512 lines) | 240 105 |  | 1.1 2.5 |  |  |  |
| imIntFlip() Flip or 180 degree rotate 8-bit 16-bit 32-bit 90 or 270 degree rotate 8-bit 16-bit 32-bit | 2.1 3.5 6.7 2.7 5.6 12.3 |  | 150 83 41 110 49 22 |  | 300 330 330 220 200 170 |  |
| imIntGainOffset() 8-bit No clip or offset Clip but no offset Clip and offset 16-bit No clip or offset Clip but no offset Clip and offset | 2.9 3.1 5.8 5.5 5.5 6.9 |  | 102 97 49 51 51 40 |  | 310 290 200 310 310 320 |  |
|  | Time (ms) |  | Rate (Mpix/s) |  | I/O (MB/s) |  |
| imIntHistogram()* 8-bit 10-bit, short table 10-bit, long table 12-bit, short table | 2.7 4.0 8.9 10.2 |  | 123 89 33 33 |  | 120 180 65 66 |  |
| imIntLabel() Few blobs (fastest) Many blobs (slower) | 9.2 15 |  | 31 17 |  | 180 100 |  |
| imIntLocateEvent()† 8-bit unsigned 100 events Number only With X, Y positions 10000 events Number only With X, Y positions | 1.3 2.0 3.5 6.6 |  | 330 260 90 47 |  | 330 260 90 47 |  |
| imIntLutMap()* Default method 8-bit to 8-bit 8-bit to 16-bit 8-bit to 32-bit 14-bit to 8-bit (16 KB table) 13-bit to 16-bit (16 KB table) 12-bit to 32-bit (16 KB table) 16-bit to 8-bit† 16-bit to 16-bit† 16-bit to 32-bit† Method with work buffer 16-bit to 8-bit 16-bit to 16-bit 16-bit to 32-bit† | 2.1 2.7 4.3 4.2 4.3 7.0 ~26 ~35 ~43 21 24 ~50 |  | 150 114 67 73 68 41 ~10 ~7 ~6 12.4 11.0 ~5 |  | 300 340 340 220 270 250 ~30 ~30 ~30 110 130 ~90 |  |
| imIntMac1() 8-bit to 8-bit 8-bit to 16-bit 16-bit to 16-bit | 3.1 3.2 3.6 |  | 96 92 82 |  | 190 280 330 |  |
| imIntMac2() 8-bit to 8-bit 8- and 16-bit to16-bit 16-bit to 16-bit | 3.2 4.6 5.2 |  | 95 63 54 |  | 280 310 320 |  |
| imIntMonadic() IM_DIV 8-bit 16-bit 32-bit IM_DIV_FRAC 8-bit to 16-bit 16-bit to 32-bit IM_MULT_MSB 32-bit All other operations (I/O bound) 8-bit 16-bit 32-bit | 15.1 25.8 47.2 27.2 49.9 12.6 2.0 3.6 6.8 |  | 17.9 10.3 5.6 9.8 5.3 21 163 82 41 |  | 35 41 45 29 32 170 330 330 330 |  |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| imIntProject() 0.0 degrees 8-bit to 32-bit 16-bit to 32-bit 32-bit to 32-bit 90.0 degrees 8-bit to 32-bit 16-bit to 32-bit 32-bit to 32-bit | 2.1 2.6 3.9 1.8 2.4 4.3 |  | 147 117 75 184 126 67 |  | 150 230 300 180 250 270 |  |
| imIntRank() IM_3X3_RECT 8-bit median 16-bit median IM_3X3_CROSS 8-bit median 16-bit median IM_3X3_X 8-bit median 16-bit median IM_1X5 8-bit median 16-bit median IM_5X1 8-bit median 16-bit median | 11.1 24.3 8.4 20.3 6.5 12.5 6.4 11.9 8.8 14.5 |  | 24.6 11.0 32.8 13.2 43.2 21.6 44.2 22.8 31.1 18.6 |  | 49 43 66 53 86 86 88 91 62 74 |  |
| imIntRecFilter() 8-bit to 16-bit (no Dst2) 8-bit to 16-bit (with Dst2) 16-bit to 16-bit (no Dst2) 16-bit to 16-bit (with Dst2) | 5.5 6.2 6.1 7.5 |  | 52 46 46 37 |  | 260 270 280 260 |  |
| imIntScale() IM_INTERPOLATE 8-bit 2×2 8-bit 4×4 8-bit 8×8 8-bit arbitrary factor 16-bit arbitrary factor IM_NO_INTERPOLATE 8-bit arbitrary factor 16-bit arbitrary factor | 3.8 3.0 2.9 6.6 7.8 3.7 5.1 |  | 82 108 112 42 36 81 56 |  | 103 115 115 |  |
| imIntSubsample()* IM_INTERPOLATE 8-bit (any supported factor) 16-bit (any supported factor) IM_NO_INTERPOLATE 8-bit 2×2 8-bit 4×4 16-bit 2×2 16-bit 4×4 | C80 2.3 3.4 1.1 0.7 1.8 1.0 | ##### NOA 2.1 3.2 | C80 140 86 380 800 180 400 | NOA 235 117 | C80 290 300 280 250 | NOA |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |
| imIntThickThin() 8-bit Single 3x3 kernel 8-band 3x3 kernel 16-bit Single 3x3 kernel 8-band 3x3 kernel | 21.2 154 41.7 308 |  | 12.8 1.73 6.4 0.86 |  | 25 27 25 27 |  |
| imIntTriadic() All operations are I/O-bound 2 bytes of I/O per pixel 3 bytes of I/O per pixel 4 bytes of I/O per pixel 5 bytes of I/O per pixel 6 bytes of I/O per pixel | 1.9 2.7 3.7 4.6 5.6 |  | 170 110 80 63 51 |  | 340 340 320 310 310 |  |
| imIntWarpLut()* Nearest neighbour 8-bit 16-bit 32-bit Bilinear interpolation 8-bit 16-bit | ~10 ~11 ~13 ~21 ~22 |  | ~27 ~25 ~20 ~12 ~12 |  | ~160 ~200 ~240 ~80 ~100 |  |
| imIntWarpPolynomial()* Nearest neighbour 8-bit 16-bit 32-bit Bilinear interpolation 8-bit 16-bit Bicubic interpolation 8-bit 16-bit | ~8 ~10 ~18 ~20 ~29 ~70 ~92 |  | ~39 ~30 ~15 ~14 ~9 ~3.8 ~2.9 |  | ~80 ~120 ~120 ~30 ~40 ~8 ~12 |  |
| imIntZoom()* IM_INTERPOLATE 8-bit (any supported factor) 16-bit (any supported factor) IM_NO_INTERPOLATE 8-bit 2x2 8-bit 4x4 16-bit 2x2 16-bit 4x4 | C80 6.5 6.7 2.2 2.0 3.0 3.6 | NOA 2.5 3.2 2.2 2.0 3.0 2.6 | C80 44 42 150 170 105 125 | NOA 180 120 220 275 135 160 | C80 190 180 130 130 | NOA 270 310 340 350 |
| imJpegDecode()†* 8-bit lossless 8-bit lossy | C80 ~23 ~14 | ##### NOA ~6.8 | C80 ~11 ~18 | NOA ~45 | C80 ~17 | NOA ~70 |
| imJpegEncode()†* 8-bit lossless 8-bit lossy | C80 ~19 ~16 | ##### NOA ~3.3 | C80 ~14 ~16 | NOA ~100 | C80 ~21 | NOA ~150 |
| imPatFindModel()† Speed IM_HIGH, accuracy IM_MEDIUM 32x32 model 64x64 model 128x128 model 256x256 model Speed IM_MEDIUM, accuracy IM_HIGH 32x32 model 64x64 model 128x128 model 256x256 model | ~26 ~12 ~8 ~14 ~26 ~12 ~12 ~25 |  |  |  |  |  |
|  | Time (ms) |  | Rate (MPix/s) |  | I/O (MB/s) |  |

* Means that the function is discussed further below.

† Means that the performance is data dependent.

## Specific functions

The behaviour of some functions is more complicated that the benchmark table indicates, and they need to be discussed individually.

imBinCountDifference()

The benchmarks given assume that the source images are byte aligned. Difference counting is slower for non-aligned buffers (i.e. child buffers with an X origin that is not a multiple of 8 pixels).

imBinMorphic()

The pre-defined kernel IM_3X3_RECT_1 has been specially optimized for the C80, and there is a particularly large performance gain when multiple iterations are required. Therefore, with the C80 you should decompose large erosions or dilations into multiple iterations of IM_3X3_RECT_1 whenever possible. However, with the NOA it is just as good to define large kernels directly and use only one iteration. In general, binary morphology is so fast with the NOA that the function overhead is very important on small images, and it might sometimes be faster to disable the NOA and use the C80 instead. However, there is a way to reduce the NOA setup overhead for the second and subsequent passes with a given kernel (see the documentation for the control fields supported by the function).

imBlobCalculate()

Performance here is data dependent, and the more blobs are present the longer it takes. If your images are noisy, so that after thresholding many spurious blobs are present, it is usually best to clean up the thresholded image with morphology before doing blob analysis. To make sure this cleaning stage runs as fast as possible, threshold your original image into a 1-bit binary buffer.

The benchmarks in the table were obtained with the saving of run information disabled (i.e. after calling imBlobControl() with IM_BLOB_SAVE_RUNS set to IM_DISABLE). If you save runs (which is the default behaviour) processing time will be slightly longer.

Another important point in blob analysis it to read back as many results as you can at once. This means making use of the data structures, IM_BLOB_GROUP1_ST etc.,  that hold a whole group of blob results. See the example program BLOB.C for usage.

imBufCopy()
Performance of PCI transfers between the host and Genesis are dependent on the host PCI chipset, so you should do your own tests to measure the performance on your system. See also the following discussion on imBufGet() and imBufPut(). Note that VM performance depends on the VM channel clock speed. This can be set to 25 or 33 MHz in the GENESIS.INI file.

The performance of both PCI and VM transfers varies somewhat with the width of the image being transferred.

imBufGet()/imBufPut()

In these functions the data transfer is driven by the host CPU. Writes to Genesis memory are much faster than reads, although actual figures are dependent on the host PCI chipset. imBufGet() is normally adequate for small amounts of data such as a histogram result (which is typically a few Kbytes), but for whole images it is much faster to allocate a host buffer and use imBufCopy(). On some systems imBufPut() might be the fastest way to get even large amounts of data from the host to the Genesis, but it ties up the host CPU. imBufCopy() should provide comparable speed and does not tie up the host CPU.

imIntConvolve()

The main point about convolution is that the 8-bit case runs considerably faster than the 16-bit case. For the C80 the 8-bit case requires that the source buffer be 8-bit unsigned, and that the kernel values all lie in the 8-bit signed range [–128, +127]. (Note that the type of the kernel buffer itself is irrelevant; the actual kernel values will be tested to see if the 8-bit convolution can be used.) Another requirement for the 8-bit case on the C80 is that it must be possible to use a 16-bit signed accumulator without causing possible overflows. This means that the sum of the positive kernel values must not exceed 128, and the sum of the negative kernel values must not exceed –128. If not all of the 8-bit requirements are met, the 16-bit benchmarks will apply.

Note that many predefined kernels use specially optimized C80 code, and execute faster than the equivalent kernel that you define yourself. Always use a predefined kernel if you can.

On the NOA the 8-bit case is not so restricted. Any 8-bit type (for image or kernel) is handled at the same speed, and there is no limit on the sum of kernel values. If you have 16-bit data with an 8-bit kernel, or 8-bit data with a 16-bit kernel, the speed will be somewhere between the listed 8- and 16-bit benchmarks. The slowest case of all is that of 16-bit data and a 16-bit kernel (and this is what is used for the listed 16-bit NOA benchmarks). Note that the NOA 8-bit benchmarks were made in exact mode (the computation control field, IM_CTL_COMPUTATION, was set to IM_EXACT so that no approximations were made). Some symmetric kernels will show a slight improvement in fast mode (where the computation control field is set to IM_FAST). The NOA 16-bit benchmarks for symmetric kernels were made in fast mode. However, if your data is 14-bit or less, you will get the same performance in exact mode as long as you specify the number of input bits.

You will see that the NOA takes advantage of symmetric kernels (positive or negative symmetry, in either the horizontal or vertical direction). The symmetric benchmarks listed assume both horizontal and vertical symmetry (which is quite common for real convolution kernels). The C80 cannot take advantage of  kernel symmetry, but it does have specially optimized code for kernels whose values are all 1. Furthermore, the time for an all 1’s kernel is only weakly dependent on the kernel size. Kernels whose values are some other constant (optionally with a different centre value) are only slightly slower on the C80 than the same-sized all 1’s kernel. The NOA does not handle constant kernels as a special case, but they are faster than general kernels because of their symmetry.

The number of special cases is simply too large to describe here. If convolution is a time critical operation for you, you should benchmark your particular kernel to see how fast it runs. When using the NOA, you should also consider using fast mode. This significantly improves the speed of some cases with only a slight loss of precision. Note also that there is a way to reduce the NOA setup overhead for the second and subsequent passes with a given kernel (see the documentation for the control fields supported by the function).

imIntErodeDilate()

With the NOA any symmetric kernel will run at the same speed as the all-zero case. With the C80 there is no advantage to symmetric kernels (only the all-zero case has been optimized). As you can see, large all-zero kernels are better implemented as multiple passes with IM_3X3_RECT_0 on the C80 (assuming they can be decomposed this way), but this is not true for the NOA. Note also that there is a way to reduce the NOA setup overhead for the second and subsequent passes with a given kernel (see the documentation for the control fields supported by the function).

imIntHistogram()

Performance here is very dependent on the type of the input data (i.e. the size of the histogram result buffer). For 12-bit or larger data, you should consider not using every pixel in the histogram calculation (see the control fields which enable subsampling). You might also consider shifting the data down to 8 or 10 bits before performing the histogram (see the control field which specifies the number of bits you want to use).

imIntLutMap()

Here the performance is very good when the lookup table is no bigger than 16KB, because then the table fits entirely into the C80’s internal RAM (assuming all four PPs are used, otherwise the table must be proportionally smaller). Performance with very large LUTs is also data dependent, so you may need to try both the available methods to see which performs best in your case. If you have 16-bit data, you should consider using an interpolated LUT mapping (which uses a smaller table).

imIntSubsample()

The quoted processing rates are in terms of input pixels/second. Since the output image is smaller, the processing rate will be lower in terms of output pixels/seconds. Note that the NOA can be used only for subsampling factors of 1 or 2, but there is only a significant speedup on images larger than about 512x512. This is because the higher processing rate of the NOA is offset by a bigger overhead. On small images it might be better to disable the NOA.

imIntWarpLut()

All figures for this function are approximate since performance is weakly dependent on the actual transformation (i.e. the values in the Xlut and Ylut buffers).

imIntWarpPolynomial()

All figures for this function are approximate since performance is weakly dependent on the actual transformation. Replace overscan may also be slightly faster than transparent overscan since in the former case some parts of the output buffer can simply be replaced with a constant value (if that region originated outside the source buffer).

imIntZoom()

The quoted processing rates are in terms of output pixels/second. The NOA can be used only for zoom factors of 1, 2 or 4. The processing rate is higher for the NOA but the overhead is also higher. Hence on small images it might be better to disable the NOA.

imJpegEncode()/imJpegDecode()

The times are slightly data dependent, and in the lossy case are also affected by the Q factor (higher compression produces slightly faster times). You can also improve performance a little by increasing the restart interval (increase the value of IM_JPEG_RESTART_ROWS from its default of 32). When you also need to transfer the compressed image on or off the board, you should also consider the time taken by imJpegReadBuf()/imJpegWriteBuf(). However, these functions can be overlapped with the actual encoding or decoding by running them in another thread. In this case they add very little extra time.