STANDARD MICROSYSTEMS CORPORATION

Novell NetWare OS/2 Requester driver

For EtherCard PLUS Series LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

MINIMUM REQUIREMENTS:

--------------------

1) 80386 or 80486-Based System

2) 4 MB System RAM

3) 30 MB Hard Disk

4) OS/2 Installation Package (Version 1.3 or later)

5) NetWare for OS/2 (Version 1.3 or later)

a) OS/2 Requester Software

b) OS/2 Requester Utilities

6) Driver Software for OS/2

a) smc8000.sys

b) net.cfg

INSTALLATION PROCEDURE:

----------------------

1) Insert the Novell NetWare OS/2 Requester diskette into your

diskette drive.

2) Type: INSTALL and press Enter.

3) From the installation option, select "Requester on

Workstation."

4) Specify the target directory, the source directory, and

select OK.

5) Select "Edit CONFIG.SYS and COPY Files..."

6) Select "SMC8000.SYS" from the Network Interface Card Driver

List. If "SMC8000.SYS" is not an available option, select

"TOKEN.SYS."

7) Select or deselect any remaining options you require. Then

select "Save."

8) Type: CONFIG.SYS in the "Save File As:" field, then select

OK.

9) Select "Copy" in the Copy Requester Files Window.

10) Follow the prompts until the installation is completed.

11) If you have not already done so, select "Utilities on

Server:" from the Installation Menu and follow the prompts

until this installation is completed.

12) If you selected "TOKEN.SYS" in Step 7, do the following:

a.  Copy SMC8000.SYS from the OS2_ODI directory on your

SuperDisk to the target directory you selected in

Step 5.

b.  Edit the CONFIG.SYS file. Change

device=<path>token.sys

to read

device=<path>smc8000.sys

USING NET.CFG FOR MULTIPLE ADAPTERS

-----------------------------------

1) Locate the NET.CFG file on your hard disk. If a NET.CFG

file does not already exist, copy the sample NET.CFG

file from the SuperDisk diskette to the appropriate

directory.

2) Using a text editor, add the "Link Driver" entry to

NET.CFG for each adapter. Separate each entry with a

blank line. For example:

Link Driver SMC8000

Link Driver SMC8000

The first adapter loaded will be the one with the lowest

I/O base address or, if the machine is Micro Channel,

the lowest slot number.

ERROR MESSAGES

--------------

"Adapter hardware failure detected (initialization)."

"Adapter unable to insert into network."

"Adapter IRQ higher than 7 not valid in 8-bit slot."

"No adapters found."

"One or more NET.CFG parameters are missing or invalid."

"I/O address in NET.CFG is on illegal boundary."

"I/O address in NET.CFG is out of range."

"Adapter IRQ higher than 7 not valid in 8 bit slot."

"IRQ specified in NET.CFG is out of range."

"RAM address in NET.CFG is on illegal boundary."

"RAM address in NET.CFG is out of range."

"RAM size specified in NET.CFG is out of range."

"Cannot access adapter RAM properly."

"Adapter hardware failure detected (self-test)."

"Adapter configuration corrupted; run software setup."

"Other system ROM already occupies adapter RAM address space."

"Other system RAM already occupies adapter RAM address space."

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.