README.DOC: Help file for ME34T2/ME14T2 Combo Card product

System Requirements:

PCMCIA Type "2" or "3" Slot.

Intel & Compatible PCMCIA Controller Chip

Memory:

20K (5000h) memory window from C0000h to DC000h on 16K boundaries.

Note: This memory cannot conflict with EMM386.EXE or other Programs.

I/O Memory:

Com Port Address: 3f8, 2f8, 3e8, 2e8, base address (8 bytes long).

Interrupts:

Modem requires any one of IRQ: 3, 4, 5, or 7

LAN requires any one of IRQ: 5, 7, 9, 10, 11, 12, 15.

LAN requires any one Software Interrupt: 60h, 61h, 62h, 63h.

Interrupts should not be the same for the LAN and Modem.

Defaults:

Memory: D8000h-DFFFFh.

Com2 = 2f8h, IRQ 3.

LAN IRQ = 15, Software Int = 62

* * * * * *

INSTALL.EXE     Installation routine

* * * * * *

Source drive: current drive

Destination Directory: C:\APEXEM\  (default)

Modes: Automatic configuration for

COMBO CARD SETUP:     Updates APEXEM.CFG

NOVELL ODI NETWORKS:  Updates NET.CFG

NDIS 2.0 NETWORKS:    Updates PROTOCOL.INI

* * * * * *

APEXEM.SYS  The PCMCIA software Enabler, installed in CONFIG.SYS

* * * * * *

Command Line Options:

/c=<PATH>\APEXEM.CFG    Path to configuration file for internal setup.

/f=<PATH>\NET.CFG       Path to configuration file for Novell ODI setup.

or...

/f=<PATH>\PROTOCOL.INI  Path to configuration file for NDIS setup.

/n   Enables PCMCIA as only Combo Card, regardless of CARD inserted.

NO check for a valid 'CIS', do not use this option when other cards

are used in the system.

/d   Enable Debug messages.

NOTE:  BELOW ARE SPECIAL PARAMETERS FOR AST Asentia 900N & COMPAQ Elite 400

/r=3e4  Option to set base address for Slot B controller (Special option).

/o=80   Option to set offset for Slot B controller registers (Special).

* * * * * *

EMVIEW.EXE  Program to setup and test the Combo Card

* * * * * *

Command options: None

OutPut: APEXEM.CFG   The configuration file used to setup the Combo Card.

Description: This program will configure the Combo Card 'on the fly' by

talking with the Enabler APEXEM.SYS.  The basic functions of

the card can be tested for verification of LAN and Modem setup.

Once the Combo Card has been setup, the configuration file

is saved and the Combo Card is ready for use.  LAN testing

will test basic hardware functions and interrupts, it will

not allow access to the LAN Network physically.  Modem testing

will open a standard terminal screen to test the Com Port and

modem.

Note: If APEXEM.CFG has been changed, the NET.CFG or PROTOCOL.INI file

may have to be modified by the INSTALL.EXE program or manually

* * * * * *

NET.CFG         Novell ODI configuration file

* * * * * *

The INSTALL.EXE program will update this file if the Novell option is chosen.

Required contents for this file:

link driver emodi

frame ethernet_802.3

frame ethernet_802.2

swint 62

sharedint

mem #1 d8000

Descriptions:

link driver emodi   Name of Novell ODI driver, supplied is EMODI.COM

frame ...           The frame statement will vary according to your Network

Setup.  More than one frame statement may be used.

swint 62            The LAN software interrupt: 60,61,62, or 64

sharedint             Always needed, dual interrupt mode.

mem #1 d8000        LAN Memory window, 16K long from C0000-DC000

* * * * * *

PROTOCOL.INI

* * * * * *

The INSTALL.EXE program will update this file if the NDIS option is chosen.

Required contents for this file:

[EMNDISNET]

DRIVERNAME=EMNDIS20$

SWINT=0X62

MEMORYADD=0XD8000

DRIVERMODE=SHAREDINT

Descriptions:

[EMNDISNET]            Driver Identifier

DRIVERNAME=EMNDIS20$   Name of NDIS 2.0 driver supplied is EMNDIS20.DOS

SWINT=0X62             The LAN software interrupt: 0X60,0X62, or 0X64

DRIVERMODE=SHAREDINT     Always needed, dual interrupt mode.

MEMORYADD=0XD8000      LAN Memory window, 16K long from C0000-DC000