MMCLOCK.SYS

An installable driver for the

National Semiconductor MM58167A

real-time clock/calendar chip

Version 1.2

June 1990

Richard B. Wales

UCLA Computer Science Dept.

3531 Boelter Hall

Los Angeles, CA 90024-1596

(213) 825-5683

Internet:  wales@CS.UCLA.EDU

WHAT IS MMCLOCK.SYS?

MMCLOCK.SYS is an installable device driver for a real-time clock/calendar

(PC or XT) based on the National Semiconductor MM58167A chip.  This chip is

used in various multifunction cards and clone motherboards.

Add-on clock cards usually come with some software for manipulating the date

and time on the card.  Many of these programs are inconvenient to use and/or

are of inferior quality.  The great majority of them do not understand about

leap year (which could give you minor headaches in 1992).  MMCLOCK.SYS per-

mits the clock card to be treated "transparently"; the system date/time is

automatically set on each reboot, and any changes to the date/time (e.g., via

the DOS commands DATE and TIME) automatically update the clock card without

the need to run any additional program.

MMCLOCK.SYS was written in assembly language and assembled using the Borland

Turbo Assembler (version 2.0).  The code has also been successfully assem-

bled using the Microsoft Macro Assembler (version 5.1).

INSTALLING THE DRIVER

In most cases, all that need be done is to put MMCLOCK.SYS on a disk drive

(generally C:) and add a DRIVER= line to the CONFIG.SYS file.  MMCLOCK.SYS

will automatically locate the clock/calendar (if present).

If MMCLOCK.SYS cannot find the clock/calendar, it may be that the hardware is

set up to use an unusual I/O port address.  In this case, you need to deter-

mine the port address used (possibly by disassembling the driver software

that came with the machine or card), and then include this address as an argu-

ment on the DRIVER= line.  For example:

DRIVER=C:\BIN\MMCLOCK.SYS 280

Note that not all add-on clock/calendar cards use the MM58167A chip.  If your

clock card does not use this chip, MMCLOCK.SYS will not work for you.

If you use MMCLOCK.SYS, you should NOT use ANY other clock driver or other

software that may have been supplied with your clock card.  This is impor-

tant, because MMCLOCK.SYS reads and writes the date on the clock card in a

highly nonstandard manner that is completely incompatible with other drivers.

COMPATIBILITY CONSIDERATIONS

MMCLOCK.SYS works with the same kind of clock/calendar hardware as does the

Strickland clock driver (CLKDRV.SYS).  Any software which claims compatibil-

ity with CLKDRV.SYS should work equally well with MMCLOCK.SYS.

MMCLOCK.SYS IMPLEMENTATION DETAILS

The following information is not essential for the user of MMCLOCK.SYS, but

may be interesting to some users.

The MM58167A chip contains a 14-decimal-digit counter, plus 14 hexadecimal

digits (56 bits) of RAM.

The counter holds the month, day of the month, day of the week, and time of

day (hours, minutes, seconds, and milliseconds).

The RAM is officially designed to hold the same kind of information as the

counter -- in conjunction with an alarm-clock interrupt feature.  However,

most clock cards which use the MM58167A do not even try to use the alarm-

clock facility, so the RAM can in practice hold arbitrary data.

The counter data is accessed via eight consecutive I/O ports, thus:

Port		High-order		Low-order

number		  nybble		 nybble

=================================================

x+0		1/1000 sec		  ZERO

x+1		 1/10 sec		1/100 sec

x+2			seconds (2 digits)

x+3			minutes (2 digits)

x+4			 hours (2 digits)

x+5		   ZERO			day of wk.

x+6		     day of month (2 digits)

x+7			 month (2 digits)

Wherever "2 digits" are indicated, the value is stored as binary coded deci-

mal (BCD) -- not as straight binary.

The "day of the week" portion of the counter cycles through the values 1

through 7 inclusive.

The RAM is laid out in the same manner as the counter, and is accessed via

eight consecutive I/O ports (x+8 through x+15).  The two nybbles shown as

"ZERO" in the counter diagram above do not have RAM associated with them; any-

thing written to these two nybbles will be discarded, and the value read for

these nybbles will always be zero.

Except for the two "ZERO" nybbles, the RAM is not restricted to BCD values;

rather, any nybble may hold any 4-bit binary value.

The MM58167A also has several status and control registers.  In particular,

one status register indicates whether the clock has ticked during a "read" or

"write" operation.  Since some hardware implementations may not provide

access to the status and control registers, MMCLOCK.SYS does not use them.

Instead, after the counter information has been read/written, MMCLOCK.SYS

reads back the low-order position of the counter and compares it with the

value originally read/written.  If the re-read value does not match the orig-

inal value, the read/write is repeated.

The MM58167A chip has two major flaws which complicate efforts to use it as a

perpetual clock/calendar.  The first problem is that it has no provisions for

keeping track of the year.  The second problem (a logical consequence of the

first) is that it has no concept of a leap year.  Many drivers or other soft-

ware for the MM58167A simply store the year somewhere in the RAM; this tech-

nique doesn't work very well for leap year, however.  Since the RAM may be

used in just about any desired fashion at all, though, it is possible to get

around these problems through clever, unorthodox use of the RAM.

MMCLOCK.SYS handles the date in a manner that completely bypasses the issue

of leap years.  The fourth and fifth byte of the clock chip's RAM are used to

store a 16-bit integer representing the number of days since January 1, 1980.

Each time the date is set in this manner, the "month" and "day" portions of

the clock counter are set to January 1, no matter what the real date might be.

When MMCLOCK.SYS reads the date from the clock, it examines both the count of

days in the RAM and the date information in the counter.  The month and day

from the counter are not used "as is", but are instead translated into an

integer number of days elapsed since January 1.  This number is added to the

RAM's day count to give the current date.

The "day count" technique turns out to be especially convenient, as this is

precisely the format in which DOS expects a clock driver to handle the date

anyway.  MMCLOCK.SYS doesn't need to convert between the day count and a con-

ventional month/day/year format (DOS already does that).  Further, there is

no need to be concerned about leap years.

Whenever the date has advanced since the last time it was set (i.e., whenever

the "month" and "day" information in the counter says something other than

January 1), MMCLOCK.SYS rewrites the date in the RAM and the counter.  This

prevents the month/day information in the counter from getting too high, and

simplifies the task of translating month and day into a number of days.  The

resetting of the date is skipped, however, if the time on the clock is 23:59;

this averts the possibility that the time might roll over to midnight (and a

new day) while the date is being rewritten.

Whenever the time of day is reset, MMCLOCK.SYS also updates the BIOS time-of-

day count (via INT 1AH, function 1).  As a result, any programs which depend

on the BIOS time-of-day count at 0040:006CH will work OK.

REQUIREMENTS FOR A DOS CLOCK DRIVER

The following information may be useful for the programmer who wishes to

write a DOS driver for a different clock/calendar card.  It is assumed that

the reader already understands the basics for writing an installable device

driver under DOS.

The "attribute word" for a clock driver should be 8008H.

The driver needs to handle the various possible I/O requests as follows:

0 (initialize)

Initialize the driver.

4 (read)

Read the date/time.

8 (write)

9 (write and verify)

Write the date/time.

1 (media check)

2 (build parameter block)

6 (current input status)

7 (flush input buffers)

10 (current output status)

11 (flush output buffers)

13 (open)

14 (close)

Set DONE status bit, but do nothing else.

5 (non-destructive read)

Set BUSY, ERROR, and DONE status bits, but do nothing else.

3 (I/O control read)

12 (I/O control write)

15 (removable media check)

16 (output until busy)

Set UNKNOWN, ERROR, and DONE status bits, but do nothing

else.

For "read" and "write" operations, DOS communicates with the clock driver

via a six-byte buffer with the following format (all fields are in straight

binary, not BCD):

Byte	Meaning

0-1	Date (16-bit count of days elapsed since 1 January 1980;

low-order byte comes first).

2	Minutes (0-59).

3	Hours (0-23).

4	Hundredths of a second (0-99).

5	Seconds (0-59).

CONDITIONS FOR USING MMCLOCK.SYS

MMCLOCK.SYS is Copyright 1990 by Richard B. Wales.  It may be freely used by

anyone without charge.  You may distribute it to anyone, but you may not

charge anything (beyond a nominal copying or communications charge).  You may

not distribute the MMCLOCK.SYS file alone, but you must distribute the source

code and this documentation along with it.  You may not distribute a modified

version of MMCLOCK.SYS, its source, or this documentation unless you clearly

indicate what you have modified.

The author (Richard B. Wales) believes that MMCLOCK.SYS is bug-free.  How-

ever, he is not responsible for any damages which you may incur as a result

of using MMCLOCK.SYS -- including incidental or consequential damages -- even

if he has been informed in advance of the possibility of such damages.  Use

of MMCLOCK.SYS is completely at your own risk, and you accept full responsi-

bility for its use.