6x86ctl

Change/view the Configuration Control Registers of a Cyrix 6x86 CPU

by Ray Van Tassle	(rayvt@comm.mot.com)

V1 Dec 26, 1996

Copyright (c) 1996 by

Ray Van Tassle

1020 Fox Run Lane

Algonquin, Il. 60102  (USA)

There are several programs floating around which set various

option bits in the Cyrix 6x86 CPU.  I didn't like any of them,

because none allowed me to see all the registers, or to change

the bits individually.  Hence this version.

The -f sets all the performance bits, and also the bit that enables

the CPUID opcode.

If you want to set only CPUID-ENABLE, use -e.

I ran some benchmarks with different settings.  The big hitter

seems to be WT_ALLOC.

With PcBench9.0, Cpumark16:

After boot	363.2

with -f		361.9

with -s		340.9

-f, no DTE_EN	361.9

-f, no NOLOCK	361.8

-f, no FAR_COF	360.5

-f, no WT_ALLOC	341.4 <===== slowest

The sparsely documented Branch Target Buffer is set to enable

far change of flow (FAR_COF).  IBM shows how to do this, but does

not explain the undocumented registers used.

I only enable FAR_COF BTB's when you give the "-f" option.  To disable,

but leave all the others set, use "-s -m -e".

At reset, the 6x86 resets all the CCR's to zero.  The bios may (or

may not) turn on some of the bits.  Below is what the settings

on mine are.

* Gigabyte GA-586HX motherboard, Award bios (1.11)

* Cyrix cpu, 6x86/133(P166+) rev 2.7

DIR0: 31  2X clock

DIR1: 17  Stepping 1, rev 7 (rev v2.7)---The good version.

CCR0: 02  00000010

......1.   NC1

CCR1: 82  10000010

......1.   USE_SMI

.....0..   SMAC

...0....  *NO_LOCK

1.......   SM3

CCR2: 80  10000000

.....0..   LOCK_NW

....0...  *SUSP_HLT

...0....   WPR1

1.......  *USE_SUSP

CCR3: 00  00000000

.......0   SMI_LOCK

......0.   NMI_EN

.....0..   LINBRST

CCR4: 17  00010111

.......1  _IORT1

......1.  _IORT2

.....1..  _IORT4

...1....  *DTE_EN

0.......  *CPUID

CCR5: 21  00100001

.......1  *WT_ALLOC

...0....   LBR1

..1.....   ARREN

Far COF's in BTB are enabled.

=======================================================================

The following info is from the Cyrix Web site,  Christian Ludloff's

great CPU info web site, http://webusers.anet-dfw.com/~ludloff,

and the IBM 6x86 BIOS Writer's Guide (document 40205).

------------------------------------------------------------------------------

CCR0  C0h  configuration control register #0

bit7..2  reserved

bit1     NC1 - caching for 640K..1M area

1=disabled (=never), 0=enabled

bit0     reserved

------------------------------------------------------------------------------

CCR1  C1h  configuration control register #1

bit7     SM3 - SMM address space address region register #3

1=enable address region register #3 for SMM addr. space

0=disabled (see also CCR5.ARREN, independent!)

bit6..4  reserved

bit4     N_LOCK - negate LOCK#

1=enabled, 0=disabled

bit3     reserved

bit2     SMAC - enable SMM memory accesses with SMAADS# active

1=enabled (SMI# ignored), 0=disabled

bit1     SMI - enable SMM pins (SMI# I/O pin and SMADS# output pin)

1=enabled, 0=disabled (=pins float)

bit0     reserved

------------------------------------------------------------------------------

CCR2  C2h  configuration control register #2

bit7     SUSP - enable SUSP# input pin and SUSPA# ouput pin

1=enabled, 0=disabled (=pins float)

bit6..5  reserved

bit4     WT1 - caching for 640K..1M area

1=force all writes to 640K..1M area that hit in cache

issued on the external bus

0=disabled

bit3     HALT - enable entering suspend mode on HLT inctructions

1=enabled, 0=disabled

bit2     LockNW - prohibits changing the state of the CR0.NW bit

1=enabled (=prohibited), 0=disabled (=allowed)

bit1..0  reserved

------------------------------------------------------------------------------

CCR3  C3h  configuration control register #3

bit7..4  MAPEN - select active control register set for D0h..FDh

0001=default (others are not valid at the moment)

bit3     reserved

bit2     LINBRST - enable linear address sequence for burst cycles

1=enabled, 0=disabled

bit1     NMIEN - enable NMI during SMM

1=enabled, 0=disabled

bit0     SMI_LOCK - SMM register lock

1=CCR1.bit3..1 and CCR3.bit1 can't be changed in SMM;

CCR3.bit0 can be changed in SMM; only RESET clears it!

0=disabled

------------------------------------------------------------------------------

CCR4  E8h  configuration control register #4

bit7     CPUID

1=enable EFLAGS.bit21 and CPUID instruction

0=disable EFLAGS.bit21 and CPUID instruction

bit6..5  reserved

bit4     DTE_EN - enable directory table entry cache

1=enabled, 0=disabled

bit3     reserved

bit2..0  IORT - I/O recovery delay time

2^x bus clock cycles (0 = fast, 6 = slow)

7 = no delay

------------------------------------------------------------------------------

CCR5  E9h  configuration control register #5

bit7..6  reserved

bit5     ARREN - address region registers enabled

1=enabled, 0=disabled (see also CCR1.SM3, independent!)

bit4     LBR1 - LBR# pin

1=assert LBR# for all accesses to 640K-1M, 0=disabled

bit3..1  reserved

bit0     WT_ALLOC - WT allocation

1=allocate new cache lines for read and write misses

0=allocate new cache lines for read misses only

------------------------------------------------------------------------------

ARR0 .. ARR7  address region register #0 thru #7

Starting address and size

#7 defines the Main Memory Region

RCR0  ..  region control register #0 thru #7

bit5     NLB - negate LBA#

1=enabled, 0=disabled

bit4     WT - write through caching

1=enabled, 0=disabled

bit3     WG - write gathering

1=enabled, 0=disabled

bit2     WL - weak locking

1=enabled, 0=disabled

bit1     WWO - weak write ordering

1=enabled, 0=disabled

bit0     RCD - cache disabled for region (for RCR0..6 only)

RCE - cache enabled for region (for RCR7 only)

1=enabled (implies that the address space outside of the

region specified by ARR7 is non-cacheable), 0=disabled