Help for using TSRmaker pop-ups

"""""""""""""""""""""""""""""""

The  pop-up screens that are created with TSRmaker can be

difficult to use if certain procedures are not followed.

1)  TSRmaker  pop-ups  watch  for  certain   keystrokes   by

intercepting INT 9. If you have any programs loaded that

also grab that interrupt, it is possible that there will

be  a  conflict.  To avoid a jam up, change the order in

which your resident programs are loaded. If your  pop-up

TSRmaker  is  being loaded last, for example, try moving

it up a couple of places in your Autoexec.bat file.

2)  TSRmaker screens only pop-up if your video display is in

modes  2  or  3.  If  the  display  is  using one of the

graphics modes, mono mode 7, or a 40 character  display,

then  the  pop-up  will  not  come up on the screen. The

solution is to switch to a CGA 80 column text  mode  and

try the key-combo again.

3)  TSRmaker  screens are being written to page 3 of the CGA

display. If your application is using  this  page  there

could  be  a  possible  conflict.  This  is  not likely,

however.

4)  TSRmaker  screens  only  work  on  CGA   or   compatible

displays!  They  will not pop-up on an EGA or VGA screen

that is not in the CGA mode.

Thanks and I hope this clears up most problems that users  run

into.

-Jack A. Orman  *  July 1988