OPL3-SAx Installation for Windows 3.1 & DOS

YAMAHA CORPORATION

Semiconductor Division

Introduction

The OPL3-SAx Driver and associated software are supplied on a 2HD floppy disk or CD-ROM.

The OPL3-SAx requires MS-DOS 6.2. Operation with other operating systems is not guaranteed. Installation from MS-DOS leads to installation of windows 3.1 & DOS driver. Along with the OPL3-SAx Driver, the application “YAMAHA STATION” are installed.

Typing optional command leads to set up DOS enviroment only. See the following writing for details.

There are two processes in this setup,whether Configuration Manager or PnP BIOS is in your system or not. See the following writing about “SETUPSA”for details.

YAMAHA STATION

The YAMAHA STATION application is installed in a new Program Group named YAMAHA. See the “README” for more information about the YAMAHA STATION.

Installation

Install OPL3-SAx Windows 3.1 driver.

An OPL3-SAx must be present in your system for successful driver installation.

This software is installed from the DOS prompt, not the Run command in Windows.

The Installation program overwrites the existing AUTOEXEC.BAT, CONFIG.SYS, and Windows SYSTEM.INI files.

To cancel the Installation program, press the Esc key at anytime.

1. Insert Disk [SA31INST] into your floppy disk drive or select appropriate (WIN31) directly of CD-ROM.

2. At the C:  DOS prompt, type

A:

then press Enter.

If your floppy disk or CD-ROM drive is the B or E drive, type B: or E: instead of A:, then press Enter. (For installation by CD-ROM, go to the directory “WIN31”)

3. Type as follows.

"install" (installation of drivers,application,setup program for Windows3.1)

"install -d" (installation of only setup program for DOS/Windows3.1)

and press Enter.

When using IDE CD-ROM interface of soundcard, If yor computer has no Configuration Manager, add the option “-c” also

as follows.

"install -c"

"install -d -c"

As a installer asks whether it makes a temporary directory that is named “OPL3TEMP”, press “Y” key. (In case of installation by CD-ROM, this message doesn’t appear.)

If you change the directory name,

1.Press “N” key and make directory for yourself.

2.Copy the archive files “INST31.EXE””WINST95.EXE” from the floppy disk to the directory which you made.

3.Type “INST31.EXE””WINST95.EXE” to extract.

4. Type "Install" (for Windows) or "Install -d" (for only DOS), and press Enter.

The following dialog box appears.

µ §

4. Select and click YES.

If you're not using a mouse, press the Return key for YES, or the Esc key to Cancel.

The following dialog box appears.

µ §

5. Specify the installation directory for the OPL3-SAx Configuration Files or accept the default, then click OK.

The files are copied to the hard disk and the following dialog box appears, informing you that parameter settings for the OPL3-SAx have been added to the AUTOEXEC.BAT file and a copy of the original AUTOEXEC.BAT file has been saved as AUTOEXEC.BAK.

The following contents are written into the new AUTOEXEC.BAT. After restart PC, these are valid.

- SET BLASTER (Setting the BLASTER function)

- SETUPSA.EXE (that is written into the directory assigned at "Configuration file will be installed to:")

µ §

If you type "install -c" or "install -d -c" to use IDE CD-ROM I/F of sound card, the following dialog box appears, informing you that parameter settings for the OPL3-SA have been added to the CONFIG.SYS file and a copy of the original CONFIG.SYS file has been saved as CONFIG.BAK.

µ §

6. Click OK to continue.

note:

The installation of DOS only, the message that the installation is completed is displayed. Then, click OK to finish the installation.

The following dialog box appears, asking whether or not you have Windows installed on your computer.

µ §

7. Click Yes if Windows 3.1 is installed on your computer. Click No if it isn't.

The following dialog box appears, asking where Windows is installed.

µ §

8. Click OK if Windows is installed in the directory C:  Windows, which is the default directory for Windows. Or specify a different directory.

The following dialog box appears, asking whether you want to replace the MIDI Mapper.

µ §

9. Select Yes to replace the current MIDI Mapper. Select No not to replace it.

If a MIDI Mapper already exists and you replace it with the OPL3-SA MIDI Mapper, the original is saved as midimap.opl. See your Windows documentation for more information about MIDI Mapper.

“SETUPSA” dialog opens automatically. If Configuration Manager or PnP BIOS is in your system, see the following writing 1-A about “SETUPSA”. If the one is not in your system, see the following writing 2-A about “SETUPSA”.

When you finish SETUPSA,  it ask you “Next time, do yo want to open this setup window in boot sequence?”. If you want, press “Y” key, and if you don’t, press “N” key.

The following dialog box informs you that the installation is complete and the original SYSTEM.INI file was saved as SYSTEM.opl.

µ §

10. Click OK to finish.

The installer asks whether it deletes a temporary directory. If you want to delete, press “Y” key, and if you don’t, press “N” key.

11. Launch Windows.

When windows started, the application (YAMAHA STATION) is extract.

note:

Even if you didn’t install the application “Audio Rack” in this installation, you can do it.

1.When this installation finish,please do not delete the temporary directory “OPL3TEMP”.

2.Launch ”FILE MANAGER”, and double click the “WINSTALL.EXE”(or “WINSTCD.EXE” for installation by CD-ROM) in the directory “OPL3TEMP”(or “WIN31” for installation by CD-ROM).

3. Follow the on-screen instructions to complete the installation.

4.After this installation, you can delete the temporary directory “OPL3TEMP”.

SETUPSA

µ §

1. Configuration Manager or PnP BIOS is in your system.

1-A. Type “SETUPSA”

Open the dialog box, and display the information of OPL3-SAx I/O address, DMA and IRQ assigned by Configuration Manager or PnP BIOS. Set the value of master volume that is written into OPL3SA.INI.

When you change the master volume and click “OK”, the value is set and written into OPL3SA.INI.

1-B. Type “SETUPSA /S”

Do not open the dialog box, but display the configuration of OPL3-SAx on the screen. Set the value of master volume that is written into OPL3SA.INI.

2. Configuration Manger and PnP BIOS are not in your system.

2-A. Type “SETUPSA”

Open the dialog box “YAMAHA OPL3-SAx Mixer”, and display and set the contents into OPL3SA.INI. When you change the configuration and the master volume and click “OK”, they are set and written into OPL3SA.INI.

If you’re not using a mouse, use the Tab key to move between the parameters.

Set unique values for the SB (Sound Blaster) and WSS (Wave Synthesizer)         Base I/O Address, IRQ, and DMA Channel. The MPU401 IRQ parameter is    automatically set to the IRQ that you chose for the Sound Blaster. The default settings, shown below, should work fine. You may need to change them, however, if you have other cards installed in your computer.

I/O

IRQ

DMA

Sound Blaster

220h

5

1

WSS

530h

5

0

MPU401

330h

5

---

In the SB and WSS Mixer sections, use the left cursor key to raise the volume and the right cursor key to decrease it. If you are using a mouse, click the [UP] and [DOWN] buttons to adjust the volume level.

In SB (Sound Blaster) mode, pressing the S key plays a WAVE sound. Pressing the F key plays an FM sound.

In WSS mode, pressing the W key plays a WAVE sound.

The settings that you make here are written to the AUTOEXEC.BAT file, so they are active every time you switch on your computer.

To change these settings after installation, run the “SETUPSA” program again from the direc-tory you specify for installation.

2-B. Type “SETUPSA /S”

Do not open the dialog box, but display and set the contents into OPL3SA.INI.

OPL SoftSynth Control Panel

The Soft Synthesizer supports GM System Level 1 MIDI data. Its volume is adjusted by the Wave volume con-trol. The OPL SoftSynth Control Panel shown below is used to setup the Soft Synthesizer.

µ §

Quality

These four options determine the sound quality (i.e., sampling rate) and the number of voices that the SoftSynth can produce simultaneously. Select a mode appropriate for the CPU performance of your computer. If you choose a high quality mode and your computer does not have adequate processing performance, the SoftSynth may not produce sound fluently. In this case, select a lower quality mode.

Reverb

If you choose ON, you can add reverb to the sound of the SoftSynth.

Filter

With the filter ON the SoftSynth sound is more fluent. This, however, requires CPU resources.

MPU401 OUT

These settings allow you to select an External MIDI instrument or the SoftSynth for use with DOS-based games that are played on Window box. The MS-DOS Prompt item in the Main Program Group to open an MS-DOS window. Select General MIDI in the game settings to use this function.

Full Duplex

It is possible to record a new Wave file while playing an existing Wave file.

note:

SoftSynth/WaveOut of OPL3-SAx is cooperative mode, you can play Wave data and SoftSynth MIDI simultaneously.

IDE CD-ROM interface

When using IDE CD-ROM interface of soundcard, Change the parameters of IDE in SETUPSA, because these are OFF as default values. CD-ROM device driver should be bundled in config.sys, and the CD-ROM driver is unique to each manufacture of CD-ROM drive.

ex. : a case of CD-ROM drive made by Mitsumi

In the “CONFIG.SYS” file, confirm or add the following two descriptions or rewrite :

DEVICE=C:\OPL3SA\SACDROM.SYS /P1E8 /I11 /A3EE

:

:

DEVICE=C:\MTM\MTMCDAI.SYS /D:MTMIDE01 /P:1F0,14 /P:1E8,11

In the “AUTOEXEC.BAT” file, confirm or add the following description or rewrite :

LH C:\DOS\MSCDEX.EXE /D:MTMIDE01 /S

:

If the CD-ROM drive cannot be recognized in your system, please contact with the manufacture of CD-ROM drive.