# Chapter 1

## Introduction

This mainboard uses the SiS530 chipset to leverage extra performance from the Socket-7 architecture. The mainboard supports all Socket-7 processors including newer designs which feature a 100 MHz system bus. The mainboard firmware supports CPU Plug and Play so that the system will automatically adopt the correct configuration for the Socket-7 processor that you install.

The mainboard is highly integrated and includes a built-in PCI 3D Sound System and a built-in 3D AGP graphics adapter.  The sound system supports 24-bit digital audio and a 4-way speaker system. The graphics system supports extended VGA resolutions with an 8MB frame buffer.

Communications and networking are supported with a V.90 Fax/Modem DAA module and a 10BaseT/100BaseTX network adapter. The mainboard supports both AT/ATX power supply. If you use an ATX supply, it supports all of the ATX power management features including ACPI (Advanced Configuration and Power management Interface), power saving modes and keyboard power on hot keys.

The mainboard has a full set of I/O ports and three expansion slots. It supports hardware monitoring and ships with trend’s ChipAway Virus virus protection system. The board adheres to the baby-AT form factor which permits an inexpensive system case and reduced power supply unit.

### Key Features

This key features of this mainboard include:

#### Socket-7 Processor Support

- Supports all recent socket-7 processors including Intel P55C (Pentium MMX), Cyrix/IBM 6x86L/6x86MX/M-II,  AMD K6/K6-2/K6-III, and IDT C6 and WinChip 2/2A CPUs

- Supports socket-7 processors with system bus frequencies of 60/66/75/83/95/100 MHz

- CPU Plug and Play support let the firmware automatically configure the CPU

- 512K/1 MB external Level 2 cache memory is provided onboard

#### Memory Support

- Three DIMM slots for 3.3V SDRAM 168-pin memory modules

- Support for 66 MHz and 100 MHz memory bus

- Maximum installed memory can be 3 x 256 MB = 768 MB

#### Expansion Slots

- Two 32-bit PCI slots

- One 8/16-bit ISA slots

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Bus mastering and UltraDMA 33/66 modes

#### Power Supply and Power Management

- Provides AT/ATX power connector

- Support for Power button/Suspend Switch, and Keyboard Power On/Off (needs Win98 keyboard)

- Supports Wake on Modem, Wake on LAN and Wake on Alarm

#### Built-in Graphics System

- Onboard 64-bit 3D AGP Graphics Accelerator

- Complies with AGP Ver.1 with 66/133 MHz operation

- Shared memory architecture allows a maximum of 8 MB main memory to act as frame buffer

- Supports high resolutions up to 1600 x 1200 pixels

#### Sound System

- Meets PC98 audio specification

- Full duplex playback and recording with built-in 16-bit CODEC

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D® compatible interfaces plus support for 4-channel speakers

- Drivers support Windows 3.1/95/98/NT 4.0

- Built-in 32 ohm earphone buffer and 3D surround

- Provides MPU-401 Game/MIDI port and legacy Sound Blaster 16 support

- Downloadable Wave-table Synthesizer supports Direct Music®

- Digital Audio Interface with 24-bit stereo, 44KHz sampling rate and measured 120dB audio quality

- Optional optic fibre interface which enables communication with MiniDisk or high-end audio systems.

- Stereo Mixer supports analog mixing from CD-Audio, Line-In, and digital mixing from voice, FM/Wave-table and digital CD-Audio

#### Onboard I/O Ports

- Floppy disk drive port with 1Mb/s transfer rate

- One serial port with 16550-compatible fast UART

- One parallel port with support for ECP and EPP

- Two USB ports & one PS/2 ports (optional)

- One optional infrared port

#### Hardware Monitoring

- Built-in hardware monitoring for CPU temperature and fan speeds

- Supports AMI’s Desktop Client Manager (ADCM)

#### Built-in LAN

- Onboard 10BaseT/100BaseTX LAN

- LAN controller integrates Fast Ethernet MAC and PHY compliant with IEEE802.3u 100BASE-TX, 10BASE-T and ANSI X3T12 TP-PMD standards

- Compliant with ACPI 1.0 and the Network Device Class Power Management 1.0

- High Performance provided by 100 Mbps clock generator and data recovery circuit for 100 Mbps receiver

#### Fax/Modem DAA Module

- 56 Kbps Fax/Modem DAA module

- Supports V.90, V.34, V.32bis, V.32, V.22bis, V.22

- Supports Auto Fallback and MNP 5, V.42bis data compression with 115200 compatible Virtual UART

- Requires 16 MB RAM and WIN 95/98/NT

#### Onboard Flash ROM

- Provides plug and play function for automatic CPU and board configuration

- Supports plug and play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s ChipAway Virus which ensures that the entire boot process is virus protected.

#### Bundled Software

- AMI Desktop Client Manager supports hardware monitoring for a stand-alone systems or a networked workstation

- PC-Cillin provides automatic virus protection under Windows 95/98

- Super Voice is Fax/Modem software with support for data and voice transmission

- MediaRing Talk is an internet telephone application

- Gamut provides professional audio applications included MP3 encoding/playback

- WordPerfect Suite 8 is a windows version office application

#### Dimensions

- Baby-AT form factor (22cm x 22cm)

### Package Contents

Your mainboard package ships with the following items:

- Mainboard

- This User’s guide

- IDE cable

- Floppy diskette drive cable

- Audio ports and Game/MIDI port extension bracket

- Serial/parallel ports extension bracket

- VGA extension bracket

- V.90 Fax/Modem DAA module

- 10BaseT/100BaseTX network adapter extension bracket

- Support software CD-ROM

#### Optional Accessories

You can purchase the following optional accessories for this mainboard.

- Digital Audio extension bracket

- ATX Form card (2 USB ports, IR port & PS/2 Port)

### Static Electricity Precautions

- Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard, and other components, in their original static-proof packaging until you are ready to install them.

- During an installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, frequently discharge any static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During the installation lay the mainboard on top of the static-proof packaging with the component side facing upwards.

- Inspect the mainboard for any damage caused during transit. Ensure that all the components that are plugged into sockets are correctly seated.

- If you suspect that the mainboard has been damaged, do not apply power to the system. Contact your mainboard vendor and report the damage.