Wellcome Windows 95 user !!!

This driver is for PCMCIA LCS-8534 TB Ethernet adapter.

Colour: red, green. NE2000 compatible.

Not for the LCS-8534 or LCS-8534 T !!! no way...

1. Install the socket service from Windows 95

( Systemcontrol, hardware, required method )

2. After that, Windows 95 recognizes the Longshine LCS-8535 TB

PCMCIA Ethernet adapter.

Requested driver from manufacture and then the path to the

PCMCARD.INF.

3. Check your systemcontrol, system, device-manager a double-klick

on the PCMCIA LCS-8534 TB Ethernet adapter, the resources.

vaild value are I/O port 220-340 and a free IRQ.

Tip: I/O = 300 IRQ = 10, when possible !!!

Shutdown system and start again. After starting you have to hear

twice piep, i.e., the socket service is correctly installed.

The Networkcontrol has to be checked.

that all folk's

greetings from Longshine Technolgie Europe GmbH

very Hot-line today