Unofficial Advanced Gravis Ultrasound Drivers #2 ("Manley Drivers")

Copyright 1994 Robert J. Manley and Multi-Player Computer Adventures

Version 0.5d - SHAREWARE - please read the REGISTRATION section for details

FEATURES

- basic GUS rev 3.7 and MAX/daughtercard mixer support

- 8 or 16 bit wavefile playback

- mono or stereo

- hardware MIDI playback (to an external synth or synth module)

- volume and balance control

- multi-session (many registered streams - one active)

- Ultrasound specific OS/2 VDD giving DOS sessions:

- reduced interrupt latency

- hardware virtualization

- MIDI port emulation of Roland MPU-401 in UART mode

(allowing WinOS2 to load and use those drivers - hooray!)

- absolute lowest system overhead possible

- rigorous testing

Here is what it has been tested and known to work with:

- OS/2 2.1 with MMPM CSD, OS/2 2.11 (with its MMPM CSD),

and Beta II of Personal OS/2 (aka WARP II)

- system sounds

- digital audio applet

- MIDI applet

- compact disc applet and CD explorer (bug in version 2.3 hangs

PM message queue when using digital transfer)

- bigben2

- dmplayer 1.01

- video player / Ultimotion

- wave doctor

- multimedia mahjongg

- GUSmixer (even while the device is in use!)

- play.cmd (thanks to TFS for reminding me about this one)

- WinOS2 apps

- media player

- Cakewalk Pro

- Midisoft Recording studio

INSTALLATION

*** STEP #1 ***

This driver requires one of the following to be installed:

- OS/2 Warp Beta I or Beta II MMPM

- OS/2 2.11 MMPM (from OS/2 2.1 Service Pack)

- OS/2 2.1 MMPM + November '93 MMPM CSD

(ftp-os2 in pub/os2/ibm/os2fixes as MMPMCS.DSK)

If neither of these versions of MMPM can be found, the installation will

fail and nothing will be added to your CONFIG.SYS.

*** STEP #2 ***

Unzip ULTRA05D.ZIP to a subdirectory or floppy.  Run MINSTALL and select that

directory as the source and then follow the directions.  Note that MINSTALL

will modify your CONFIG.SYS to append the driver line.  This shouldn't be a

problem as I have done it numerous times without a glitch.  You may install

this software over a previous version without first uninstalling.  Also note

that the driver can be UNINSTALLED by selecting the number of Ultrasounds to

install as zero.

*** IMPORTANT STEP #3 (NEW) ***

If you have previously installed an earlier version of this driver since

installing your current system, you must read this step carefully and follow

all instructions.  There is a limitation in the MMOS2 installation program

which forces the user to manually edit the CONFIG.SYS.  After installing this

software, search for a line containing the token "AUDIOVDD" such as the

following:

DEVICE=c:\MMOS2\AUDIOVDD.SYS ULTRA1$

where "c:" is the drive MMOS2 has been installed on.  If this line is present

as shown here, comment it out with a REM statement at the beginning.  If you

have other devices also listed on this line (e.g. "PAS16$"), just delete the

"ULTRA1$" token.

HARDWARE EMULATIONS

In addition to virtualizing the Ultrasound I/O ports to DOS sessions, the

supplied OS/2 virtual device driver (VDD) ULTRAVDD.SYS also emulates the

following devices when selected in the AUDIO_ADAPTER_EMULATION DOS setting:

- Roland MPU-401 MIDI interface in UART mode (e.g. Windows).

Select I/O address of 0x330 and IRQ of 2/9.

MIDI is re-routed through the GUS MIDI port to an external synth

(not played natively as in MegaEm).

Future directions are to add Sound Blaster emulation and implement Roland GM

natively on the GUS (without requiring an external synth).

WinOS2 TIPS

It is recommended that all WinOS2 users keep a separate copy of their

SYSTEM.INI file for real Windows and WinOS2.  Doing this will allow

the Gravis supplied Windows drivers to work while under real Windows

and these drivers to provide MPU-401 emulation under WinOS2.

If you are running full OS/2 (with IBM's WinOS2), you may wish to copy

the TIMER.DRV from your real Windows SYSTEM directory to the equivalent

WinOS2 directory after renaming the original.  This driver seems to give

more accurate MIDI timing for Windows based MIDI sequencers.

KNOWN BUGS AND LIMITATIONS

- moving Ultimotion window can generate noise or loss of sound

- the Digital Audio, MIDI, and Digital Video icons in the

multimedia folder are replaced with incorrect versions

(these can be easily changed back manually)

- the installation program is lacking in functionality (see STEP #3

in the INSTALLATION section)

- MPU-401 interrupt driven recording isn't working yet (booo, hiss)

- some people experience a problem with starting DOS sessions as a

side-effect of ULTRAVDD.SYS.  If you are experiencing this problem,

replace "...ULTRAVDD.SYS" with "...AUDIOVDD.SYS ULTRA1$" in your

config.sys - please let me know if you do

MOST COMMONLY ASKED QUESTIONS

Q. Why don't I hear anything when using the MIDI applet?

A. If you don't have a MIDI synth connected, you won't.

A. Why don't I hear anything when using WinOS2 MIDI applications?

A. If you don't have a MIDI synth connected, you won't.

Q. Why is there a delay before hearing a system sound?

A. OS/2 must swap out blocks of memory to give the MMPM subsystem the

storage that it needs for holding the sound sample.  When enough memory

is already free (such as immediately after a system sound), the delay

should be almost unnoticeable.

Q. Why do I get the error message "The Ultrasound driver requires enhanced

mode to run" when trying to start a full-screen WinOS2 session?

A. Gravis' drivers include a Windows Virtual Device Driver (VxD != OS/2 VDD)

which cannot be loaded by OS/2 for system integrity reasons.  When the

load fails, WinOS2 (and Windows) interpret this error as the system not

running in 386 enhanced mode since that is the mode required for loading

and using VxDs.

Q. Why do my seamless WinOS2 sessions just sit there and never start?

A. See Q and A above.  When the VxD load fails, WinOS2 can't continue nor

can it properly exit.  Use a session killer like Watchcat to kill it.

LEGALESE

This software is provided AS IS.  No warranty of any kind, expressed or

implied, is granted including any warranties of quality or fitness for

a particular purpose.  Under no circumstances will either Robert J. Manley

or Multi-Player Computer Adventures be held responsible for any damage

that results directly or indirectly from the use of this product.

REGISTRATION

Please fill in the following information and send it along with a cheque

or money order for $25.00 US or CAN (we need the break) to:

Multi-Player Computer Adventures

14845-6 Yonge Street, Box 314

Aurora, Ontario, Canada

L4G 6H8

Name: __________________________________________________

Company: __________________________________________________

Address: __________________________________________________

: __________________________________________________

: __________________________________________________

e-mail: __________________________________________________

phone: __________________________________________________

Price includes 7% GST for Canadians and 8% PST for residents of Ontario.

Some future versions of this product may only be distributed commercially

but will be sent free to all registered users.

BUG REPORTS

Please send all bug reports, gratuitous worship, whatever, to me through

e-mail at: rjm@dretor.dciem.dnd.ca.  I enjoy hearing from all users.

Problem symptom: _____________________________________________

Fix or workaround: _____________________________________________

OS/2 version: ______

GUS hardware revision: ___

Daughtercard? (y/n): _

External MIDI? (y/n): _

GUS memory: ____ K

System memory: ____ M

WHAT'S NEXT

- Europe for the month of September!

- Sound Blaster emulation to DOS sessions

- MIDI synth

- Roland GM emulation to DOS sessions

- Digital Audio recording

- multi-track mod support at the driver level (need help here)

- getting Advanced Gravis to licence this software from me

FINALLY

I cannot emphasize enough my thanks to Brent Davis and especially Linden

deCarmo who have provided invaluable information when I needed it most.

Enjoy.  Robert.