CMOSTIME Version 1.0

Copyright 1994 by David J. Arcand

The Programmers Bulletin Board System of Western Massachusetts.

413-732-8323 14.4Kbps 24 hours/day

July 30, 1994  12:35 PM

I've been running some kind of BBS since August of 1985.  One

problem that has continually plagued me was the lack of a reliably

accurate clock in TRS-80's and now the PC.  In the past it was always a

matter of going in once a week and correcting the 40 or so minute

difference between the computer that ran the BBS and the rest of the

universe.

That was until I started running Wildcat 3.9m and DESQview with

three lines.  Now I was finding sometimes a 90 minute timewarp per day.

It was not unusual for me to even miss several mail run windows and

folks complaining about slow network mail.  Now I was having to manually

reset the time every day I came home from work !   This state of affairs

takes a lot of fun out of running a BBS.  Totally unacceptable.

Now I had many solutions, most involve spending money that I

didn't have.  Purchasing a faster motherboard, purchasing an RTC chip,

or calling long distance to some national time standard.

Now, the thing was, when I rebooted the system, the time was

accurate again.  This meant that the HARDWARE REAL TIME CLOCK was still

dead balls accurate, but the BIOS interrupt driven clock was loosing its

mind getting interrupted all the time.

What was going on was that with three COM ports, (two with high

speed modems) there was not always a timely interrupt to increment the

BIOS clock.  Sometimes it got missed and during high speed uploads, I

could lose upwards of 5 minutes per session.  Well, now that I knew what

the problem was, and a possible solution in mind, I called the Mustang

HQ BBS and asked about something to help.  John Neafus and Samual Koh

came back with the inspiration that I'd probably have to write this

utility myself.

What does CMOSTIME do ?  It reads the time from the hardware RTC

and writes it to the BIOS clock.  I could have written it in about 10

lines of code but there are some other stuff there to report on how

badly the clock was off in the first place.

I run this program every 8 hours as part of a routine batch event

from within Wildcat.  There is absolutely no reason why this program

would not work on other BBS software or usage.  For instance a SERVER

that is constantly being access by other workstations is sure to lose

time and could be fixed up by running this program.

The program is freeware.  The source code is included with the

distribution, and I reserve all rights.  But if you make changes, you

should to forward me a copy and keep my credit in it.

Whenever I write a program, I play this game called "GOOD, BUT

COULD BE BETTER" and wonder what I've done to improve the program.  Best

I can come up with is color characters for the final message report to

warn of really wild ass time differences.  Command line options to just

report the difference but not make any changes.  I mention these only

for those folks who are learning to program and want to have something

easy to learn with.

David Arcand -- Springfield Massachusetts -- July 30, 1994