TASK 1 Testing adb and fastboot

We need to make sure that the installed drivers are working correctly

for adb and fastboot to be able to communicate with an Android device

If you're lucky, this could be the easiest task, if not ..........

There is no way you can brick a device until it is rooted so relax and follow along

ADB:

You can get to adb via your desktop .bat file or manually

right click windows icon choose Run then type in CMD then enter

then at  c:\users\yourusernameappearshere>  type  cd\abd  enter

NB (adb after cd\  is the folder name where you installed adb.exe and fastboot.exe)

C:\adb>adb devices

List of devices attached

* daemon not running; starting now at tcp:5037

* daemon started successfully

nothing was returned or sometimes you'll get the HDX serial number and unauthorized like this:

B0180706350205R9        unauthorized

OR

B0180706350205R9        offline

this means the pc & HDX are not communicating  you need to turn on Android debugging and/or the popup rectangle to authorize debugging

C:\adb>adb devices

List of devices attached

B0180706350205R9        devices

this means the pc & HDX are communicating and ready for action

FASTBOOT:

CAUTION CAUTION CAUTION

fastboot can be your best friend or your worst enemy  relax and treat it with respect and it will reward you greatly

what we are going to do is very safe  BUT only do as follows for the time being

You can get into fastboot from the power buttom menu or use this adb command

C:\adb>adb reboot bootloader

when the bootloader screen appears it is not ready to execute commands and you probably will NOT see a ready prompt

if you do you can skip to testing fastboot

NO ready prompt:

we need to access the drivers that will make fastboot work as follows:

access win10 Device Manager

if you do not see something similiar to this you will have to do some research and update your usb drivers until you do

right click on the line showing the yellow triangle to update driver

choose Browse my computer ............

choose let me pick a list ............

scroll down to Fire Devices and double click

highlight Android Composite ADB interface

click next

click Yes

the HDX screen should now show  "Ready."   no quotes

type the following:

C:\adb>fastboot devices          and then press the enter key

B0180706350205R9        fastboot

this means HDX and pc are communicating and ready for action - (fastboot commands)

anything else means you need to find and apply the right drivers to win10

This maybe your biggest huddle and I can't help you from here  fingers crossed

If you were to choose Bootloader from the power button menu and find

yourself in bootloader mode with no "Ready.” prompt  you will need to press and

hold the power button until the HDX turns itself off