**************************************************************************

****			       UNREAL MODE !			      ****

****		(C) 19/11/1994	Daniele Paccaloni [DDT/HBT]	      ****

**************************************************************************

***	Contact me in Internet at:    paccalon@linux.infosquare.it     ***

***			       or     paccalon@ghost.dsi.unimi.it      ***

**************************************************************************

0.0 : INTRODUCTION

If you are not a programmer this stuff may be useless to you.

If you have not a 386 or better you can't run it.

Nevertheless you could find the following very interesting...

Archive content:

UNREAL.DOC : This text file.

UNREAL.EXE : The stand-alone program to enter UNREAL MODE (v1.0).

TEST.ASM   : Source of program to test UNREAL MODE.

TEST.EXE   : Executable of test program.

This program, docs, sources are distributed as PUBLIC DOMAIN.

However if you like them and want to send me money, donations, hardware, food,

girls... PLEASE DO IT  :-)  !!!

You can distribute this archive providing it is not changed nor altered in

any part.   You can't SELL IT, nor gain money/food/etc unless you are me !

If you use UNREAL.EXE in your productions you MUST include this DOC in your

archive or disk.

1.0 : WHAT IS UNREAL MODE ?

Unreal mode is something like a new CPU mode. NO JOKE: listen to me !

LINEAR MEMORY ADDRESSING POWER WITHOUT ANY DOS-EXTENDER !

A little introduction...

You know that the 286/386/486/Pentium CPU has 2 main mode of operation:

- REAL MODE

- PROTECTED MODE

Using MSDOS the CPU normally runs in REAL MODE. This is a must for compatibi-

lity with older CPUs (8088/86).  The problem is that in REAL MODE you can't

access memory above 1 MB... or, well, you can using special system drivers like

HIMEM.SYS that allow you to see memory as XMS. There are also EMM386, QEMM,

etc., for EMS memory.  Using these drivers is not so funny if you are a

programmer, expecially if you do it using Assembly Language. (C-programmers:

READ ON !). I almost HATE SYSTEM DRIVERS very much, and believe you do the same.

Even most normal users (not programmers) hate them, having problems configuring

their MSDOS system with multiple CONFIG.SYS and AUTOEXEC.BAT because an appli-

cations wants XMS, another wants EMS, another wants NONE, etc. It's a mess !!!

Remember: XMS & EMS (expecially the latter) are SLOW and troubleful.

Another very BIG problem of REAL MODE is memory segmentation.

You can only access a segment of 64 KiloBytes at a time. If you have, say,

a vector of 67000 bytes to fill it all you must switch between two segments

because 67000 exceeds the 64 KB limits... hey, WAKE UP ! We got 386, 486,

Pentiums, stop programming on a 8-bit machine !!! Stop using pentiums like

TURBO 8088s !!!

What can we do to avoid all these problems ??? We could use PROTECTED MODE.

This is a very powerful but complicated CPU mode, with segments, protection,

priorities, and so on... the biggest problem is that it is NOT compatible

with normal MSDOS operations and to use it within MSDOS you need a DOS-EXTENDER:

a program that "interfaces" MSDOS with your powerful application.

DOS-EXTENDERS are very complex, memory consuming, and difficult to use.

Ok... let's get to the point.

If you join together the ADVANTAGES of REAL MODE and the ADVANTAGES of PROTECTED

MODE ... What you get ?  UNREAL MODE !!!

I repeat: this is NOT a joke. I called it this way 'cause your programs will

run in real mode, like common ones, but they have immediate full access to

4 GigaBytes of memory, WITHOUT USING ANY LAME DRIVER/EXTENDER, WITHOUT MAKING

ALL THOSE SYSTEM CALLS TO STRANGE DRIVERS.

Just run UNREAL.EXE before !  Ok.. keep on reading.

1.1 : HOW UNREAL MODE IS MADE POSSIBLE

It takes advantage of a strange feature (or BUG ??) of the 386/486/Pentium

CPU that makes it possible to destroy segment limits (64 KBytes) so that you

can access the full 4 GigaBytes address space without using Protected Mode.

Maybe also the 286 has this bug but it's no use because you need special 386

addressing modes.

These adressing modes are usually illegal in REAL MODE and cause the system to

HALT until you perform a RESET.

An example (REAL MODE):

.386					;We use 386 code !

mov	byte ptr [ds:0ffffh],0		;LEGAL ! No problems.

mov	byte ptr [ds:10000h],0		;ILLEGAL ! MACHINE HALTS !

The second instruction will crash the system because you can't exceed the

64 KBytes segment limit (0ffffh = 64KBytes).

To get past this strong limitation you must run UNREAL.EXE !

This program just enters protected mode.. makes some mess with the segment

registers and exits leaving the system in UNREAL MODE !!!

It is NOT A TSR, it WASTES NO MEMORY.  JUST ACTIVATE THE BUG AND QUIT.

The system will not suffer from that, cause there are no problems with that !

You can do SOMETHING MORE, and NOTHING LESS than before.  I tested it with many

programs, Windows in 386 enhanced mode too, and they all work fine like they

ever did !!! Do NOT expect they run any faster than before !!!	Programs will

run much faster only if purposely designed for UNREAL MODE !

I hope MicroSoft will include this feature in future releases of MSDOS as

built-in since it is new power for MSDOS without drawbacks !

An example (UNREAL MODE):

; Assume you got 8 MegaBytes of RAM and you want to clear the last megabyte

; (from absolute linear address 700000h to 800000h).

.386				;We use 386 code !

xor	ax,ax			;AX = 0

mov	ds,ax			;DS = AX = 0  ..Use LINEAR ADDRESSING !

mov	EAX,700000h		;Start is beginning of 8th MegaByte

Clear:	mov	byte ptr [ds:EAX],0	;Clear memory !!!

inc	EAX

cmp	EAX,800000h

jnz	Clear			;Loop until done !

No special assembler required. I use old TASM and it works just perfect !

Rember to use the ".386" directive to assemble 386 instructions !!!

Can you feel the power ?? Linear addressing using real .. ahem, UNREAL MODE !

Access easily and without extenders up to 4 GigaBytes of memory using 386

special addressing modes !

Be sure to run UNREAL.EXE before trying these examples or your machine will

hang till a reset.

1.2 : FUTURE RELEASES

Watch out for the soon coming UMM386 (UNREAL MEMORY MANAGER) !!!

1.3 : FINAL WORDS !

Don't worry, this is NO JOKE. I tested it with a great deal of software and

hardware and it worked OK ! Let me know !

Contact me:

EMAIL : paccalon@linux.infosquare.it

paccalon@ghost.dsi.unimi.it

MAIL  : Daniele Paccaloni

Via Agadir 10/A,

20097 S.Donato (MILAN)

ITALY