T.A.P. Version 2.0 (C) 1989-1996

(Triode Amplifier Program)

FREEWARE

Courtesy of KD9JQ

T.A.P. Version 2.0 (Triode Amplifier Program) provides the Amateur with a new

Tool in Designing Triode Power Tube Amplifiers for Grounded Grid Service.

This Program should be considered as a designers aid only and does not represent

all facets of High Power Amplifier design.  It is left to the user to verify

correct Filament Voltages and Currents for the frequency of operation, to verify

the correct plate voltage and dissipations per operating class, and as well as

the proper cooling and mounting methods recommended by the tube vendor. The user

does not need to have to Plate/Grid-Voltage/Current Plots to use this program

unless a new device data file is required.

The following information will guide the designer with Menus and Prompts.

Start:

1.  Press <CR> to pull up the Library files of typical triodes.

2.  Type in the file name (.TUB extension assumed) and press <CR>

3.  A menu will pop up indicating 4 possible design selections or Exit.

F1 will find IBdc, RL Given PO, VDC, VBmin

F2 will find IBdc, PO Given RL, VBmin, VDC

F3 will find VBmax, PO Given IBdc, RL, VBmin, VDC

F4 will find VBmin, PO Given RL, IBdc, VDC

F9 Exit  "Ends Program"

4.  A series of prompts will appear requesting input from the designer.

Enter the requested information at this time.  The program utilizes

Data from the Tube Library File to simulate the Tube Characteristics

at VBmin (Plate Voltage at full IB).

VDC = DC Plate Voltage @ no load.

Guideline;

Plate voltage should not exceed

VDC (Maximum rated) - (.5 * Vp-p Drive)

VBmax = Maximum Instantaneous Plate Voltage at IB @ cutoff.

Should not exceed VDC.

Page 2

VMmin = Minimum Instantaneous Plate Voltage at Peak Plate Current

** Hint **  Use 10% +/- 5% of VB to start.

Lower  VMmin => higher Grid current, higher drive

Higher VMmin => lower Grid current, lower drive

Guideline;

VKmax < .75 * VBmin  where VKmax = Peak Grid Voltage

VP = VBmax - VBmin  (1/2 Plate Swing)

IBdc = DC Plate Current at Full Power

PO = Output Power Desired (Po to Load will reflect FeedThru Drive)

RL = Plate Load in Ohms (Final Load reflects FeedThru Drive)

FO = Operating Frequency (Mhz)

N  = Number of Tubes used in Parallel (Not for Push Pull)

CLASS =   C ( 165 Deg Conduction)

B ( 180 Deg Conduction)

AB1 ( 185 Deg Conduction

AB2 (~191 Deg Conduction)

USER (Conduction Angle or Bias)

USER Option:

You will be prompted for [A] Angle or [B] Bias.

The Angle allows the user to select a particular conduction

angle from 140 to 220 Degrees.  The corresponding Bias voltage will be

calculated.

The Bias option allows you to enter a particular fixed voltage. The

corresponding Conduction Angle will be calculated.  This option is

useful when using zero bias triodes.  A bias of "0" volts must be

entered as a small number such as .0001.

5.  A list of the calculated Design parameters will now be displayed.

Any exceeded values will flash red.

6.  To obtain Input/Output Matching circuit data press either the up or

down arrow keys.

7.  A display of matching circuits for input and outputs is now shown.

Input T or PI Matching circuit component values can be obtained by

Toggling the TAB key.

Page 3

VG = Peak Cath Voltage Swing

IB = Peak Plate Current   IB(DC) = IB/Kb

IG = Peak Grid  Current   IG(DC) = IB/Kg

CA = Conduction Angle (Radians)

VK = Cathode Bias Voltage     VK =(Cos(CA/2)*(VG+VBmin/U))+VDC/U)

Kb = PI^2/CA (Radians)        Kg is a function of VK/(VG+VK)

Expected efficiency ~= (1 - (1/Kb)) * 100 %

PDCinput = VDC * IBdc  Po = VP^2/(2*RL)

PDISS = PDC - Po

N  (efficiency) = Po/PDC * 100 %

Po to Load = Po + Pfdthru  where Pfdthru = (VG+VK)^2/(2*RB)

PGDISS = (VG+VK)^2/(2*RG)  Grid Dissipation

RB = 2*(VG+VK)/IB      RG = 2*(VG+VK)/IG

RK = 2*(VG+VK)/(IB+IG)  Parallel Real component of Input Impedance

XP = -1 / (WO * CP)

CP = Cath-Grid Capacitance.

where WO = 2 * Pi * Fo

Converting to Series

RS =  RP/((RK/XP)^2 + 1)  XS = -RP*RS/XP

Zin = SQR(RS^2+XS^2)  Magnitude of Zin

Phase Angle = 2*ATN(XS/(RS+Zin))

PIN = (VG+VK)^2/(2*Zin)

The Output is designed for a PI design with an additional C/L

circuit for intermediate impedance step up Transformation.  If only

the PI circuit is desired, the user can input a new transformation

Impedance equal to the Load (50 ohms).  The Plate Tuning Capacitance

has the RFC Plate Choke and Tube Capacitance reactance removed.

Items 1 thru 8 can be changed by the user at this time.

Select the Item number to change and enter the new value.  Enter a

Negative number when entering Inductance, and a Positive number for

Capacitance.  Only used for RFC's.  <CR> to update screen to new

values.

Toggling the UP or Down Arrow Keys will alternately pop up the Data

Display or Matching Circuits.

8.  The center Menu allows for "Dynamic" design of the matching

circuits.  To use this option, you must have initially designed the

amplifier at the low end of the Band of interest and at the highest

expected output power with the F1 selection.  Print out Data at

this time.

Press "D" and the program will prompt you for new power output level

(Lowest) and Upper Frequency end of the Band.  The screen will now

display Matching circuit values required while maintaining constant

inductor values.  Loaded Q and capacitance values are recalculated.

Print out the new Data at this time.

MENU will return you to the main menu.

FILE will return you to the .Tub Library.

PRINT will output the Data to your Printer.

QUIT will exit program.

Page 4

Note:  The designer should account for drive and bias current

equalization of parallel devices.  If there is adequate grid and

plate dissipation then the problem may not pose a problem at HF.

Circuit component losses are not figured in to the Gain.  Use of

High Q components for low losses are required in high power

amplifiers.

Example:

A QL of 12 to QU of 400 => (1-(QL/QU))*100% efficiency or .22dB Loss

in the Tank Circuit.  Subtract all Input & Output LC losses from

calculated Gain.

REFERENCES

RADIO HANDBOOK  22ND ED  By  William I. Orr  Section 7

RCA TRANSMITTING TUBE  TECHNICAL MANUAL  TT-5 10-62  Pages 46 - 62

CARE & FEEDING OF POWER GRID TUBES  VARIAN  4TH PRINTING 1982