ACCENT USER'S MANUAL.

NOTE: This disk version of Accent User's Manual is generated from

the original Manual for Accent to read.  Section 3.2.2 (TELEPHONE

OPTION COMMANDS) is removed for saving the disk space.  There is

no variation in the contents; only punctuation marks are added to

the end of the lines in the titles, tables, etc..

Aicom Corporation.

First printing, December 1986.

Second printing, April 1987.

Third printing, November 1988.

Fourth printing, April 1989

Copyright 1986, 1987, 1988, 1989 by Aicom Corporation.

All rights reserved.

Aicom Corporation believes the information in this

publication is accurate as of its publication date; such

information is subject to change without notice.  Aicom

Corporation is not responsible for any inadvertent errors.

Aicom is a registered trademark of Aicom Corporation.

Accent is a registered trademark of Aicom Corporation.

IBM is a registered trademark of International Business Machine Corporation.

IBM PC is a trademark of International Business Machine Corporation.

Touchtone is a registered trademark of American Telephone and

Telegraph Company.

Toshiba is a registered trademark of Toshiba International Corporation.

Accent User's Manual.

(Version 3.1)

Accent-PC,	for IBM PC/XT/AT & Compatibles.

Accent-mini,	for IBM PC/XT/AT & Compatibles.

Accent-1200,	for Toshiba T1200 & Compatibles.

Accent-SA,	Stand-Alone for all computers.

Accent-ST,	Accent-SA plus telephone interface for all computers.

Aicom Corporation, 2375 Zanker Road, Suite 205, San Jose, CA. 95131.

Telephone: (408) 922-0855.

Fax: (408) 922-0859.

TABLE OF CONTENTS.

1.		ABOUT THE MANUAL.

1.1. 		INTRODUCTION TO ACCENT PRODUCTS.

1.2.		STANDARD FEATURES FOR ALL ACCENT MODELS

1.3.		DESCRIPTIONS OF MODELS.

1.3.1.		ACCENT-PC.

1.3.2.		ACCENT-mini.

1.3.3. 		ACCENT-1200.

1.3.4.		ACCENT-SA.

1.3.5.		ACCENT-ST.

2.		INSTALLATION.

2.1.		INSTALLING ACCENT-PC.

2.1.1.		SWITCH SETTINGS.

2.1.2.		JUMPER SETTINGS.

2.1.3.		INSTALLING ACCENT'S SOFTWARE DEVICE DRIVER.

2.1.4.		INSTALLING ACCENT-PC HARDWARE.

2.1.5.		SPEAKER CONNECTION.

2.1.6.		RUNNING THE DEMO PROGRAM.

2.1.7.		EXTERNAL POWER SUPPLY FOR STAND-ALONE OPERATION.

2.2.		INSTALLING ACCENT-mini.

2.2.1.		JUMPER SETTINGS.

2.2.2.		INSTALLING ACCENT-mini SOFTWARE DEVICE DRIVER.

2.3.		INSTALLING ACCENT-SA.

2.3.1.		RS-232C CONNECTION.

2.3.2.		SPEAKER CONNECTION.

2.3.3.		POWERING UP ACCENT-SA.

2.3.4.		RUNNING THE DEMO PROGRAM.

2.4.		INSTALLING ACCENT-ST.

2.4.1.		TELEPHONE LINE CONNECTION.

2.5.		INSTALLING ACCENT-1200.

2.5.1.		INSTALLING ACCENT-1200 SOFTWARE DEVICE DRIVER.

3.		OPERATION.

3.1.		INTERFACE TO ACCENT.

3.1.1.		DEVICE DRIVER INTERFACE.

3.1.2.		RS-232C INTERFACE.

3.1.3.		TELEPHONE INTERFACE.

3.2.		SOFTWARE COMMANDS.

3.2.1.		SYSTEM OPTION COMMANDS.

3.2.1.1.	Reset.

3.2.1.2.	Initialize.

3.2.1.3.	Flush.

3.2.1.4.	RS-232C Echo On.

3.2.1.5.	RS-232C Echo Off.

3.2.1.6.	Enable Appending <CR> for Status Reply.

3.2.1.7.	Disable Appending <CR> for Status Reply.

3.2.1.8.	Enable Speech Synthesizer.

3.2.1.9.	Disable Speech Synthesizer.

3.2.1.10.	Speech Pause.

3.2.1.11.	Speech Continue.

3.2.1.12.	Host Input Time-out.

3.2.1.13.	Enable <CR> as Alternative Start-Speaking Character.

3.2.1.14.	Disable <CR> as Alternative Start Speaking Character.

3.2.1.15.	Enable CTL-Q/S as Xon/Xoff Characters.

3.2.1.16.	Disable CTL-Q/S as Xon/Xoff Characters.

3.2.1.17.	Flush Accent-ST Output Buffer.

3.2.1.18.	Enable Fast Processing for Speech.

3.2.1.19.	Disable Fast Processing for Speech.

3.2.1.20.	Enable DCD Verbal Message.

3.2.1.21.	Disable DCD Verbal Message.

3.2.1.22.	Enable Delayed Process for Speech Option Commands.

3.2.1.23.	Disable Delayed Process for Speech Option Commands.

3.2.1.24.	Enable Verbal Stress on Single-Character Alphabets.

3.2.1.25.	Disable Verbal Stress on Single-Character Alphabets.

3.2.1.26.	Set Audio Output Volume.

3.2.1.27.	Increase Overall Volume by One Step.

3.2.1.28.	Decrease Overall Volume by One Step.

3.2.1.29.	Increase Average Pitch by One Step.

3.2.1.30.	Decrease average pitch by One Step.

3.2.1.31.	Increase Speech Rate by One Step.

3.2.1.32.	Decrease Speech Rate by One Step.

3.2.1.33.	Increase Voice Characteristic by One Step.

3.2.1.34.	Decrease Voice Characteristic by One Step.

3.2.1.35.	Increase Space Pause Time by One Step.

3.2.1.36.	Decrease Space Pause Time by One Step.

3.2.1.37.	Increase Sentence Intonation by One Step.

3.2.1.38.	Decrease Sentence Intonation by One Step.

3.2.1.39.	Enable PC to RS-232C Transmission.

3.2.1.40.	Disable PC to RS-232C Transmission.

3.2.1.41.	Enable RS-232C to PC Transmission.

3.2.1.42.	Disable RS-232C to PC Transmission.

3.2.1.43.	Command String Sent to Auxiliary Port.

3.2.1.44.	Command String Not Sent to Auxiliary Port.

3.2.1.45.	Status Reply to Host Only.

3.2.1.46.	Status Reply to Auxiliary Port Only.

3.2.1.47.	Status Reply to Host and Auxiliary Port.

3.2.1.48.	Enable LPT Auxiliary Device Chained Operation.

3.2.1.49.	Disable LPT Auxiliary Device Chained Operation.

3.2.1.50.	Set PC Host.

3.2.1.51.	Set RS-232C Host.

3.2.1.52.	Enable CTL-X as the Instant Flush Command.

3.2.1.53.	Disable CTL-X as the Instant Flush Command.

3.2.1.54.	Enable Xon-Always-Sent after Flush.

3.2.1.55.	Disable Xon-Always-Sent after Flush.

3.2.1.56.	Enable Index Marker Auto-Prompt.

3.2.1.57.	Disable Index Marker Auto-Prompt.

3.2.2.		TELEPHONE OPTION COMMANDS.

3.2.2.1.	Off Hook.

3.2.2.2.	On Hook.

3.2.2.3.	Select DTMF Dialing.

3.2.2.4.	Select Pulse Dialing.

3.2.2.5.	Select Auto Answer Mode.

3.2.2.6.	Select Manual Answer Mode.

3.2.2.7.	Dial Number.

3.2.2.8.	Off Hook and Dial Number.

3.2.2.9.	Enable Telephone Prompt.

3.2.2.10.	Disable Telephone Prompt.

3.2.2.11.	Set Normal Transmission Gain.

3.2.2.12.	Enable 0.8dB Transmission Gain Pad.

3.2.2.13.	Enable 1.5dB Transmission Gain Pad.

3.2.2.14.	Enable DTMF Input in Digit Mode.

3.2.2.15.	Enable DTMF Input in String Mode.

3.2.2.16.	Disable DTMF Detector.

3.2.2.17.	Initiate a Dial-Tone Detect.

3.2.2.18.	Enable Auto-Flush at Valid DTMF Input.

3.2.2.19.	Disable Auto-Flush at Valid DTMF Input.

3.2.2.20.	Hang up Automatically at Speech-end.

3.2.2.21.	Flash Hook-Switch.

3.2.2.22.	Set DTMF String Mode Input Terminator.

3.2.2.23.	Set Number of Rings for Auto Answer.

3.2.2.24.	Set Dial-Tone Time-out.

3.2.2.25.	Initiate a DTMF Time-out.

3.2.3.		SPEECH OPTION COMMANDS.

3.2.3.1.	Text Mode.

3.2.3.2.	Spell Mode.

3.2.3.3.	Punctuation Not Spoken.

3.2.3.4.	Punctuation Spoken.

3.2.3.5.	<-> Not Spoken.

3.2.3.6.	<-> Spoken as DASH.

3.2.3.7.	<-> Spoken as MINUS.

3.2.3.8.	Enable Abbreviation Dictionary.

3.2.3.9.	Disable Abbreviation Dictionary.

3.2.3.10.	Full Set Punctuation Spoken.

3.2.3.11.	Subset Punctuation Spoken.

3.2.3.12.	<A> and <a> Always Spoken as /EI/.

3.2.3.13.	<A> and <a> are Context Dependant.

3.2.3.14.	Set Average Pitch.

3.2.3.15.	Set Voice Characteristic.

3.2.3.16.	Set Speech Rate.

3.2.3.17.	Set Sentence Level Intonation.

3.2.3.18.	Set Space Pause Time.

3.2.3.19.	Set Number Processor.

3.2.3.20.	Direct Parameter Input to Synthesizer Chip.

3.2.4.		STATUS REQUEST COMMANDS.

3.2.4.1.	Reply End of Speech.

3.2.4.2.	Reply Synthesizer Status.

3.2.4.3.	Reply Telephone Status.

3.2.4.4.	Reply DTMF Input.

3.2.4.5.	Reply Version Number.

3.2.5.		INDEX MARKER COMMANDS.

3.2.5.1.	Enable CTL-F as the Index Marker.

3.2.5.2.	Disable CTL-F as the Index Marker.

3.2.5.3.	Reply <ESC>Y plus Index-Marker Character for Auto-Prompt.

3.2.5.4.	Reply Index-Marker Character only for Auto Prompt.

3.2.5.5.	Enable Index-Marker-Count Replying for Status Reply.

3.2.5.6.	Disable Index-Marker-Count Replying for Status Reply.

3.2.5.7.	User Defined Index-Marker.

3.2.6.		TABLE-1 (Command Summary).

4.		STANDARD ACCENT TEXT PROCESSOR.

4.1.		PRONUNCIATION RULES.

4.1.1.		ABBREVIATIONS.

4.1.2.		ALTERNATIVE PRONUNCIATIONS.

4.1.3.		MONETARY $ AMOUNTS.

4.1.4.		NUMERALS.

4.1.5.		TIME OF DAY.

4.1.6.		WORDS WITH NO VOWEL.

4.2.		PHONEME INPUT.

4.2.1.		TABLE-2 (Accent Phoneme List).

5.		STANDARD ACCENT DCD UTILITY.

5.1.		DCD SOFTWARE OPERATION.

5.2.		RULES FOR BUILDING THE CUSTOM DICTIONARY.

5.2.1.		THE ROOT WORD.

5.2.2.		THE PART-OF-SPEECH.

5.2.3.		WORDS WITH TWO PRONUNCIATIONS.

5.2.4.		A SAMPLE DCD SOURCE FILE.

5.2.5.		ERROR MESSAGES.

6.		PROGRAMMING ACCENT.

7.		SPECIFICATIONS.

7.1.		ACCENT-PC SPECIFICATIONS.

7.2.		ACCENT-mini SPECIFICATIONS (Not ACCENT-1200).

7.3.		ACCENT-SA SPECIFICATIONS.

7.4.		ACCENT-ST SPECIFICATIONS.

LIST OF TABLES.

TABLE-1		(Command Summary).

TABLE-2		(Accent Phoneme List).

LIST OF FIGURES.

FIGURE-1,	Accent-PC Hardware.

FIGURE-2,	Accent-mini Hardware.

FIGURE-3,	Accent-SA and Accent-ST Back Panel.

1. ABOUT THE MANUAL.

It is extremely important for the user to read this manual

before trying to operate Accent, since the manual provides accurate,

complete information for ALL ACCENT MODELS regarding installations,

system options, and operations.  The manual also provides a guide

to applications.

This manual has seven Sections.

Section 1 serves as the general introduction to Accent products (models).

Section 2 tells how to connect each specific Accent model for operation.

Section 3 details Accent's hardware and software operation,

and particularly is important for advanced users wishing

to write application programs.

Section 4 describes the standard Accent's text processor;

the specific pronunciation rules are covered as well as the

phoneme input definitions.

Section 5 describes the operation for the Downloadable

Custom Dictionary (DCD) utility.

Section 6 provides hints and techniques for programming

Accent products in general applications.

Section 7 gives the hardware specifications.

It is suggested the user read through Section 1 for all Accent models,

then jump to model dependant sections from Section 2 on.

The < > are used throughout this manual to bracket an ASCII character

when confusion might occur, for example:

<ESC>		Represents the Escape character.

<CR>		Represents the Carriage Return character.

<LF>		Represents the Line Feed character.

<SP>		Represents the Space character.

<HT>		Represents the Horizontal Tabulation character.

<->		Represents the Hyphen character.

<ESC>WC		Represents the Escape, W, and C characters.

A table of contents, list of tables and list of figures have been provided

at the beginning of this manual.  Numbered paragraphs and subparagraphs

allow easy reference.

1.1. INTRODUCTION TO ACCENT PRODUCTS.

Accent speech synthesizer is a high quality text-to-speech convertor.

It converts ordinary ASCII text into intelligible speech.

Accent's unlimited vocabulary capability, plus its many feature options,

comprise a useful voice-output device for various applications.

To cover a wide range of the system configurations, cost requirements,

and environmental requirements, five different Accent models are available

to the user.  These five models are: Accent-PC for PC plug-in,

Accent-mini for PC plug-in, Accent-1200 for the Toshiba

laptop PC (model T1100-plus and T1200) plug-in,

Accent-SA for stand-alone operation, and Accent-ST for

stand-alone operation with direct telephone line interface.

For the PC plug-in models, the text and commands (three-character

escape sequences) are sent to Accent through a re-entrant device driver

provided, while the stand-alone models through an RS-232C serial port

with either hardware or Xon/Xoff protocol.

All Accent models employ the standard Accent text-processor

which can speak in two modes: the text mode and the spell mode.

In the text mode, Accent will convert text into ordinary speech,

employing grammatical rules for parsing phrases and sentences,

for words having two pronunciations, for the abbreviations, for the $ amounts,

for numbers, and for special signs (#, %, & and so on).  In the text mode,

Accent also employs punctuation marks for sentential intonations.

Through software commands, speech parameters such as average pitch,

Accent's voice characteristic, and speech rate, among others,

can be programmed.

In the spell mode, all text including punctuation will be spoken

letter by letter.

Accent's standard vocabulary is larger than 20,000 most

commonly used words with precise pronunciation.  For special,

customized vocabulary, a standard DCD (Downloadable Custom Dictionary) utility

is provided, in addition to the phoneme input capability.  The DCD

hardware is built in for all Accent models.

1.2. STANDARD FEATURES FOR ALL ACCENT MODELS.

Standard Accent text-processor; vocabulary over 20,000 most

commonly used words.

Four groups of software commands: system option commands,

speech option commands, index marker commands, and status request commands.

Speech rate can be adjusted from about 60 to 850 words per minute.

Control-X character as instant speech-flush command.

Average response time less than 20 milli-seconds in Fast-processing mode.

Demo/Utility diskette including DCD development software.

8k-byte DCD hardware built in.

On-board audio power amplifier with volume control; 0.5 Watt.

3.5mm miniature phone jack for 8-Ohm speaker, 4-Ohm speaker, or headphone.

1.3. DESCRIPTIONS OF MODELS.

The following sections describe the model dependant

features for all five different Accent models.  Throughout this manual,

the general term "Accent" represents all Accent models.

1.3.1. ACCENT-PC.

Accent-PC is a self-sufficient, single-board microcomputer

with a 640-byte, wrap-around text buffer.  It can be plugged directly

into any IBM PC, PC/XT, PC/AT or the compatibles.

A standard RS-232C serial port is also provided for interface

with a computer, terminal, or printer.

The user can select either the PC or the RS-232C as Accent-PC's host.

Text and software commands are sent from the host to Accent-PC for processing

by an on-board processor.  Communication between the PC and RS-232C

can also be set up by software commands.

FEATURES.

On-board microprocessor, 640-byte wrap-around text buffer.

IBM PC bus compatible; I/O addresses switch selectable.

Standard RS-232C serial port as a DCE device; baud rate, parity and data bits

switch selectable.

Power connector provided for stand-alone operation or system integrations.

2k-byte re-entrant device driver provides PC interface either

through DOS (device name) or BIOS (LPT port) requiring no CPU time overhead

or any hardware interrupt from PC for the realtime speech processing.

SETACNT.EXE utility program for changing Accent-PC's device name,

I/O addresses, auxiliary device (LPT) number, and chained-LPT operations.

Four groups of software commands: system option commands,

speech option commands, index marker commands, and status request commands.

1.3.2. ACCENT-mini.

Accent-mini consists of a half-length plug-in card and a re-entrant

software device driver for the desktop IBM PC and compatibles.

It requires 123k-byte memory and one hardware interrupt from the PC

for realtime speech processing.

Accent-mini's utility program SETMINI.EXE is a powerful

tool for users to set Accent-mini's system parameters and

configurations, such as changing the Device Name,

selecting the I/O addresses and Hardware Interrupt,

assigning Accent-mini to any LPT number as an auxiliary

device, setting the chained operation for multiple devices

using the same Interrupt, etc.

FEATURES.

Realtime text-to-speech synthesis; hardware interrupt driven.

2k-byte wrap-around internal phoneme buffer.

SETMINI.EXE utility program for changing Accent-mini's device name,

I/O addresses, hardware interrupts, auxiliary device (LPT) number, chained

operations, etc.

IBM PC bus compatible; I/O addresses and Interrupt jumpers selectable

in conjunction with SETMINI.EXE utility.

Four groups of software commands: system option commands,

speech option commands, index marker commands, and status request commands.

All commands and interface format compatible with Accent-PC device driver.

1.3.3. ACCENT-1200.

Accent-1200 is functionally identical to Accent-mini,

except that it has an on-board EPROM.  It uses the same

device driver as Accent-mini uses.

The hardware is built with low-power CMOS circuits.  An on-board 2 inch

speaker is provided for portability, and a 3.5 mm miniature, stereo phone jack

is also provided for an external 8-Ohm speaker, 4-Ohm speaker, or headphone.

This model is to be plugged in the Modem slot of the Toshiba T1100-plus

or the T1200 models laptop PC.  The memory requirement for the device driver

is 12k bytes.  All software features, commands and operations of

Accent-mini also apply to Accent-1200.  Read Accent-mini sections of

this manual for instructions for programming and operating Accent-1200.

Accent-1200 fully supports the T1200's RESUME and EMM.SYS features.

1.3.4. ACCENT-SA.

Accent-SA is the stand-alone version of the Accent-PC.

It is a self-sufficient, microcomputer subsystem with a 640-byte wrap-around

text buffer housed in a compact metal case.

A standard RS-232C serial port is provided for interface

with a computer or terminal as the host.  A wall-mount

AC adaptor is provided for power.

FEATURES.

Self sufficient stand-alone text-to-speech convertor.

640-byte wrap-around text buffer.

Standard RS-232C serial port as a DCE device; baud rate, parity and data bits

switch selectable; hardware protocol as default, Xon/Xoff protocol may be

selected via software command.

Wall-mount AC adaptor included.

Power On/Off switch.

Internal speaker.

Four groups of software commands: system option commands,

speech option commands, index marker commands, and status request commands.

1.3.5. ACCENT-ST.

Accent-ST consists of an Accent-SA synthesizer plus

built-in direct telephone line interface.  In addition to all

Accent-SA's features, Accent-ST can answer calls, initiate

calls, transfer calls, detect DTMF (Touchtone) signals and send them

to the host in the ASCII format, and accept text from the host then

deliver the speech to the caller.

Accent-ST has two wrap-around buffer, the 640-byte input buffer,

and the 64-byte output buffer. The input buffer is used to store input text

and commands, while the output buffer is used to store the DTMF digits detected

and the status data to be replied to the host.

FEATURES.

All Accent-SA features and capabilities; refer to Section 1.3.4.

64-byte wrap-around output buffer.

Two standard RJ-11 telephone jacks.

Five groups of software commands: system option commands,

telephone option commands, speech option commands, index marker commands,

and status request commands.

2. INSTALLATION.

The installation for each specific model may be carried out by the user,

and is described in the following sections.

2.1. INSTALLING ACCENT-PC.

This Section describes step-by-step how to install the

Accent-PC in the PC for standard operation, and how to connect to

an external power supply for stand-alone operation (custom system-integrations

only).  When setting up the hardware, please refer to Figure-1 which shows

the locations of the switches, jumpers, and connectors on Accent-PC's

printed circuit board.

2.1.1. SWITCH SETTINGS.

SW1 SETTINGS.

SW1 is used to set up the RS-232C baud rate, the parity

bit and the length of the data bits.  Factory default is set

to select 9600 baud rate, parity disabled, and 7 data bits.

To change the baud rate, parity option or data bit length,

please refer to Section 3.1.2.  The default settings for SW1

have positions 4 and 8 ON and positions 1, 2, 3, 5, 6, and 7 OFF.

SW2 SETTINGS.

SW2 is used to assign PC's I/O addresses to Accent-PC.

Factory default addresses are set to 2A8-2AB Hex which

will not conflict with any PC, PC/XT, or PC/AT standard

or reserved I/O device.  The default settings for SW2

have positions 1, 3, 5, 7 ON and 2, 4, 6, 8 OFF.  Should

these default addresses conflict with any custom built

I/O device or if there is to be more than one Accent-PC

installed in a single PC, please refer to the following

paragraphs for appropriate address settings, and to

Section 3.1.1 for changing the same addresses in

Accent-PC's device driver.

Four consecutive I/O addresses of the PC bus are

possessed by each Accent-PC.  The address always starts

with an even number as a multiple of four.  The starting

address may be selected by setting SW2 as shown in

Figure 1.  SW2 is an 8 position switch, each position

(starting from 1) corresponding to an address bit (starting

from 0) of the PC bus is listed as follows:

SW2 POSITION:	1	2	3	4	5	6	7	8

ADDRESS BIT:	A2	A3	A4	A5	A6	A7	A8	A9

When a SW2 position is set to "ON", its corresponding PC

address bit is set to "0", and vice versa.  "A0" and "A1"

are ignored (they are "0" for calculating the starting

address) when setting up SW2.

2.1.2. JUMPER SETTINGS.

J2 SETTINGS.

J2 is used to select Accent-PC's host.  Factory default

setting selects the PC as host.  To select the PC as host,

pin 1 and pin 2 of J2 should be jumpered together, using

the jumper header provided.  To select the RS-232C as

host, pin 2 and pin 3 of J2 should be jumpered together.

J4 SETTINGS.

J4 is used to connect the Reset signal from the PC bus to

Accent-PC. Factory default provides jumpered J4.  If J4 is

jumpered, whenever the Reset signal from the PC bus is

active (asserted), Accent-PC will be reset.  If J4 is not

jumpered, Accent-PC will automatically reset itself only

during power-up.  To use external power supply for

stand-alone operation, J4 must not be jumpered.

FIGURE-1,   Accent-PC Hardware.

(Not shown for the disk version).

2.1.3. INSTALLING ACCENT'S SOFTWARE DEVICE DRIVER.

From the Demo/Utility diskette provided, copy the

following files to a system diskette or the hard disk

which is to be used with Accent :

CONFIG.SYS

SPKACNT.DVC

DEMO.BAS

DEMO.TXT

(NOTE: Use SPKMINI.DVC for Accent-mini or

Accent-1200 instead of SPKACNT.DVC)

If there are other device drivers already installed in the system diskette,

the file CONFIG.SYS should reside in the system diskette.

If this is the case, then instead of copying the file CONFIG.SYS from

the Demo/Utility diskette, simply add (edit) the following statement in the

resident file CONFIG.SYS :

DEVICE=SPKACNT.DVC

or

DEVICE=SPKMINI.DVC  for Accent-mini or Accent-1200.

Re-boot the PC with the system diskette, and if the

message "Accent-PC Device Driver (Version 3.1) Installed", or

"Accent-mini Device Driver (Version 3.1) Installed" for Accent-mini

or Accent-1200, appears on the screen then Accent's device driver

has been successfully installed.  If not, carefully go through the device

driver installation procedures again.

Accent's default device name and auxiliary device name

are "SPK" and "LPT3" respectively.  After the hardware

installation, COPYing any text or text file to either "SPK"

or "LPT3" will cause Accent to speak the COPYed text or

text file.  Also after the hardware installation, every time

Accent device driver is loaded, Accent will verbally

prompt "ACCENT READY.".

Accent's demonstration program DEMO.BAS is written in

IBM PC's BASIC Language, so that the system diskette to

be used with Accent must contain the BASIC Interpreter

"BASIC.COM" or "BASICA.COM".

2.1.4. INSTALLING ACCENT-PC HARDWARE.

After turning off all power to the PC, Accent-PC may be

plugged in the PC.  First, the bottom of the metal bracket

on the Accent-PC board should be slid into the bracket

slit located at the edge of the PC mother board.  Then Accent-PC

should be held upright and pushed symmetrically down into the PC

edge connector.

After installing the screw tightly to secure Accent-PC's

metal bracket against the PC frame, the volume-control

knob may be installed.  Push it onto the volume-control shaft,

but note that the knob can be pushed on only when the flat side

of the shaft is aligned with the flat side of the knob's opening hole.

Pushing on the knob is necessary only if it had removed before

installing the synthesizer.

2.1.5. SPEAKER CONNECTION.

An external speaker may be connected using a 3.5 mm male miniature

phone plug (mono).  An 8-Ohm or 4-Ohm speaker is recommended.

2.1.6. RUNNING THE DEMO PROGRAM.

Only when the Accent-PC is properly installed, a speaker

has been connected, and the volume has been turned up

to approximately one-third (clock-wise), may the PC be powered up.

When the PC prompts, type in "BASIC DEMO"

or "BASICA DEMO" (DEMO232 for Accent-SA and Accent-ST) to execute

the demonstration program.  If Accent says "AICOM..." at this point,

Accent has been successfully installed.  If not, turn off the power

and go through the installation procedures again, making sure,

among other things, that the switch settings are correct

for your particular application.

In the DEMO diskette provided, there is a document file named "READ.ME"

which explains the applications of each file provided.

2.1.7. EXTERNAL POWER SUPPLY FOR STAND-ALONE OPERATION.

If Accent-PC is to be integrated into a custom system for

stand-alone operation, power to Accent-PC must be

provided through PS1.  PS1 is a 5-pin male power

connector (Molex 09-64-1051 compatible).  Any suitable

female connector may be used to plug into PS1.  Power

requirements are listed in Section 7.  Please note that

stand-alone operation is for advanced users only.  The Warranty is void for,

and does not cover, stand-alone operation.

Power connections for PS1 are the following:

Pin-1,	GND (Ground).

Pin-2,	Key (not used).

Pin-3,	-12 volt.

Pin-4,	+5  volt.

Pin-5,	+12 volt.

Pin-6,	Not used.

2.2. INSTALLING ACCENT-mini.

This Section describes step-by-step how to install

Accent-mini in the PC. If your PC does not have any

non-standard card, the default settings of Accent-mini's

hardware and software device driver should function

properly without any change.

If your PC has any non-standard card, which uses I/O addresses 3EE-3EF Hex

or hardware interrupt IRQ2/IRQ9 (IRQ2 is named for the PC/XT, and IRQ9 is

named for the PC/AT), please refer to Figure-2 to resolve any conflict,

and to Section 3.1.1 to change the same parameters in Accent-mini's device

driver before the installation.

When setting up the hardware, please refer to Figure 2

which shows the locations of the jumpers and connectors

on Accent-mini's printed circuit board.  For some PCs,

Accent-mini's volume control knob needs to be

unplugged before the card can be plugged in the PC.

FIGURE-2, Accent-mini Hardware.

(Not shown for the disk version).

2.2.1. JUMPER SETTINGS.

J1 SETTINGS.

J1 is used to select the PC's hardware interrupt for Accent-mini.

Factory default setting selects IRQ2/IRQ9 (IRQ2 for the PC/XT,

or IRQ9 for the PC/AT).

J2 SETTINGS.

J2 is used to assign PC I/O addresses to Accent-mini.

Two consecutive I/O addresses are used by each Accent-mini.

Factory default addresses are set to 3EE-3EF Hex.

2.2.2. INSTALLING ACCENT-mini SOFTWARE DEVICE DRIVER.

The DOS version 2.11 or higher is required to run Accent-mini.

With a system diskette containing DOS-2.11 or higher,

the rest of the installation procedure is the same as the Accent-PC.

Go to Section 2.1.3 through 2.1.6 to install the software device driver

and the hardware, and to run the Demo program.

2.3. INSTALLING ACCENT-SA.

This Section describes how to connect Accent-SA to the

AC adaptor, speaker, and RS-232C for operation.

When setting up the hardware, please refer to Figure-3

which shows Accent-SA's back panel.

FIGURE-3, Accent-SA and Accent-ST Back Panel

(Not shown for the disk version).

2.3.1. RS-232C CONNECTION.

Accent-SA's (and Accent-ST's) RS-232C interface is

configured as a DCE device. Please refer to Section 3.1.2

if software Xon/Xoff handshaking is used.  All unused

output pins (signals) from Accent-SA or Accent-ST

SHOULD NOT be connected to any output pin (signal)

from the host.  Caution: pin 22 of the 25-pin D-connector

is an output from Accent-ST indicating the ringing of the

telephone line.

SW1 is used to set up the RS-232C baud rate, the parity bit and the length

of the data bits.  Factory default is set

to select 9600 baud rate, parity disabled, and 7 data bits.

To change the baud rate, parity option and data bit length,

please refer to Section 3.1.2.  The default settings

for SW1 have positions 4 and 8 ON, and positions 1, 2, 3, 5, 6, and 7 OFF.

2.3.2. SPEAKER CONNECTION.

Accent-SA has an internal speaker.  If desired,

an external speaker may be connected using a 3.5 mm male miniature

phone plug (mono).  An 8-Ohm or 4-Ohm speaker is recommended.

2.3.3. POWERING UP ACCENT-SA.

Turn on the Accent-SA power switch only after the following have

been done:

Accent-SA is properly set up and connected to a host.

An external speaker has been connected, if desired.

The volume has been turned up to approximately 1/3 (clock-wise).

The AC adaptor has been plugged into the wall (Notice the AC adaptor

can be plugged into Accent-SA in one direction only).

Accent's power-on message, "ACCENT READY.", should be heard.

If the message is not heard, turn off the power

and carefully go through the installation procedures again.

2.3.4. RUNNING THE DEMO PROGRAM.

Accent-SA may be demonstrated with an IBM PC.

First, connect a straight-through, 25-pin RS-232C cable from the PC's COM1

to Accent-SA's RS-232C port.  Second, from the Demo/Utility diskette provided,

copy the DEMO232.BAS and DEMO.TXT files to a system diskette containing

IBM PC's BASIC Interpreter "BASIC.COM" or "BASICA.COM".

Then go to Section 2.1.6 to run the Demo program.

2.4. INSTALLING ACCENT-ST.

This Section describes how to connect Accent-ST's RS-232C,

telephone line interface, speaker jack, and AC adaptor for operation.

When setting up the hardware, please refer to Figure-3 which shows

Accent-ST's back panel.

2.4.1. TELEPHONE LINE CONNECTION.

Facing Accent-ST's front panel, two standard RJ-11

telephone jacks are located at the right side of

Accent-ST's metal case.  For telephone applications,

the telephone line can be plugged directly into either of the

two RJ-11 jacks.  A telephone set can also be plugged

into either of the RJ-11 jacks for normal use.

Since Accent-ST consists of an Accent-SA, it's basic

operation is identical to Accent-SA's operation.  Please go

to Section 2.3.1 for the rest of the installation.

2.5. INSTALLING ACCENT-1200.

Accent-1200 hardware is recommended for dealer installation.

Installation is not discussed here.

2.5.1. INSTALLING ACCENT-1200 SOFTWARE DEVICE DRIVER.

The DOS version 2.11 or higher is required to run Accent-1200.

With a system diskette containing DOS-2.11 or higher,

the rest of the software installation procedure is the same as the Accent-PC.

Go to Section 2.1.3 and Section 2.1.6 to install the software device driver

and to run the Demo program.

3. OPERATION.

This Section describes Accent's user-interface, telephone

interface (for Accent-ST only), and software command

options.  The hardware setup during the installation will

define Accent's interface options, and the software

commands will allow users to change the system options,

the speech options, the telephone options (for Accent-ST

only), and to request status from Accent.

The host computer communicates with Accent as

a peripheral device either via the software device driver

or via the RS-232C port.

If Accent is used with a software device driver on a PC,

the PC BUS protocol is automatically handled in the device driver.

The user-interface to the software device driver is explained in Section 3.1.1.

If the RS-232C interface is used, the handshaking for

the RS-232C can either be hardware (Default) or Xon/Xoff type.

The RS-232C interface is described in Section 3.1.2.

3.1. INTERFACE TO ACCENT.

The following three sections (Section 3.1.1 through 3.1.3)

are model dependant, the user may refer to the

appropriate sections for any specific model.

3.1.1. DEVICE DRIVER INTERFACE.

This section is for Accent-PC and Accent-mini only, since

they are to be plugged into a PC.  A device driver is

used so that an application program (usually written in a High Level Language)

may communicate with a specific device such as a disk drive, a printer,

or a COM port.  If the system diskette used to boot the PC has a device driver

in it, the PC will automatically load the device driver during boot time.

Each device driver has its specified name.

Accent's default device name is SPK.

After the device driver has been loaded into the PC, any

application program can OPEN or CLOSE the device as

a disk file, READ and WRITE may be performed to that device.

The device names are reserved (can't be used elsewhere) by the system

once they are installed.

Since Accent is a special device (not a standard device,

like a standard PC printer), a custom-built device driver

is needed to allow the user to access Accent, that is, to

OPEN, CLOSE, READ and WRITE Accent like a standard

device with a specific name.  In this case, the name of

Accent's character device driver is SPK.

Please refer to IBM PC DOS Technical Reference manual

for detailed descriptions of Installable Device Drivers.

Accent's device driver is a character device driver, and it

provides the functions of INIT, INPUT, and OUTPUT as

described in the DOS Technical Reference.

Although Accent's device drivers are re-entrant, the Calls

to DOS will not be re-entrant.  Therefore an alternative

entry is provided -- the LPT port.  Since Calls to LPT

port (by using INT 17H) is through the BIOS, which is

re-entrant, only Calls from LPT to Accent will be

re-entrant.  The default LPT port for Accent is set to

LPT3.  The following shows the LPT interface definitions:

FUNCTION:	Print Character.

ENTRY:		Same as IBM BIOS.

RETURN:		Same as IBM BIOS.

NOTE:		None.

FUNCTION:	Initialize.

ENTRY:		Same as IBM BIOS.

RETURN:		Same as IBM BIOS.

NOTE:		Accent will be Flushed.  This includes all previous inputs,

text and commands. FOR A QUICK FLUSH, USE THIS FUNCTION.

FUNCTION:	Status Read.

ENTRY:		Same as IBM BIOS.

RETURN:		Same as IBM BIOS for AH bit-0, and bit-3 to bit-7.

AH bit-2,1 = 00 for not end-of-speech.

AH bit-2,1 = 01 for complete end-of-speech.

AH bit-2,1 = 10 for near end-of-speech (about 0.3

to 2 second to end.)

DH = Index-marker-count (only if <ESC>YZ command has

been previously issued.)

NOTE:		With the index marker feature enabled, if a constant

status-check loop is performed to the device driver LPT BIOS

call, Status Read, the loop time-period is recommended

to be at least 20 milli-seconds.  There is no time constraint

if the index marker feature is disabled.

If Factory default setups for the system parameters and

configurations are not desired, the utility program

SETACNT.EXE for Accent-PC, or SETMINI.EXE for

Accent-mini, may be used to change them.  Please refer

to the menu when running the SETACNT.EXE or SETMINI.EXE.

3.1.2. RS-232C INTERFACE.

This section is not for Accent-mini.  Accent's RS-232C is

a standard serial interface port for Data Communication Equipment (DCE).

If hardware hand-shaking is adopted, the RTS, DTR, CTS and DSR signals

must be used/checked for proper operation.  If software Xon/Xoff

handshaking is adopted, the software command <ESC>=V must be sent to

Accent first after power-up for proper operation.

After Accent has received the <ESC>=V command, the CTS and DSR signals

will be asserted by Accent at all times, and the RTS and DTR signals from

the host will not be checked by Accent.

For Accent-ST, the RING signal (pin 22 of the 25-pin D-connector) will be

asserted by Accent-ST when the telephone line is ringing,

and will continue to be asserted until the telephone line stops ringing,

or until the call has been answered by Accent-ST or by the telephone set

that is plugged into Accent-ST.

To insure hardware engineers of a clear understanding of the RS-232C signals,

each signal on the 25-pin RS-232C D-connector is listed below for reference.

25-PIN		I/O			RS-232C		RS-232C

CONNECTOR	TO/FROM			SIGNAL		SIGNAL

PIN NUMBER	ACCENT			NAME		SOURCE

1		BOTH			PGND		BOTH

2		INPUT			TXD		DTE

3		OUTPUT			RXD		DCE

4		INPUT			RTS		DTE

5		OUTPUT			CTS		DCE

6		OUTPUT			DSR		DCE

7		BOTH			SGND		BOTH

20		INPUT			DTR		DTE

22		OUTPUT			RING		DCE

The RS-232C options include baud rate, parity, and data

bit selections. SW1 (refer to FIGURE-1 for Accent-PC, and

FIGURE-3 for Accent-SA and Accent-ST) is used to set

these options, as follows:

BAUD RATE.

SW1 POSITION:	1	2	3	4	5

75		ON	OFF	OFF	OFF	ON

150		OFF	ON	OFF	OFF	ON

300		ON	OFF	OFF	OFF	OFF

600		OFF	ON	OFF	OFF	OFF

1200		OFF	OFF	ON	OFF	ON

2400		OFF	OFF	OFF	ON	ON

4800		OFF	OFF	ON	OFF	OFF

9600		OFF	OFF	OFF	ON	OFF

PARITY.

SW1 POSITION:	6	7

NONE		OFF	OFF

ODD		ON	OFF

EVEN		ON	ON

The data bit length is set up by SW1 position 8:  ON for 7 data bits,

OFF for 8 data bits.  The stop bit is fixed at 1 bit.

Factory default settings for these options are: 9600 baud,

no parity, and 7 data bits.

3.1.3. TELEPHONE INTERFACE.

This section is for Accent-ST only.  Two standard RJ-11

telephone jacks (in parallel) are provided for connecting

to the telephone line and to a standard telephone set.

The telephone functions are controlled via the software commands.

If Accent-ST is used for speaker output only, the

telephone line should be disconnected, or the Disable

Telephone Prompt (Section 3.2.2.10) and Manual Answer

(Section 3.2.2.6) software commands should be issued to

Accent-ST to prevent any unwanted telephone prompt

(such as ringing, answering, etc.) from Accent-ST to the host.

3.2. SOFTWARE COMMANDS.

Once Accent hardware has been set up, operation is

completely under control of the host.  Accent can accept

two kinds of data from its host: text (including phoneme input), and commands.

Text may be translated into speech and commands will control

Accent's operation.  For telephone applications, a typical programming is

described in Section 6.

In the text-to-speech mode, input text from the host is

stored in Accent's text buffer and will not be spoken

until a sentence punctuation mark is received, the

time-out described in Section 3.2.1.12 is encountered, or a

<CR> is received if the option in Section 3.2.1.13 is enabled.

Section 4 describes Accent's text processing rules in detail.

In the spell mode, the text input will be spelled out character

by character immediately after it is received.

Commands are sent from the host to Accent as escape

sequences.  An escape sequence is a string of ASCII

characters preceded by an <ESC> character (1B Hex and 27 decimal).

There are five groups of commands: System option commands,

telephone option commands, speech option commands, status request commands

and index marker commands.

An Accent command consists of the <ESC> character

immediately followed by two more characters.  Unless

otherwise noted, the speech option commands and the

status request command <ESC>?S will not be executed

until all text, previously sent from the host, has been

processed, that is, until Accent's internal text buffer has been emptied.

Other commands will be executed immediately upon receipt of the command.

Illegal Escape Sequences SHOULD NOT be sent to Accent.

Each command is described in detail in the following

sections.  A Commands Summary table (Table-1), which

also gives Accent's software default settings, is located in Section 3.2.6.

In these commands, the upper and lower cases are distinguished.

The model dependant commands are noted in the descriptions.

3.2.1. SYSTEM OPTION COMMANDS.

Various system option commands are available for

different application configurations.

3.2.1.1. Reset.

<ESC>=R

Reset Accent, equivalent to power up reset.  If the

RS-232C is used for host interface, allow at least one

second for Accent to complete its self-diagnostics.  No

data should be sent to Accent during this two-second

period or the data will be lost.  "ACCENT READY."

verbal message will be issued by Accent.  All options

will be reset to Accent's default states.

3.2.1.2. Initialize.

<ESC>=X

Reset Accent, equivalent to power up reset but with no

power-on verbal message replied.  If the RS-232C is used

for host interface, allow at least 100 milli-seconds for

Accent to complete its self-diagnostics.  No data should

be sent to Accent during this 100 milli-seconds period or

the data will be lost.

3.2.1.3. Flush.

<ESC>=x

All Accent's internal text/speech buffers will be flushed

and speech will be stopped immediately, all options and

status remain unchanged.  This command is most

frequently used to flush the speech.  Important notice:

Also refer to Section 3.2.1.52 through 3.2.1.55 for

Control-X character as instant Flush command and for

Xon-always-sent after Flush.

3.2.1.4. RS-232C Echo On.

<ESC>=E

This command is not for Accent-mini.  Characters

received from Accent's RS-232C will be echoed back to

the sending device immediately.

3.2.1.5. RS-232C Echo Off.

<ESC>=e

This command is not for Accent-mini.  Characters

received from Accent's RS-232C will not be echoed back

to the sending device.  This is the power-up default state.

3.2.1.6. Enable Appending <CR> for Status Reply.

<ESC>=C

The reply to the Status Request Commands in Section

3.2.4 will have a <CR>, Carriage Return, appended at the end.

This command is useful for application software

written with High-Level Languages.  This is the power-up default state.

3.2.1.7. Disable Appending <CR> for Status Reply.

<ESC>=c

The reply to the Status Request Commands in Section 3.2.4 will not

have a <CR> appended at the end.

3.2.1.8. Enable Speech Synthesizer.

<ESC>=A

Text received from the host will be converted to speech.

3.2.1.9. isable Speech Synthesizer.

<ESC>=a

Text received from the host will not be converted to

speech and will not be stored in the text buffer either.

All other options are unaffected.

3.2.1.10. Speech Pause.

<ESC>=Q

Speech will be stopped (paused) immediately, or after the

current syllable is spoken if Accent is speaking.  Further

text input from the host will be stored in the text buffer

and not spoken until a Speech Continue Command

described in Section 3.2.1.11 is received.  If Accent is in

the Speech Pause state, only the Speech Continue, Reset,

Initialize, Flush, Set Average Pitch, Set Speech Rate, and

Set Voice Characteristic commands will be accepted.

3.2.1.11. Speech Continue.

<ESC>=q

Accent will continue to speak, beginning where speech

was stopped (paused) by the Speech Pause Command

described in Section 3.2.1.10.

3.2.1.12. Host Input Time-out.

<ESC>Tn

This command is not for Accent-mini.  The n in this

escape sequence represents any digit 0 through 9, which

determines the time-out period while Accent awaits text

input from the host.  The time-out period ranges

approximately from 0.04 seconds to 20 seconds for digits

0 through 9.  Any higher digit doubles the time-out

period of the preceding lower digit.

With any occurrence of time-out, if there is text in the

text buffer with no end-of-sentence punctuation or a

terminating <CR> (if the option in Section 3.2.1.13 is

enabled), Accent will begin to speak out the text. Default

time-out is set to 7, which gives a time-out period of about 5 seconds.

3.2.1.13. Enable <CR> as Alternative Start-Speaking Character.

<ESC>=F

Accent will start speaking when a <CR> is received.  This

command is useful for Screen-Review applications such as

terminal for the visually impaired.  However, it may

slightly affect sentence intonation.

3.2.1.14. Disable <CR> as Alternative Start-Speaking Character.

<ESC>=f

Accent will start speaking only when a sentence

punctuation mark is received, or the host input time-out

is encountered. This is the default condition.

3.2.1.15. Enable CTL-Q/S as Xon/Xoff Characters.

<ESC>=V

This command is not for Accent-mini. Xon/Xoff software

handshaking will be turned on for the RS-232C.  The Xon

character is Control-Q (11 in Hex) and the Xoff character

is Control-S (13 in Hex).

IMPORTANT NOTE: If Xon/Xoff is to be used, this

command should be sent to Accent immediately after

power-up.  After Accent has sent an Xoff to the host,

there are still 128 bytes free space for input.  If further

inputs exceed the 128-byte limit, buffer overrun will

occur.  For Accent-PC, Accent's host must be set to

RS-232C before this command can be issued.

3.2.1.16. Disable CTL-Q/S as Xon/Xoff Characters.

<ESC>=v

This command is not for Accent-mini. Accent's Xon/Xoff

software handshaking will be turned off.  The RS-232C

protocol is controlled by the RTS, DTR, CTS, and DSR

hardware signals.  This is the power-up default state.

3.2.1.17. Flush Accent-ST Output Buffer.

<ESC>=Z

The following description of this escape sequence is for

Accent-ST only.  This escape sequence has a different

meaning for Accent-PC and Accent-mini device driver

interface; refer to Section 3.2.1.48 and 3.2.1.49.

Accent-ST's 64-byte wrap-around output buffer will be

flushed.  This command is used when error condition has

occurred during either DTMF input or call processing.

3.2.1.18. Enable Fast Processing for Speech.

<ESC>=B

If Accent's speech buffer is empty, Accent will process

the input text immediately after a phrase or a word is

received.  This command is mainly used in applications

requiring fast response time such as in the Screen-Review

applications. Note that if this option is enabled with high

speech rate, the sentence intonation will be affected.

3.2.1.19. Disable Fast Processing for Speech.

<ESC>=b

Accent will process the text when a complete sentence is

received. This is the normal and default condition.

3.2.1.20. Enable DCD Verbal Message.

<ESC>=J

Upon completion of a DCD download, Accent will deliver

a verbal message of "Custom Dictionary Download

Completed".  This is the power-up default state.

3.2.1.21. Disable DCD Verbal Message.

<ESC>=j

Upon completion of a DCD download, Accent will not

deliver any verbal message.  This command may be used

if more than one set of DCD are to be used while Accent

is active.

3.2.1.22. Enable Delayed Process for Speech Option Commands.

<ESC>=K

Speech option commands will only be processed after all

text, previously sent from the host, has been processed.

This is the normal and default operation condition.

3.2.1.23. Disable Delayed Process for Speech Option Commands.

<ESC>=k

Speech option commands will be processed immediately

after they are received. This command is used only when

a instant speech parameter change (such as pitch, speech

rate, etc.) is required.

3.2.1.24. Enable Verbal Stress on Single-Character Alphabets.

<ESC>=U

Every single-character alphabet will be verbally stressed

when spoken by Accent.  This command is useful when

Accent is reading a string of alphabets and/or mixed

with digits such as a serial number.

3.2.1.25. Disable Verbal Stress on Single-Character Alphabets.

<ESC>=u

Normal text-processing rule will be employed to

pronounce single-character alphabets.

Verbal stress/non-stress is context dependant.  This is the default state.

3.2.1.26. Set Audio Output Volume.

<ESC>An

The n in this escape sequence represents any digit 0

through 9, which determines the overall volume of the

audio output.  The digit 9 has the highest volume which

is the default state.  Unless definitely needed for special

effect, this parameter should be left as 9 for the best

resolution.  The volume is recommended to be controlled

by the volume control knob.

3.2.1.27. Increase Overall Volume by One Step.

<ESC>+A

The overall volume will be increased by one step

instantaneously.  NOTE: All <ESC>+ and <ESC>- related

commands will be processed instantly, and if the

parameter has reached its upper or lower limit, it will

stay at its limit.  These zoom features are useful for

locating the most comfortable voice for the user.

3.2.1.28. Decrease Overall Volume by One Step.

<ESC>-A

The overall volume is decreased by one step.

3.2.1.29. Increase Average Pitch by One Step.

<ESC>+P

The average pitch is increased by one step.

3.2.1.30. Decrease average pitch by One Step.

<ESC>-P

The average pitch is decreased by one step.

3.2.1.31. Increase Speech Rate by One Step.

<ESC>+R

The speech rate is increased by one step.

3.2.1.32. Decrease Speech Rate by One Step.

<ESC>-R

The speech rate is decreased by one step.

3.2.1.33. Increase Voice Characteristic by One Step.

<ESC>+V

The voice characteristic is increased by one step.

3.2.1.34. Decrease Voice Characteristic by One Step.

<ESC>-V

The voice characteristic is decreased by one step.

3.2.1.35. Increase Space Pause Time by One Step.

<ESC>+S

The space pause time is increased by one step.

3.2.1.36. Decrease Space Pause Time by One Step.

<ESC>-S

The space pause time is decreased by one step.

3.2.1.37. Increase Sentence Intonation by One Step.

<ESC>+M

The sentence level intonation is increased by one step.

3.2.1.38. Decrease Sentence Intonation by One Step.

<ESC>-M

The sentence level intonation is decreased by one step.

3.2.1.39. Enable PC to RS-232C Transmission.

<ESC>=P

This command is for Accent-PC only.  Characters received

from PC will immediately be sent to Accent-PC's RS-232C port.

Refer to Section 3.2.1.43 and 3.2.1.44 for command character string.

3.2.1.40. Disable PC to RS-232C Transmission.

<ESC>=p

This command is for Accent-PC only.  Characters received

from PC will not be sent to the RS-232C port.

3.2.1.41. Enable RS-232C to PC Transmission.

<ESC>=S

This command is no longer supported.

3.2.1.42. Disable RS-232C to PC Transmission.

<ESC>=s

This command is for Accent-PC only.  Characters received

from RS-232C port will not be sent to PC.  This is the default state.

3.2.1.43. Command String Sent to Auxiliary Port.

<ESC>=L

This command is for Accent-PC only.  Command

character string (the three-character escape sequence) will

be sent to the auxiliary port if the transmission path is

enabled through Section 3.2.1.39 or 3.2.1.41. Note: If PC is

set to be Accent-PC's host, then the RS-232C port is

considered to be Accent's auxiliary port, and vise versa.

This command has no effect if Accent-PC is set to RS-232C host.

3.2.1.44. Command String Not Sent to Auxiliary Port.

<ESC>=l

This command is for Accent-PC only.  Command

character string will not be sent to the auxiliary port

whether or not the transmission path is enabled through

Section 3.2.1.39 or 3.2.1.41.  This is the default condition.

3.2.1.45. Status Reply to Host Only.

<ESC>=G

This command is for Accent-PC only.  The reply to the

Status Request Commands in Section 3.2.4 will be sent to

the host only.  This is the default condition.

3.2.1.46. Status Reply to Auxiliary Port Only.

<ESC>=g

This command is for Accent-PC only.  The reply to the

Status Request Commands in Section 3.2.4 will be sent to

the auxiliary port only.

3.2.1.47. Status Reply to Host and Auxiliary Port.

<ESC>=H

This command is for Accent-PC only.  The reply to the

Status Request Commands in Section 3.2.4 will be sent to

the host and to the auxiliary port.

3.2.1.48. Enable LPT Auxiliary Device Chained Operation.

<ESC>=Z

The description of this escape sequence is for Accent-PC

and Accent-mini PC device drivers only.  This escape

sequence has different meaning for Accent-ST, refer to

Section 3.2.1.17.

Characters received from LPT auxiliary device will be

processed by Accent, and the same characters will also be

sent to the original software interrupt vector (by a FAR

JUMP) previously saved by the device drivers during

system-boot.  For example, if the LPT number is set to 1

(by SETACNT.EXE or SETMINI.EXE) and the LPT chain

flag is set, all text sent to Accent via LPT1 or PRN will

be spoken by Accent, and will also be sent to the

standard system printer to be printed.

3.2.1.49. Disable LPT Auxiliary Device Chained Operation.

<ESC>=z

This command is for Accent-PC and Accent-mini device

drivers only. Characters received from LPT auxiliary

device will be processed by Accent only and will not be

sent to the standard system printer.

3.2.1.50. Set PC Host.

<ESC>HP

This command is for Accent-PC only.  Set the PC as

Accent-PC's host.  When Accent-PC is first powered up,

the host is selected by Hardware J2 as described in

Section 2.1.2.  This command allows users to change the

host and utilize the auxiliary port for communication

with a second host or with a peripheral, as desired.

Note: when Accent-PC's host is changed by a set-host

command, all options and status in Accent remain valid,

except that they will then be referred to the newly

designated host.

3.2.1.51. Set RS-232C Host.

<ESC>HS

This command is for Accent-PC only.  Set the RS-232C as

Accent's host.

3.2.1.52. Enable CTL-X as the Instant Flush Command.

<ESC>=M

This command will enable the Control-X character (18

Hex) as the instant Flush command.  After this command

has been issued to Accent, both the <ESC>=x and the

Control-X will cause Accent to flush.  The Control-X will

flush instantly since it is a single-character command.

3.2.1.53. Disable CTL-X as the Instant Flush Command.

<ESC>=m

Only the <ESC>=x command will cause Accent to flush.

This is the default state.

3.2.1.54. Enable Xon-Always-Sent after Flush.

<ESC>=I

This command is not for Accent-mini.  If Accent is in

Xon/Xoff protocol mode, this command will cause Accent

to always send an Xon to the host after Accent has been

Flushed by the host.  This Xon is mainly used to signal

the host that all index marker Auto-Prompts (as described

in Section 3.2.1.56) have been sent to the host.

3.2.1.55. Disable Xon-Always-Sent after Flush.

<ESC>=i

This command is not for Accent-mini.  If Accent is in

Xon/Xoff protocol mode, Accent will send an Xon to the

host after a Flush only if an Xoff has been previously

sent to the host.  This is the default state.

3.2.1.56. Enable Index Marker Auto-Prompt.

<ESC>=Y

This command is not for Accent-mini.  This command

will enable Accent to automatically prompt each index

marker to the host via the RS-232C, when Accent has

spoken to the position of each index marker.

3.2.1.57. Disable Index Marker Auto-Prompt.

<ESC>=y

This command is not for Accent-mini.  No index marker

will be prompted automatically.  This is the default state.

3.2.2. TELEPHONE OPTION COMMANDS.

IMPORTANT NOTE FROM AICOM: THIS SECTION IS REMOVED FOR SAVING DISK SPACE.

3.2.3. SPEECH OPTION COMMANDS.

This command group applies to all Accent models, it

allows user to tailor Accent to speak text and punctuation

marks in a number of desired formats according to the

requirements of a specific application.  Furthermore, the

speech rate, the average (baseline) pitch, the voice

characteristic, the Space pause time, sentence level

intonation, as well as the internal abbreviation dictionary

and schwa sound manipulations can be programmed through these commands.

3.2.3.1. Text Mode.

<ESC>OT

Text from the host will go through Accents text processor for speech output.

This is the normal text-to-speech operation.

3.2.3.2. Spell Mode.

<ESC>Ot

Text from the host will be spoken character by character

(i.e., spelled out) by Accent.  This includes all special

signs and punctuation marks. For listing, please refer to Section 4.1.

3.2.3.3. Punctuation Not Spoken.

<ESC>Op

No punctuation mark will be spoken in the Text Mode.

This is the normal text-to-speech operation.

3.2.3.4. Punctuation Spoken.

<ESC>OP

Punctuation marks will be spoken as described in Section

3.2.3.10 and Section 3.2.3.11.  This command may be used,

for example, in applications where the punctuation marks

must be spoken out precisely, such as in reading a

program written in a High-Level Language.

3.2.3.5. <-> Not Spoken.

<ESC>Om

The <-> sign will not be spoken by Accent.

This is the normal text-to-speech operation.

3.2.3.6. <-> Spoken as DASH.

<ESC>OM

The <-> sign will be spoken as DASH by Accent.

3.2.3.7. <-> Spoken as MINUS.

<ESC>ON

The <-> sign will be spoken as MINUS by Accent.

3.2.3.8. Enable Abbreviation Dictionary.

<ESC>OI

Accent's internal abbreviation dictionary will be searched

during text-to-speech rule processing.  This is the default condition.

3.2.3.9. Disable Abbreviation Dictionary.

<ESC>Oi

Accent's internal abbreviation dictionary will not be

searched during text-to-speech rule processing.

3.2.3.10. Full Set Punctuation Spoken.

<ESC>OR

If Punctuation Spoken command is enabled as in Section 4.2.3.4,

all punctuation marks will be spoken as described in Section 4.1.

This is the default condition.

3.2.3.11. Subset Punctuation Spoken.

<ESC>Or

If Punctuation Spoken command is enabled as in Section 4.2.3.4,

only  ,.:;!?  punctuation marks will be spoken as described in Section 4.1.

3.2.3.12. <A> and <a> Always Spoken as /EI/.

<ESC>OA

The character <A> and <a> will always be pronounced as /EI/ phoneme

rather than context dependant.

3.2.3.13. <A> and <a> pronunciation are Context Dependant.

<ESC>Oa

The character <A> will always be pronounced as /EI/ phoneme,

while the character <a> is context dependant.

In most cases, the <a> will be pronounced as the schwa phoneme /UU/.

3.2.3.14. Set Average Pitch.

<ESC>Pn

The n in this escape sequence represents any digit, 0

through 9, which determines the average pitch level of Accent.

The range of the average pitch is from about

70 Hz to 140 Hz for digits 0 through 9. The nominal and

default pitch level for Accent is 5, which gives an average

pitch of about 115 Hz.

3.2.3.15. Set Voice Characteristic.

<ESC>Vn

The n in this escape sequence represents any digit, 0

through 9, which determines the characteristic of Accent's voice.

The lower numbers will give lower and deeper

sounding voices (like those from a male with a larger head).

The higher numbers will give higher and sharper sounding voices.

Usually, when the average pitch is set at

a lower number, the voice characteristic will also be set at

a lower number to match, and vice versa.

However, for special applications such as Games, this

parameter may be set to any number and, in conjunction

with other Speech Option Commands, will provide two

thousand voices representing different personalities.

The default number for this parameter is set to 5 which will

give a male voice of average tone.

3.2.3.16. Set Speech Rate.

<ESC>Rn

The n in this escape sequence represents any digit, 0

through F (in Hex), G, and H, which determines the speed

of Accent's speaking (speech rate).  With the Space Pause

Time set to 0, the speech rate runs from about 120 words

per minute to about 850 words per minute for those 18

digits from 0 to H.  The nominal and default rate for

Accent-ST is set to 5 which gives a speech rate of about

180 words per minute.

3.2.3.17. Set Sentence Level Intonation.

<ESC>Mn

The n in this escape sequence represents digits 0 through 4.

If n is 0, the regular (full) intonation will be

employed, and if n is 1, a robotic monotone pitch (using

the average pitch level set by the command described in

Section 3.2.3.14), ranging from 60 Hz to 150 Hz, will be employed.

If n is 2, 3, or 4, the regular intonation is employed yet

the sentence level intonation is reduced with respect to n=0,

while n=4 has the least sentence level intonation reduction,

and n=2 the most sentence level intonation reduction.

For high speech rate output, for example, in Screen-Review applications,

the reduced intonation (or the monotone) will provide a more

comfortable listening. For proof-reading type of applications,

the reduced intonation is also useful.  This command applies to both

the text mode and the spell mode.

3.2.3.18. Set Space Pause Time.

<ESC>Sn

The n in this escape sequence represents any digit, 0

through 9, which determines the pause time for the Space

character between two words. Lower numbers represent

shorter pauses, and higher numbers represent longer pauses.

The default is set to 0, which gives the most natural sounding speech.

3.2.3.19 Set Number Processor

<ESC>Nn

The n in this escape sequence represents any digit, 0, 1, and 2,

which determines Accent's number processing level.  Accent's number

processor is described in Section 4.1.3 and 4.1.4.

For n=0, Accent's number processor is turned on.  If a word has number(s)

mixed in it, the word portion will be spelled out automatically.  This is

the power up default state.

For n=1, Accent's number processor is turned on.  If a word has number(s)

mixed in it, the word protion will be spoken as a word.  This mode is most

commonly used in the Screen-Review applications for faster speech delivery.

For n=2, Accent's number processor is turned off; all numbers will be

spoken digit by digit.  If a word has number(s) mixed in it, the word

portion will be spoken as a word.

3.2.3.20 Direct Parameter Input to Synthesizer Chip

<ESC>|~..~

This command is for advanced users only; it allows the host to send

parameters directly to the synthesizer chip on the Accent board.  The

synthesizer chip on the Accent board is an SSI263A.

The .. in this escape sequence represent the parameters (in ASCII Hex numbers)

to be sent to the synthesizer chip.  The Direct Parameter Input may be

terminated anytime with a ~ (tilda) character.  The Interrupt will be

handled internally.  To use this feature, the parameters must meet the

following conditions.  Refer to the chip's specification for parameter

definitions.

Use upper case Hex numbers only: 0-9, A-F.

Only the Phoneme Response and the Transitioned Pitch mode are allowed.

The 5-byte parameters should be sent in reverse order: the Filter-Frequency

byte first, the Phoneme and Duration byte last.

The lower nibble of any given byte is sent first.

Upon receiving the first 5-byte of the parameters (10 characters), Accent

will start speaking.

The parameters sequence may be infinitely long.

3.2.4. STATUS REQUEST COMMANDS.

There are five commands for the host to request specific

status from Accent. If a constant status-check loop is

performed to Accent via the RS-232C port, the loop

time-period is recommended to be at least 50 milli-seconds.

There is no loop time constraint if the interface is via the PC device drivers.

The Reply Synthesizer Status command is most frequently used

for checking Accent's speech buffer status.

The Reply Telephone Status command is used to check

Accent-ST's line status, especially useful when the host is

constantly polling in multi-line applications.

The Reply DTMF Input command is used to request Accent-ST's DTMF input

when the automatic Telephone Prompt option is disabled.

3.2.4.1. Reply End of Speech.

<ESC>?S

When speech has ended, Accent will reply with a capitalized character <S>.

3.2.4.2. Reply Synthesizer Status.

<ESC>?s

This request will be immediately replied with a status-character by Accent.

If the synthesizer is idle (not speaking, speech ended),

Accent will reply with a capitalized character <S>.

If the synthesizer is speaking, and is not near to the end,

Accent will reply with a lower-case character <s>.

If the synthesizer is speaking but near to the end (about 0.3 second

to 2 seconds of speech left, dependant on the speech rate), Accent will

reply with an <M> for more text to have uninterrupted speech.

For host-interface using the RS-232C, the index-marker-count will

be appended right after the status-character if (and only if) the

index-marker-count replying is enabled as described in Section 3.2.5.5.

3.2.4.3. Reply Telephone Status.

<ESC>?W

This command is for Accent-ST only.  Accent-ST will

immediately reply with an ASCII digit (numbered from 0 to 7)

indicating telephone line status.  The replied line status are listed

as follows:

0 :	Idle (on hook and not ringing).

1 :	Ringing (on hook).

2 :	Off hook.

3 :	DTMF digit or digit string ready (off hook).

4 :	DTMF input time-out (off hook).

5 :	Dial-tone detected from <ESC>WK command (off hook).

6 :	Dial-tone detect time-out from <ESC>WK command (off hook).

7 :	Dial-tone detect time-out from <ESC>WC command (on hook).

3.2.4.4. Reply DTMF Input.

<ESC>?w

This command is for Accent-ST only.  Accent-ST will

immediately reply with the DTMF digit most recently

received from the telephone line (if in digit mode), or

with the DTMF digit string (if in string mode).  After the

reply, the status is reset.  Note: Whenever Accent-ST's

automatic Telephone Prompt option (Section 3.2.2.10) is

disabled, this command should be sent immediately after

Accent-ST has replied that a DTMF digit or digit string is ready.

3.2.4.5. Reply Version Number.

<ESC>|1

Accent will speak out its Version number for identification.

3.2.5. INDEX MARKER COMMANDS.

The index marker feature is mainly used in the

Screen-Review applications.  It allows the application

programs to track exactly which word in a given text

Accent has spoken to, so that the screen cursor (or

pointer) movement can be synchronized with the speech.

Accent can accept two kind of index markers, the

Control-F character or an user defined escape-sequence.

The Control-F is recommended since it takes less memory

space and less time to process.  The marker will be

treated as a Space character by the Accent text-processor,

and it is recommended to be placed right after the word to be marked.

If the host-interface is through Accent's RS-232C port,

the host has two methods to obtain Accent's marker status:

through the Auto-Prompt feature or through Accent's

Status-Reply command <ESC>?s.  With the Auto-Prompt

feature (Section 3.2.1.56) enabled, Accent will

automatically prompt the index marker character to the

host each time a marked word has been spoken (also refer

to Section 3.2.1.54 and 3.2.1.55).  With the Status-Reply

command enabled for replying the index-marker-count,

Accent will append the index-marker-count (in one-byte

binary form) to the status-character (S, M, or s), and then

clear Accent's index-marker-counter for a new count.

Either of the above two methods may be used, but NOT BOTH at the same time.

If the host-interface is through Accent's device driver,

the index marker is supported only through the LPT BIOS

call, Status Read.  With the Status-Reply command

enabled for replying the index-marker-count, on return of

the LPT BIOS call, Status Read, the DH register contains

the index-marker-count (in one-byte binary form), and

Accent's index-marker-counter is cleared for a new count.

Refer also to Section 3.1.1.

3.2.5.1. Enable CTL-F as the Index Marker.

<ESC>YX

Accent will treat the Control-F character (06 Hex) as the index marker.

3.2.5.2. Disable CTL-F as the Index Marker.

<ESC>Yx

Accent will not treat the Control-F character (06 Hex) as the index marker.

This is the default state.

3.2.5.3. Reply <ESC>Y plus Index-Marker Character for Auto-Prompt.

<ESC>YY

This command is not for Accent-mini.  The <ESC>Y plus

the index-marker character (one character) will be sent to

the host via the RS-232C port when Accent is prompting

the index marker in the Auto-Prompt mode. This three-character prompt

is used only if the host needs an escape-sequence for index marker prompt.

3.2.5.4. Reply Index-Marker Character only for Auto-Prompt.

<ESC>Yy

This command is not for Accent-mini. Only the

index-marker character (one character) will be sent to the

host via the RS-232C port when Accent is prompting the

index marker in the Auto-Prompt mode.  This is the default state.

3.2.5.5. Enable Index-Marker-Count Replying for Status Reply.

<ESC>YZ

For host-interface using the RS232C, the index-marker-count

will be appended for replying the <ESC>?s command.

For host-interface using device driver, the index-marker-count

will be returned in the DH register for LPT BIOS call, Status Read.

Note that the index-marker-count should be checked frequent enough

so that it does not overflow.  It is recommended that the

time between two checks should not be over five seconds

if every word in the text has a marker.

3.2.5.6. Disable Index-Marker-Count Replying for Status Reply.

<ESC>Yz

No index-marker-count will be replied.  This is the default state.

3.2.5.7. User Defined Index-Marker.

<ESC>Y_

This three-character escape-sequence is the user defined index-marker.

The <_> character represents the user defined index-marker character

which may be any character A-W, a-w, or 0-9. Once a <_> is used,

it should not be altered.  This user defined index-marker is used

only if the host must use an escape-sequence as the index-marker.

3.2.6. TABLE-1 (Command Summary).

The following Table summarizes the software commands for all Accent models.

The model dependant commands are denoted as follows:

*	For Accent-PC only.

**	For all models except Accent-mini or Accent-1200.

***	For Accent-ST only.

****	For Accent-PC, Accent-mini and Accent-1200's device drivers only.

SEL 	Selected for power-up default.

The columns are in the following order: Command, Note, Default, Description.

System Option Commands:

<ESC>=R			Reset Accent.

<ESC>=X			Initialize Accent per H/W set-up.

<ESC>=x			Flush all internal buffers.

<ESC>=E	**		RS-232C echo on.

<ESC>=e	**	SEL	RS-232C echo off.

<ESC>=C    	SEL	Append <CR> for Status Reply.

<ESC>=c			Append no <CR> for Status Reply.

<ESC>=A   	SEL	Enable speech synthesizer.

<ESC>=a			Disable speech synthesizer.

<ESC>=Q			Speech pause.

<ESC>=q	   	SEL	Speech continue.

<ESC>=F			Enable <CR> as alternative start-speaking character.

<ESC>=f	   	SEL	Disable <CR> as alternative start-speaking character.

<ESC>=B			Enable fast processing mode.

<ESC>=b	   	SEL	Disable fast processing mode

<ESC>=J	   	SEL	Enable DCD verbal message.

<ESC>=j			Disable DCD verbal message.

<ESC>=K	   	SEL	Process Speech Option Commands.

only when speech buffer is empty.

<ESC>=k			Process Speech Option Commands immediately.

<ESC>=U			Verbal stress on single character.

<ESC>=u	   	SEL	No stress on single character.

<ESC>=V	**		Enable CTL-Q/S as Xon/Xoff char.

<ESC>=v	**	SEL	Disable CTL-Q/S as Xon/Xoff.

<ESC>=Z	***		Flush Accent-ST's output buffer.

<ESC>Tn	** 	7	Host input time-out; n = 0 to 9.

<ESC>An	   	9	Set Audio Volume; n = 0 to 9.

<ESC>+A			Increase Audio Volume by 1 level.

<ESC>-A			Decrease Audio Volume by 1 level.

<ESC>+P			Increase Average Pitch by 1 level.

<ESC>-P			Decrease Average Pitch by 1 level.

<ESC>+R			Increase Speech Rate by 1 level.

<ESC>-R			Decrease Speech Rate by 1 level.

<ESC>+V			Increase Voice Characteristic by 1 level.

<ESC>-V			Decrease Voice Characteristic by 1 level.

<ESC>+S			Increase Space Pause Time by 1 level.

<ESC>-S			Decrease Space Pause Time by 1 level.

<ESC>+M			Increase Sentence Intonation by 1 level.

<ESC>-M			Decrease Sentence Intonation bY 1 level.

<ESC>=P	*		Enable PC to RS-232C transmission.

<ESC>=p	*	SEL	Disable PC to RS-232C transmission.

<ESC>=S	*		Enable RS-232C to PC transmission.

<ESC>=s	*	SEL	Disable RS-232C to PC transmission.

<ESC>=L	*		Command string sent to Aux port.

<ESC>=l	*	SEL	Command string not sent to Aux port.

<ESC>=G	*	SEL	Status Reply to Host only.

<ESC>=g	*		Status Reply to Aux port only.

<ESC>=H	*		Status Reply to Host and to Aux port.

<ESC>=Z	****		Enable chained LPT operation for PC via device driver.

<ESC>=z	****	SEL	Disable chained LPT operation for PC via device driver.

<ESC>HP	*	SEL	Set PC Host.

<ESC>HS	*		Set RS-232C Host.

<ESC>=M			Enable CTL-X as the instant Flush command.

<ESC>=m	  	SEL	Disable CTL-X as the instant Flush command.

<ESC>=I	**		Enable Xon-always-sent after Flush.

<ESC>=i	**	SEL	Disable Xon-always-sent after Flush.

<ESC>=Y	**		Enable index marker Auto-Prompt.

<ESC>=y	**	SEL	Disable index marker Auto-Prompt.

Speech Option Commands:

<ESC>OT	   	SEL	Text mode.

<ESC>Ot			Spell mode.

<ESC>Op	   	SEL	Punctuation will not be spoken.

<ESC>OP			Punctuation will be spoken.

<ESC>Om	   	SEL	<-> will not be spoken.

<ESC>OM			<-> will be spoken as DASH.

<ESC>ON			<-> will be spoken as MINUS.

<ESC>OI	   	SEL	Enable Accent's standard abbreviation table.

<ESC>Oi			Disable Accent's standard abbreviation table.

<ESC>OR 	SEL	Full-set punctuation spoken.

<ESC>Or			Subset punctuation (,.;:!?) spoken .

<ESC>OA			<A> or <a> always spoken as /EI/ phoneme.

<ESC>Oa		SEL	<A> always spoken as /EI/ phoneme,

<a> is spoken as /EI/ or UU/ dependant on context.

<ESC>Pn	   	5	Average pitch level; n = 0 to 9.

<ESC>Vn		5	Voice characteristic; n = 0 to 9.

<ESC>Rn	   	5	Speech rate; n = 0 to F, G, and H.

<ESC>Mn	   	0	Sentence level intonation; n = 0 (full intonation),

1 (monotone), 2, 3, or 4.

<ESC>Sn	   	0	Space pause time; n = 0 to 9.

<ESC>Nn		0	Set number processor; n=0, 1, 2.

<ESC>|~..~		Direct parameter input to synthesizer chip.

Status Request Commands:

<ESC>?S			Reply <S> when speech has ended.

<ESC>?s			Reply synthesizer statua immediately;

<S> for speech-end,

<s> for speech-not-end,

and <M> speech-near-end.

<ESC>?W	***		Reply telephone status.

<ESC>?w	***		Reply DTMF input.

<ESC>|1			Reply Accent's Version verbally.

Index Marker Commands:

<ESC>YX			Enable CTL-F as the index marker.

<ESC>Yx	   	SEL	Disable CTL-F as the index marker.

<ESC>YY	**		Reply <ESC>Y plus index-marker character

for Auto-Prompt.

<ESC>Yy	**	SEL	Reply index-marker character only for Auto-Prompt.

<ESC>YZ			Enable index-marker-count replying for Status Reply.

<ESC>Yz		SEL	Disable index-marker-count replying for Status Reply.

<ESC>Y_			User defined index-marker.

Telephone Option Commands:  ***

<ESC>WO			Off hook (seize line).

<ESC>Wo	  	SEL	On hook.

<ESC>WD	  	SEL	Select DTMF for dialing.

<ESC>Wd			Select Pulse for dialing.

<ESC>WA	  	SEL	Select Auto Answer mode.

<ESC>Wa			Select Manual Answer mode.

<ESC>W=n..n/		Dial "n..n" digits; 19 digits maximum.

<ESC>WCn..n/		Off hook and dial "n..n" digits.

<ESC>WE		SEL	Enable telephone prompt.

<ESC>We			Disable telephone prompt.

<ESC>WG	  	SEL	Normal transmission gain.

<ESC>Wg			0.8 dB higher than normal gain.

<ESC>WH			1.5 dB higher than normal gain.

<ESC>WI			Enable DTMF detect in digit mode.

<ESC>Wi			Enable DTMF detect in string mode.

<ESC>Wj  	SEL	Disable DTMF detect.

<ESC>WK			Initiate a dial-tone detect.

<ESC>WX	  	SEL	Enable Auto Flush for DTMF input.

<ESC>Wx			Disable Auto Flush for DTMF input.

<ESC>WZ			Hang up when speech has ended.

<ESC>In			Flash hook-switch; flash-time = n = 1 to F.

<ESC>in	  	#	Set DTMF string-mode input-terminator.

<ESC>Jn	  	1	Set number of rings for auto answer; n = 1 to F.

<ESC>Kn	  	5	Set dial-tone time-out; n = 1 to F.

<ESC>Ln			Initiate DTMF time-out; n = 0 to F.

4. STANDARD ACCENT TEXT PROCESSOR.

Accent text processor is standard for all Accent models.

A set of legal ASCII characters is defined for Accent's text processor.

Illegal characters will be ignored and not pronounced.

The legal characters are here listed:

!"#$%&'()*+,-./0123456789:;<=>?@

ABCDEFGHIJKLMNOPQRSTUVWXYZ[\]^_`

abcdefghijklmnopqrstuvwxyz{|}~

plus <SP>, <CR>, <LF>, and <HT>.

The Accent text processor employs a highly sophisticated

text-to-speech algorithm.  Text may be input with or

without capital letters (unless otherwise specified, as for

the abbreviations). The key to, and the only requirement

for, attaining natural, intelligible speech with Accent is

that the input text must be properly punctuated as in

normal writing.  Hyphens should not be omitted in

hyphenated words.  Pauses are automatically inserted into

speech by punctuation marks and rules.  If longer pause

is required, consecutive sentence ending punctuation

marks (.:;!?) may be used.  The following Sections describe

each pronunciation rule in detail.

4.1. PRONUNCIATION RULES.

Punctuation marks will not be spoken by Accent in

normal text mode unless the option is enabled by the

command described in Section 3.2.3.4.  To insure natural

speech from Accent, any word (or special sign) with

punctuation mark following it may have only one or no

Space character, <SP>, between the word and the

punctuation mark.  Each ASCII character which Accent

considers to be a mark of punctuation is here listed :

SIGN		TYPE			SPOKEN

<SP>		Word end		Space

,		Phrase end		Comma

(		Phrase begin		Opening Parenthesis

[		Phrase begin		Opening Bracket

{		Phrase begin		Opening Brace

)		Phrase end		Closing Parenthesis

]		Phrase end		Closing Bracket

}		Phrase end		Closing Brace

"		Phrase begin/end	Quotation Mark

.		Sentence end		Period

;		Sentence end		Semicolon

:		Sentence end		Colon

?		Sentence end		Question Mark

!		Sentence end		Exclamation Point

The <`> (grave accent) character is used to mark a stress

for its following syllable in a word.  It may be placed

anywhere within a word which is not an Arabic number,

an abbreviation, or a special sign.  ASCII characters which

Accent considers to be special signs are listed with their

pronunciations as follows:

SIGN		SPOKEN

&		And

*		Asterisk

@		At

|		Bar

=		Equals

_		Underline

>		Greater than

<		Less than

#		Number

%		Percent

/		Slash

\		Back Slash

+		Plus

$		Dollar (also see Section 4.1.3)

-		(See Section 3.2.2.5 through 3.2.2.7)

^		Caret (spell mode only)

~		Tilde (spell mode only)

'		Apostrophe (spell mode only)

`		Grave accent (spell mode only)

<CR>		Carriage return (spell mode only)

<LF>		Line feed (spell mode only)

<HT>		Tab (spell mode only)

<SP>		Space (spell mode only)

4.1.1. ABBREVIATIONS.

An abbreviation may be input without a period following

it unless it is at a sentence end or otherwise noted.

A capitalized initial letter (the first letter) may not be

required unless otherwise noted.  Accent's abbreviations

list includes the following :

APR.	APRIL (1)		MG.		MILLIGRAMS

APT.	APARTMENT (2)		MGS.		MILLIGRAMS

AUG.	AUGUST			MI.		MICHIGAN

AV.	AVENUE			ML.		MILLILITERS

AVE.	AVENUE			MM.		MILLIMETERS

BLVD.	BOULEVARD		MR.		MISTER

BLDG.	BUILDING		MRS.		MISSES

CA.	CALIFORNIA		MS.		MIZ (2)

CM.	CENTIMETERS		MON.		MONDAY

CMS.	CENTIMETERS		MT.		MOUNT

CO.	COMPANY (2)		NOV.		NOVEMBER

CT.	COURT			NY.		NEW YORK

CU.	CUBIC			OCT.		OCTOBER

DEC.	DECEMBER		OZ.		OUNCES

DEPT.	DEPARTMENT		OZS.		OUNCES

DR	DRIVE			PL.		PLACE

DR.	DOCTOR (2)		PP.		PAGE

EA.	EACH			PPD.		POST PAID

ETC.	ET CETERA		PRES.		PRESIDENT

EXT.	EXTENSION		RD.		ROAD

FEB.	FEBRUARY		REP.		REPRESENTATIVE

FIG.	FIGURE (1)		REV.		REVEREND

FRI.	FRIDAY			SAT.		SATURDAY (2)

FT	FEET			SEC.		SECONDS

FT.	FORT (2)		SECS.		SECONDS

GEN.	GENERAL			SEN.		SENATOR

GM.	GRAMS (1)		SEP.		SEPTEMBER

GOV.	GOVERNOR		SEPT.		SEPTEMBER

HQ.	HEADQUARTERS		SQ.		SQUARE

HR.	HOURS			SR.		SENIOR

HRS.	HOURS			ST		STREET

HZ.	HERTZ			ST.		SAINT (2)

INC.	INCORPORATED		SUN.		SUNDAY (2)

IL.	ILLINOIS		TX.		TEXAS

JAN.	JANUARY (2)		TUE.		TUESDAY

JUL.	JULY			TUES.		TUESDAY

JUN.	JUNE			THU.		THURSDAY

JR.	JUNIOR			THUR.		THURSDAY

KG.	KILOGRAM		THURS.		THURSDAY

KGS.	KILOGRAMS		UNIV.		UNIVERSITY

KM.	KILOMETERS		VOL.		VOLUME

LB.	POUNDS			VS.		VERSUS

LBS.	POUNDS			WED.		WEDNESDAY (2)

LTD.	LIMITED			YD.		YARDS

MAR.	MARCH (2)		YDS.		YARDS

Note (1): Must have an ending period.

Note (2): Must have a capitalized initial letter and an ending period.

Besides the preceding abbreviations, Accent includes hundreds of

other commonly used abbreviations which are normally "spelled out"

(such as EIA, CCITT, CIA, USA, USSR, SOS, etc.) when pronounced.

Such abbreviations are not listed here.  Note: if there is no vowel in an

abbreviated word, it will automatically be spelled out by Accent.

4.1.2. ALTERNATIVE PRONUNCIATIONS.

A special feature of Accent's text processor is that

grammatical rules are applied to words with two

pronunciations, and adjustments are automatic.  If this

feature is not desired (such as in single-word

pronunciation), a <^> immediately following an

ambiguous word will elicit that word's alternative pronunciation.

4.1.3. MONETARY $ AMOUNTS.

The following examples show different ways of specifying

monetary amounts, which may be spoken as high as trillion(s):

Input :	$1

Speech:	One dollar

Input :	$235

Speech:	Two hundred thirty-five dollars

Input :	$.10

Speech:	Ten cents

Input :	$0.10

Speech:	Zero dollar and ten cents

Input :	$1.234

Speech:	One point two three four dollars

Input :	$1.26

Speech:	One dollar and twenty-six cents

Input :	$6723

Speech:	Six seven two three dollars

Input :	$30,456.17

Speech:	Thirty thousand four hundred fifty-six dollars and seventeen cents

4.1.4. NUMERALS.

Any number having five or more digits without a comma

inserted is pronounced digit by digit.  Four-digit numbers

without leading zero(s) will be pronounced as two pairs

of two-digit numbers.  If a number has a comma inserted

at every third digit, the following counting units are

available: Hundred, Thousand, Million, Billion, and Trillion.

For any number with a decimal point, a leading

digit must be present, otherwise it will be treated as a period.

Following are examples of how Accent pronounces numerals:

Input :	16

Speech:	Sixteen

Input :	1986

Speech:	Nineteen eighty-six

Input :	168

Speech:	One six eight

Input :	0304

Speech:	Zero three zero four

Input :	1,663

Speech:	One thousand six hundred sixty-three

Input :	166-1688-K8

Speech:	One six six, one six eight eight, K eight

Input :	0.123

Speech:	Zero point one two three

Input :	(408)922-0855

Speech:	Four zero eight, nine two two, zero eight five five

Input :	#123-45-6789

Speech: Number one two three, four five, six seven eight nine

4.1.5. TIME OF DAY.

Input :	12:45

Speech: Twelve: forty-five

Input :	7:07

Speech:	Seven: zero seven

4.1.6. WORDS WITH NO VOWEL.

If a word has no vowel letter, it can not be pronounced,

and Accent will automatically spell it out.

The vowel letters are A E I O U and Y.

Examples of such words are PM, BC, KWPP, etc.

4.2. PHONEME INPUT.

The beginning and ending of a phoneme input string to

Accent must be marked by a <~> character.

Immediately following the beginning mark is a </> character.

In a phoneme input string sequence, each phoneme is

separated by the </> character, and the sequence must be

ended by a </> character.

The Space, Comma, and Period punctuation marks can

also be embedded in a phoneme sequence.

No word should exceed 32 phonemes in length.  The standard phoneme

symbols used for Accent products are listed in TABLE-2.

The following shows an example of the phoneme input:

Text:		This is a test.

Phoneme:	~/DH/X/I/S/ /I/Z/ /UU/ /T/'/E/S/T/./~

or	This ~/I/Z/~ a ~/T/'/E/S/T/./~

4.2.1. TABLE-2 (Accent Phoneme List).

The capitalized alphabets in the following EXAMPLES words represent

the pronunciation for the respective phonemes.

PHONEMES	EXAMPLES

Vowels:

IY		mEEt

II		sillY,marY,beautY

I		sIx,bEfore

IE		bEfore

E		bElt

EA		dAd

AE		After

UH		lOve

UU		About (Schwa sound)

A		gOt

AA		whAt

O		stOre

OW		Office

U		lOOk,cOUld

OO		cartOOn

Diphthongs:

ER		bIRd

EI		dAY

OU		bOAt

AI		bUY

AU		hOUse

OI		bOY

IU		mUsic,nEW

Consonants:

M		More

N		NiNe

NG		siNGiNG

Y		Year

W		Water

R		Rug,fiRing

RU		Roof (Used before phonemes OO and U)

L		Look

K		Kit

G		taG

P		Put

B		Buy

T		siT

D		DaD

CH		CHurCH

J		Just

F		aFter

V		loVe

S		Sit

Z		Zero

SH		diSH

ZH		pleaSure

TH		wiTH

DH		THere

H		Hat

TX		buTTer

DX		laDDer

'		~/T/'/E/S/T/~ (Word primary stress mark (TEST))

X		~/X/A/R/~ (Word secondary stress mark (ARE))

Q		~/F/'/AI/R/Q/A/R/M/~ (Morpheme boundary mark (FIREARM))

5. STANDARD ACCENT DCD UTILITY.

The DCD utility is a development software which runs on

an IBM PC for generating the user's own customized

dictionary as DCD files.  If needed, the DCD file can be

uploaded from the PC to any host and then downloaded

to Accent by the COPY Command appropriate to that

host's Operating System.  All Accent models have a

built-in, 8k-byte memory block which can hold

approximately 500 10-character words.  Once the DCD is

downloaded, Accent will always search through the DCD

first for any given word during the text-to-speech processing.

5.1. DCD SOFTWARE OPERATION.

To generate a custom dictionary for Accent, the DCD

development software (the program named DCD.EXE on

the diskette provided) and a DCD source file are needed.

The DCD source file may be created (with any Editor for

straight ASCII characters) under any file name with any file extension.

After the DCD source file has been created, the DCD.EXE

program may then be executed (by typing in "DCD

xxxxxx.xxx <CR>") to generate a Hex file and an absolute

Object file. The Hex file will be used for download. Both

the Hex file and the Object file have the same name as

that of the DCD source file, and the file extensions are listed as follows:

Hex file:	xxxxxx.DLX

Object file:	xxxxxx.OBJ

A sample DCD source file is shown in Section 5.2.4. The followings

are general rules for creating the DCD source file.

(1) Only upper case characters are allowed.

(2) Legal characters for spelling are: ABCDEFGHIJKLMNOPQRSTUVWXYZ

(3) The spelling, phonemes, and part-of-speech are entered

in that order, and are separated only by two commas.

Phonemes should conform to the standard Accent

phoneme input format as shown in Section 4.2.1.

(4) The words containing the same first-letter should not

exceed 255 in number.  That is, each Alphabetic group

should not have more than 255 words.

(5) No Space character is allowed in the DCD source file.

(6) No empty line is allowed in the DCD source file.

During the generation of the DCD, the <.> will be

prompted to indicate the word count.  Each <.> prompt

indicates that ten words have been processed.

After the DCD Hex file and Object file have been

generated, the memory size (in bytes) of the DCD and the

error message (if any) will be shown on the screen.  The

user should check to assure that the memory size does not

exceed 8k bytes.

To download the DCD to Accent, use the host computer's

COPY Command to copy the Hex file to Accent through

the communication link.  The device driver or the RS-232C

port.  Since each time a DCD file is downloaded to

Accent, it overwrites the old DCD that was in Accent,

multiple DCD applications may be implemented easily.

To totally disable Accent's DCD search after a DCD file

has been downloaded, simply download the DUMMY.DLX

file to Accent.  The DUMMY.DLX file is in the

Demo/Utility diskette provided.

5.2. RULES FOR BUILDING THE CUSTOM DICTIONARY.

Any word in the DCD source file must contain only the

capitalized Alphabetic letters.  Because the Accent Text

Processor already processes numerous prefixes and

suffixes, the DCD user will save memory by entering only

the root words, and thereafter testing for any of their

derivatives which he wishes Accent to speak.

The correct part-of-speech for each entry word is required

for the text processor to correctly parse the

phrase/sentence and generate more-natural sounding speech.

5.2.1. THE ROOT WORD.

Any specific word which is entered into the DCD will

always be pronounced as phonemically entered.  In

addition, that word may combine with various of its

prefixes and/or suffixes to form yet other words which

will also be pronounced correctly.

Nevertheless, due to the complexity of the text processing

rules, internal rule conflicts may result in skewed

pronunciation of words derivative of the root word

entered. Therefore it is suggested that when any root

word is entered into the DCD, all related words for which

the user requires accurate, correct pronunciation be tested

as soon as the DCD has been downloaded. Should a

related word or words be mispronounced (which might

happen when, for example, the word has both a prefix

and multiple suffixes), the user should simply enter the

mispronounced word or words into the DCD. The Accent

text processor supports the following prefixes and

suffixes:

PREFIXES:

ab-	 down-	mal-	poly-	sur-

anti-	 en-	micro-	post-	sus-

auto-	 ex-	mis-	pre-	tele-

be-	 grand-	mono-	pro-	trans-

bi-	 half-	multi-	re-	tri-

co-	 im-	non-	right-	ultra-

com-	 in-	out-	semi-	un-

con-	 inter-	over-	short-	under-

counter- ir-	para-	sub-	up-

de-	 left-	per-	super-	vice-

dis-

SUFFIXES:

-ic	-ible	-ial	-ern	-est

-d	-wise	-ual	-ship	-ist

-ed	-ate	-ful	-er	-ency

-hood	-uate	-ism	-or	-ly

-ance	-ure	-n	-s	-ally

-ence	-ive	-man	-es	-ery

-side	-ize	-en	-less	-ory

-age	-ing	-ion	-ness	-ty

-like	-ish	-ation	-ous	-ity

-able	-al	-son	-ment

5.2.2. THE PART-OF-SPEECH.

Those parts-of-speech which are to be assigned by the

DCD user as he enters vocabulary items, and which are

supported by the Accent text processor, are listed as follows:

PART-OF-SPEECH		REPRESENTATION

1		Noun.

2		Verb.

3		Adjective.

4		Adverb.

7		Present Participle.

8		Past Participle.

13		Interjection.

19		Verb or Noun (with same pronunciations).

62		Verb or Noun/Adjective having -ate as

suffix (with different pronunciations).

63		Verb or Noun (with different pronunciations).

5.2.3. WORDS WITH TWO PRONUNCIATIONS.

If a word has two pronunciations with 63 as its

part-of-speech, both words should be entered into the

DCD adjacently.  The first word should always have the

verb pronunciation, with the part-of-speech as 63.  If a

word has two pronunciations with 62 as its part-of-speech,

only the verb needs to be entered into the DCD.  The

Accent text processor will search for the correct

pronunciation when parsing the sentence.

5.2.4. A SAMPLE DCD SOURCE FILE.

SUPERMAN,S/'/OO/P/ER/M/EA/N/,1

COMBO,K/'/A/M/B/OU/,1

ESTEEM,UU/S/T/'/IY/M/,19

GRADUATE,G/R/'/EA/J/U/EI/T/,62

EAT,'/IY/T/,2

EATING,'/IY/T/I/NG/,7

EATEN,'/IY/T/UU/N/,8

NICELY,N/'/AI/S/L/II/,4

RECORD,R/I/K/'/O/R/D/,63

RECORD,R/'/E/K/ER/D/,1

FINE,F/'/AI/N/,3

5.2.5. ERROR MESSAGES.

0	Phoneme name too long, or  missing the phoneme separator </>.

1	Phoneme name not valid.

4	Primary or secondary stress not followed by the </>.

5	Part of speech not valid.

6	Primary or secondary stress disallowed on non-vowel phoneme.

7	DCD memory size exceeds 32k-bytes.

6. PROGRAMMING ACCENT.

The following are some general rules and hints for

programming Accent in various applications:

(1) Once a specific voice (average pitch, speech rate, voice

characteristic, sentence intonation, etc.) has been chosen

for output, the escape sequences along with system option

parameters and DCD (if any) may be sent to Accent as

part of power-up initialization.  The initialization for the

Screen-Review applications is recommended to be

<ESC>=F <ESC>=B <ESC>OA <ESC>M4 <ESC>=M <ESC>N1.  Other

parameters may be user tailored or selected.

(2) It is important that all text be punctuated normally,

and that compound words be properly hyphenated.  Any

word not pronounced satisfactorily by Accent may be

corrected by using the DCD, phoneme input, or intentional misspelling.

(3) Speech is often better perceived when the pitch is set

at a lower level.  At low pitch, the speech rate may thus

be set at a level higher than usual without losing

intelligibility.  From experiments and listening tests,

<ESC>P3 <ESC>V4 settings give an average-low male

voice with very intelligible, natural sounding speech for

regular text at the default speech rate.

(4) Text and escape-sequence commands for any

application may be entered and tested first using the

DEMO.BAS (or DEMO232.BAS) program provided or any

suitable Editor.  After which the text and/or the escape

sequences may be saved as a file for further use or editing.

(5) The <ESC>=F and <ESC>=B commands will affect

sentence intonation.  They are not recommended for

general applications, neither for Screen-Review

applications when slow speech-rate is used for proof-reading.

(6) If, for the first-time synthetic speech listener, the word

boundaries in a sentence are not perceived distinctly

enough, the Set Space Pause Time command may be used

to increase the pause between words.  Medium short,

descriptive type of sentences always give good

comprehension to the listeners, especially when the

listeners are expecting certain prompts.

(7) For applications requiring different voices (by using

speech option commands) as well as using the Flush

command frequently, always try to Flush Accent first then

send the speech option commands.  Enable and use the

Control-X character as the Flush command.

(8) For applications requiring frequent, constant Flush of

speech, unless the application program can assure that

Accent's input text buffer is never filled to full by using

the Status Request command in Section 3.2.4.2, the

following recommendations should be taken into

consideration:  If Accent's device driver is used for host

interface, the LPT port interface should be adopted

because it is re-entrant, speech may be flushed anytime

with a Flush command (LPT's Initialize command).  If the

RS-232C is used for host interface, the Xon/Xoff protocol

should be used because even when Accent has sent Xoff

to the host, it still has 128-byte free space for further text

input or Flush command.

(9) This is for Accent-PC only.  In applications requiring a

multi-host configuration, if the text is both to be spoken

and to be sent to the auxiliary port, it is the user's

responsibility to guarantee that the auxiliary port (the

RS-232C port) has an uninterrupted receiving capability

so that Accent does not get hung up waiting for the auxiliary

port ready.

(10) The typical and most frequent application for

Accent-ST is remote access to a database through the

telephone line.  In these applications, Accent-ST acts as a

link between the telephone and a computer.  The

application software's task is to instruct Accent-ST to do

the following:

<A>	Answer the phone after a predetermined number of rings.

<B>	Respond with a greeting and/or menu.

<C>	Accept input from the telephone keypad, and then send the messages

to the host computer.

<D>	Convert ASCII text, received from host, into speech.

<E>	Continue looping through a menu of choices.

<F>	Hang up on a predetermined time-out or exit signal.

<H>	Wait for the next call.

(11) For Accent-ST, an efficient telephone application

program is written in small segments or modules.

Each module corresponds to a particular function.

A typical program with Accent-ST's Telephone Prompt option

enabled can be divided into the following four segments:

<A>	Initialization:

<ESC>=V		;Xon/Xoff protocol, if required.

<ESC>V4<ESC>P3	;Select a voice as desired.

<ESC>WA		;Auto answer (default).

<ESC>J2		;2 rings for auto answer.

<ESC>WX		;Auto flush at DTMF input (default).

<ESC>Wg		;Set 0.8dB gain pad.

.......		;Other desired setups.

<B>	Answer phone:

<ESC>Wo		;On hook to clear all flags if the last call is not hung up.

<ESC>=Z		;IF error, flush output buffer.

<ESC>J1		;Busy hour! One-ring for auto-answer.

.......		;Other desired setups.

<C>	Conversation:

<ESC>Wi		;Enable the keypad in string mode for early DTMF entry.

<ESC>L9		;Initiate the DTMF time out to 9 seconds.

Hello		;Greeting/Password/Menu.

Text/Menu	;Conversation loop.

.......		;Go to Hang up if caller exit, or

.......		;re-Initiate DTMF if DTMF time-out.

<ESC>=x		;Flush at valid DTMF input if the Auto flush is not enabled.

<D>	Hang up:

<ESC>Wj<ESC>=x	;Disable the keypad; Flush.

<ESC>=Z		;Flush Accent-ST's output buffer.

Good-day	;End message.

<ESC>WZ		;Auto-hangup when speech ends.

;Go to Answer Phone for the next call.

7. SPECIFICATIONS.

The following sections describe the specifications of each Accent model.

7.1. ACCENT-PC SPECIFICATIONS.

Parallel Bus: IBM Personal Computer.

Slot: One full-length slot in IBM PC, PC/XT, PC/AT, or Compatibles.

Address: switch-selectable (4 I/O Addresses, Default 2A8 to 2AB Hex).

Serial I/O: RS-232C as DCE device.

Power:

500 milli-Amp Typical @ +5V.

600 milli-Amp Maximum @ +5V.

40 milli-Amp Typical @ +12V.

80 milli-Amp Maximum @ +12V.

10 milli-Amp Typical @ -12V.

20 milli-Amp Maximum @ -12V.

Load: 1 TTL load/bus line Maximum.

Audio Output Power: 0.5W Maximum, 8-Ohm or 4-Ohm Speaker.

Physical Dimensions: 4.2 inch x 10.04 inch x 0.57 inch (Max component height).

7.2. ACCENT-mini SPECIFICATIONS (Not for Accent-1200).

Parallel Bus: IBM Personal Computer.

Slot: One half-length slot in IBM PC, PC/XT, PC/AT, or Compatibles.

Address: 3EE to 3EF or 2EE to 2EF Hex, jumper-selectable (2 I/O Addresses;

Default 3EE to 3EF Hex).

Interrupt: IRQ2(PC/XT)/IRQ9(PC/AT), IRQ3, IRQ4, or IRQ7;

Jumper-selectable (Default IRQ2/IRQ9).

Power:

40 milli-Amp Typical @ + 5V.

80 milli-Amp Maximum @ +5V.

40 milli-Amp Typical @ +12V.

80 milli-Amp Maximum @ +12V.

Load: 1 TTL load/bus line Maximum.

Audio Output Power: 0.5W Maximum, 8-Ohm or 4-Ohm Speaker.

Physical Dimensions: 4.2 inch x 4.7 inch x 0.5 inch (Max. component height).

DOS Version Required: DOS-2.11 or higher.

Software Device Driver Size: 123K Bytes.

7.3. ACCENT-SA SPECIFICATIONS.

Serial I/O: RS-232C as DCE device.

Power:

450 milli-Amp Typical @ +5V.

550 milli-Amp Maximum @ +5V.

40 milli-Amp Typical @ +12V.

80 milli-Amp Maximum @ +12V.

10 milli-Amp Typical @ -12V.

20 milli-Amp Maximum @ -12V.

Audio Output Power: 0.5W Maximum,8-Ohm or 4-Ohm Speaker.

Physical Dimensions: 5.5 inch x 11.5 inch x 1.6 inch.

Weight: 1.3 lbs. (Not including AC adaptor).

AC Adaptor Input: 110V/60 Hz.

7.4. ACCENT-ST SPECIFICATIONS.

Serial I/O: RS-232C as DCE device.

Power:

460 milli-Amp Typical @ +5V.

560 milli-Amp Maximum @ +5V.

40 milli-Amp Typical @ +12V.

80 milli-Amp Maximum @ +12V.

20 milli-Amp Typical @ -12V.

30 milli-Amp Maximum @ -12V.

Audio Output Power: 0.5W Maximum, 8-Ohm or 4-Ohm Speaker.

Physical Dimensions: 5.5 inch x 11.5 inch x 1.6 inch.

Weight: 2.8 lbs. (Not including AC adaptor).

AC Adaptor Input: 110V/60 Hz.

Telephone Interface (FCC Part 68 Registered):

Connection: CO or PBX; 600 Ohm impedance.

DTMF dialing: 8 digits per second; -3 dBm0.

Pulse dialing: 10 pulse per second; 600 milli-seconds interdigital pause.

Ring Detect: 38 Vrms minimum.

Ringer Equivalence: 0.0B.

FEDERAL COMMUNICATION COMMISSION RADIO FREQUENCY INTERFERENCE STATEMENT.

(Applies to Accent-PC, Accent-SA, and Accent-ST).

WARNING : This equipment complies with the limits for a

Class B computing device in accordance with the

specifications in Subpart J of Part 15 of the FCC rules.

These specifications are designed to  minimize radio

frequency interference in a residential installation.

However, there is no guarantee that radio or television

interference will not occur in any particular

installation.  If this equipment does cause interference

to radio or television reception, which can be determined

by turning the equipment off and on when the radio or

television is on, the user is encouraged to try to correct

the interference by one or more of the following measures:

(1) Reorient the radio or television receiving antenna,

(2) Relocate the equipment with respect to the receiver,

(3) Move the equipment away from the receiver,

(4) Plug the equipment into a different outlet so that the

equipment and the receiver are on different circuits.

If necessary, the user should consult the dealer or an

experienced radio/television technician for additional

suggestions.  The user may find the following booklet

prepared by the Federal Communications Commission helpful:

"How to Identify and Resolve Radio/TV Interference Problems".

This booklet is available from the U.S. Government

Printing Office, Washington, DC 20402 by ordering stock

number 004-000-00345-4.

FCC REQUIREMENTS FOR ACCENT-ST.

This equipment complies with Part 68 of the FCC rules. On the

bottom of this equipment is a label that contains, among other

information, the FCC Registration Number and Ringer Equivalence

Number (REN) for this equipment.  If requested, this information

must be given to the telephone company.

The REN is useful to determine the quantity of devices you may

connect to your telephone line and still have all of those devices

ring when your telephone number is called.  In most, but not all

areas, the sum of the REN's of all devices connected to one line

should not exceed five (5.0). To be certain of the number of

devices you may connect to your line, as determined by the REN, you

should contact your local telephone company to determine the

maximum REN for your calling area.

If your telephone equipment causes harm to the telephone network,

the Telephone Company may discontinue your service temporarily.  If

possible, they will notify you in advance.  But if advance notice

isn't practical, you will be notified as soon as possible.  You

will be informed of your right to file a complaint with the FCC.

Your telephone company may make changes in it's facilities,

equipment, operations or procedures that could affect the proper

functioning of your equipment.  If they do, you will be notified in

advance to give you an opportunity to maintain uninterrupted

telephone service.

If you experience trouble with this telephone equipment, please

contact Aicom Corporation at (408)922-0855 for information on

obtaining service or repairs.  The telephone company may ask that

you disconnect this equipment from the network until the problem

has been corrected or until you are sure that the equipment is not

malfunctioning.

This equipment may not be used on coin service provided by the

telephone company.  Connection to party lines is subject to state

tariffs.

LIMITED WARRANTY.

We warrant, for a period of ninety (90) days from the date of

shipment to the original purchaser, that all Accent models are free

from defects in workmanship and materials under normal use and

service, provided the equipment is used in accordance with the

user's manual.  This Warranty shall be void if the equipment is

modified, the equipment's case (if any) is opened, or as mentioned

elsewhere in the user's manual.  If any failure to conform to this

Warranty appears during the ninety (90) day period, we will correct

the defect by either suitable repair or replacement, at our option

and at our expense, if you return the product, transportation

prepaid, to Aicom Corporation, 2375 Zanker Road, Suite 205, San

Jose, Ca. 95131. We will pay the cost of returning the repaired or

replaced equipment to you.  The equipment must be returned with

proof of purchase in its original (or other adequate) packaging to

the point of purchase or, by prior arrangement with Aicom

Corporation, to Aicom Corporation.

Aicom Corporation reserves the ultimate authority to determine

what constitutes warranty repair.  If it is determined that the

product is not under Warranty, it will be repaired using our

standard rates for parts and labor.  Aicom Corporation advises you

to make arrangements with Aicom Corporation prior to your returning

of any equipment for repair.

The above warranty is the only warranty authorized by Aicom Corporation.

THE ABOVE WARRANTY STATES THE PURCHASER'S EXCLUSIVE REMEDY FOR ANY

BREACH OF AICOM CORPORATION'S WARRANTY AND FOR ANY CLAIM, WHETHER

SOUNDING IN CONTRACT, TORT OR NEGLIGENCE, FOR LOSS OR INJURY CAUSED

BY THE SALE OF THIS PRODUCT.  WITHOUT LIMITING THE GENERALITY OF

THE FOREGOING, AICOM CORPORATION SHALL IN NO EVENT BE RESPONSIBLE

FOR ANY LOSS OF BUSINESS OR PROFIT, DOWNTIME OR DELAY, LABOR,

REPAIR, OR MATERIAL COSTS, INJURY TO PERSON OR PROPERTY OR ANY

SIMILAR OR DISSIMILAR CONSEQUENTIAL LOSS OR DAMAGE INCURRED BY

PURCHASER, EVEN IF AICOM CORPORATION HAS BEEN ADVISED OF THE

POSSIBILITY OF SUCH LOSSES OR DAMAGES.

Some States do not allow the exclusion or limitation of incidental

or consequential damages or limitations on how long an implied

warranty lasts, so the above limitation or exclusion may not apply to you.

This Warranty shall not be applicable to the extent that any

provision of this Warranty is prohibited by any Federal, State or

Municipal Law which cannot be preempted.  This Warranty gives you

specific legal rights, and you may also have other rights which

vary from State to State.

###############################################################################################################