Z88 CONNECTIONS TO HAYES MODEM

------------------------------

1.  Official Approach from Cambridge

The following set-up is recommended by Cambridge North America (uploaded

by Frank Grady on 1/6/89).

"Here's the official pinout for Z88 to any modem, courtesy of Cambridge --

new tech bulletin received yesterday via phone.

9-pin           25-pin

male to         male to

Z88             Modem:     Sysop's Note:  I've now tried this

-------         -------      =============  cable and it is the

2 (TxD) --------> 2                       best I've found.  I

3 (RxD) --------> 3                       belatedly add my strong

7 (Gnd) --------> 7                       endorsement!!!

5 (Cts) -------->

8 (Dcd) --------> 20 (all wired to 20)

9 (+5V) -------->

Cambridge offers a 1-foot cable to this spec for $19.95, but if  you're

handy you can make your own.  I'm happy to report that it works just fine.

-- Frank Grady, Detroit 1/5/89

2. Connections Recommended by Wordmongers

The following connections are recommended by Clive (Selvage?) at Wordmongers

(telecon of 1/4/89 at 0530 pst).  They sell comm software for the Z88, and

use the Worldport 1200 modem, so they should know what they are talking

about!

Z88                 Modem

(DB-9)              (DB-25)

----                -----

2 ------------------ 2

3 ------------------ 3

4 --| 4,5

5 --| shorted

7 ------------------ 7

8 --| 8,9

9 --| shorted

Clive added that all the modem cables they make have only three wires, per

the above diagram.

They also added the following re the switches in the Worldport 1200  (note

that these are considerably different than the default setting I use with my

portable PC and my Model 100):

SW      WM Recomm       Default      Function with WM Recommendation

--      ---------       -------    -----------------------------------

1          OFF            OFF      Bell 103/212A or CCITT (ON in U.K.)

2          ON             ON       Autoanswer disabled

3          ON             OFF      Receive sense (vs. DTR sense)

4          ON             OFF      DTR ignored (vs. Enabled)

NOW -- even with these settings, and cable they specified, my Worldport

would not give result codes.  Worse yet, I could not even log on with manual

dialing as I could before.  Therefore, functionality with my portable modem

deteriorated.  The Hayes did work, but only after disabling the DTR sense

function.

Phil Wheeler

Torrance, CA

1/20/89 (Vers. 1.06)