%OP%DEMiscellaneous useful routines

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,12,80%%C%%H1%%H2%M i s c e l l a n e o u s   u s e f u l   r o u t i n e s%H2%%H1%

This section explains how to manipulate system parameters, i.e. from the Panel,

the Printer Editor and other system resources. The rest of this section

displays useful routines which does not conveniently fit in any of the other

sections.

%H2%System resource parameters%H2%

These parameters are divided into 5 groups. Their mnemonic identifier is a

16bit identifier code where the high byte of the code identifies the related

group:

$80xx

$83xx

$86xx

$89xx

$8Cxx

%H2%Enquiries%H2%

The OS_NQ call is used to read the system parameters, including information on

the screen, memory, available streams and settings from the Panel and Printer

Editor. A call is made by loading the BC register with a reason code (the

system parameter mnemonic) and the other registers with values specific to that

reason code. Register changes and output specifications are different for each

reason code.

Here is the call specification for OS_NQ:

OS_NQ, enquire

RST 20H, DEFB $66

IN:

OUT if call succeeded:

Out if call failed:

%P0%

%H2%Window Management%H2%

The following enquiry calls are for fetching low level window information:

NQ_WBOX (BC = $8300), return window information

NQ_WCUR (BC = $8303), return cursort information

NQ_RDS (BC = $8306), read text from the screen

The above window enquiry calls are described in detail in the OS_NQ call

specification.

%H2%Process Management%H2%

The following codes return system handles in IX:

NQ_Ain ($8600)

NQ_Khn ($8603)

NQ_Shn ($8606)

NQ_Phn ($8609)

NQ_Nhn ($860C)

NQ_Wai ($860F)

NQ_Com ($8612)

NQ_Ihn ($8615)

NQ_Ohn ($8618)

NQ_Rhn ($861B)

%H2%Memory Management%H2%

NQ_Mfs ($8900)

NQ_Slt ($8903)

%H2%Director and CLI%H2%

These enquiries return extended pointers of a null-terminated string in BHL:

NQ_Dev ($8C00)

NQ_Dir ($8C03)

NQ_Fnm ($8C06)

%H2%Director and CLI handles%H2%

The following calls return system handles in IX

NQ_Dmh ($8C09)

NQ_Inp ($8C0C)

NQ_Out ($8C0F)

NQ_Prt ($8C12)

NQ_Tin ($8C15)

NQ_Tot ($8C18)

NQ_Tpr ($8C1B)

NQ_Chn ($8C1E)

%P0%

%H2%Specifies%H2%

The OS_SP call is a companion to the OS_NQ and is used to set system

parameters, notably the Panel and the PrinterEd settings. A call is made by

loading register pair BC with a reason code and the other registers with values

specific to that reason code. Register changes and output specifications are

different for each reason code. The OS_SP interface call is as follows:

OS_SP, specify

RST 20H, DEFB $69

IN:

OUT, if call succeeded:

OUT, if call failed:

%H2%System Values%H2%

%H4%Symbol (reason code)%H4%

PA_Gfi ($8000)

PA_Mct ($8001)

PA_Rep ($8002)

PA_Kcl ($8003)

PA_Snd ($8004)

PA_Bad ($8005)

%H2%Application values%H2%

%H4%Symbol (reason code)%H4%

PA_Iov ($8010)

PA_Dat ($8011)

PA_Map ($8012)

PA_Msz ($8013)

PA_Dir ($8014)

PA_Dev ($8015)

%H2%Serial port values%H2%

%H4%Symbol (reason code)%H4%

PA_Txb ($8016)

PA_Rxb ($8017)

PA_Xon ($8018)

PA_Par ($8019)

%P0%

%H2%Printer Editor values (highlights & translation tables)%H2%

%H4%Symbol (reason code)%H4%

PA_Ptr ($8020)

PA_Alf ($8021)

PA_Pon ($8022)

PA_Pof ($8023)

PA_Eop ($8024)

PA_Mip ($8025)

PA_Mis ($8026)

PA_Mio ($8027)

The format of each highlight sequence is as follows:

PA_On<n>

PA_On<n>+1

PA_On<n>+2

PA_On<n>+3

The printer editor has a facility for overprinting characters, which is most

often used to implement underlining on certain printers, by printing a

character followed by a backspace and an underline character. The user enter a

? (query), to represent the character being printed, in the ON sequence (the

OFF sequence being blank), and this sequence is output for every character with

the current character being substituted for the query. Internally the Printer

Editor usually represents the current character as an $FF (it will show up like

this in the Printer Editor, if a string is installed, updated, and the Printer

Editor is KILL'ed and re-entered). However, because it might be necessary to

use an $FF in your overprinting sequence, it is possible to change the internal

representation of the current character to some other value. If you want to use

an overprinting sequense you must set up PA_SP<n> yourself and use your value

in the ON string to represent the current character. To implement backspace

underlining you might set the ON sequence and special character like this:

PA_ON1

PA_SP1

%H2%Highlight bases%H2%

PA_On1 ($8028)

PA_On2 ($802C)

PA_On3 ($8030)

PA_On4 ($8034)

PA_On5 ($8038)

PA_On6 ($803C)

PA_On7 ($8040)

PA_On8 ($8044)

%H2%Translations%H2%

The format of each translation code sequence is as follows:

%H4%Symbol (reason code)%H4%

PA_Fr<n> (base+0)

PA_To<n> (base+1)

%H4%Symbol (reason code)%H4%

PA_Tr1 ($8048)

PA_Tr2 ($804A)

PA_Tr3 ($804C)

PA_Tr4 ($804E)

PA_Tr5 ($8050)

PA_Tr6 ($8052)

PA_Tr7 ($8054)

PA_Tr8 ($8056)

PA_Tr9 ($8058)

PA_Fr<n> will normally be one character long. If there are more characters in

this sequence then the first character is replaced by the translation string,

nut the others are printed as well.

International versions of the machine have an extra page of 28 translations

(using the <>ISO command). The reason codes for these translations start at

$8080 up to $80B6.

The settings do not come into effect until OS_SP is called with the reason

code PA_GFI and A = 0, which installs both the Panel and Printer Editor values.

No error checking is done, so make sure the values are valid.

In addition to the Panel and Printer Editor, the following parameters can be

set. They expect HL to point to a null-terminated string:

SP_Dev ($8C00)

SP_Dir ($8C03)

SP_Fnm ($8C06)

%P0%

%H2%Interrupts%H2%

Two system calls are available to enable and disable interrupts. It is very

important to use these calls and not use the Z80 interrupt instructions because

of a hardware bug in the Z80, which allows for interrupts to occur while

reading the interrupt status. Note that disabling interrupts should be avoided

if possible and that NMI (Non Maskable Interrupts) will still occur. If a HALT

is executed when interrupts are disabled the system will stop. HALT should not

be used anyway, nor should EI, DI or IM <n>.

OZ_DI, disable interrupts

CALL $0051

IN:

OUT:

Registers changed after return:

OZ_EI, restore old interrupt state

CALL $0054

IN:

OUT:

Registers changed after return:

Thus one might do:

and be assured for the interrupt status is preserved.

%P0%

%H2%Save and restore%H2%

The OS_SR call was designed for internal use, but has a three rather handy

features which can be used by applications:

OS_SR

RST 20H, DEFB $6C

IN:

OUT, if call successful:

OUT, if call failed:

SR_SUS ($01)

This is a very powerful call, but should not be over used. Most of the time it

is perfectly possible to regenerate a screen without resorting to this kind of

sledge-hammer approach, requiring around 2K a time. It is probably preferable

to use this call, rather than making your application one which has its screen

saved automatically (as BBC BASIC), if there are only a few situations in your

application where saving the screen is essential. One final point is that it is

essential to check that the call was successful and to try and cope if it

fails, as it is likely to do on an unexpanded heavily used machine. Please

refer to OS_SR reference for parameter call details.

SR_RUS ($02)

Restores the previously saved image and releases the corresponding memory.

Please refer to OS_SR reference for parameter call details.

SR_FUS ($05)

This performs the function of freeing the memory that was used to save a screen

but without actually affecting the screen image. Please refer to OS_SR

reference for parameter call details.

SR_WPD ($03)

SR_RPD ($04)

Please refer to OS_SR reference for parameter call details. Mailboxing is

explained in greater detail in "Application Static Structures".

SR_PWT ($08)

This call displays the page-wait message at the edge of the screen and then

does the equivalent of OS_IN. It is very important that you check for all

pre-emption codes, especially RC_QUIT, when using this call if they are not

coped within an error handler. Please note that a successfull return only

return keyboard output with A = <BACKSPACE> ($08), and otherwise the error

return codes. When output is redirected, page waits are conveniently

surpressed. Please refer to OS_SR reference for parameter call details.

SR_RND ($09)

The return value (random number) only changes when certain operating system

functions occur in between calls to OS_SR. Repeated fetching a random number

may well result in the same value being returned!

%P0%

%H2%Fast code%H2%

Some operations, such as bank switching, can take up quite a lot of processing

time. In order to speed up such tasks the operating system provides a facility

for making certain system calls operate more rapidly. The fast code interface

call provides a fragment of code to perform a particular task and places it an

address supplied by the application. The current version of the Z88 has only

one fast code routine which is to rapidly switch bank bindings.

OS_FC, select fast code (fast bank switching)

RST 20H, DEFB $8A

IN:

OUT:

Registers changed after return:

The OS_FC fast bank switching code has now been established. Subsequent

calls to OS_FC with:

IN:

OUT:

Notes:

The bank switching code can now be accessed at the address supplied in DE(in).

If HL(in) = 0 then use CALL DE(in), else use JP  DE(in).

Bank switching is a fairly simple process of updating a softcopy and writing to

a hardware register. The reason for using the fast-code interface is that on

any future versions of the machine the application will still work even though

the bank switching process may be different. For this reason at least 30 bytes

should be available for the code. Since the actual length is returned, memory

can be reclaimed if necessary. One final point to bear in mind is that you must

not place the fast code in the segment which it is rebinding! Think of cutting

the branch you are sitting on...

%P0%

%CO:B,12,68%

PANEL system parameters (timeout, baud rates, date format...)

Window management parameters.

Process management parameters.

Memory management parameters.

Director/CLI parameters.

BC = reason code

ADEHLIX parameters

Fc = 0

Depends on BC and other arguments

Fc = 1

A = return code:

BC = reason code

HL = pointer to data

A = length (size) of data

Fc = 0

Fc = 1

A = return code:

255,8,95

255

-

A = old I register

Fc = old interrupt status (Fc = 1 disabled; Fc = 0 enabled)

..BCDEHL/IXIY same

AF....../.... different

A = old I register

Fc = old interrupt status (Fc = 1 disabled; Fc = 0 enabled)

-

AFBCDEHL/IXIY same

......../.... different

call OZ_DI

push af

; operations requiring disabled interrupts

pop  af

call OZ_EI

A = reason code:

SR_SUS ($01)

SR_RUS ($02)

SR_WPD ($03)

SR_RPD ($04)

SR_FUS ($05)

SR_CRM ($06)

SR_CIN ($07)

SR_PWT ($08)

SR_RND ($09)

BC = arguments

DE = arguments

HL = arguments

IX = arguments

Fc = 0

returned values depend on A(in)

Fc = 1

A = error code:

A = 1, for fast bank switching

DE = address to copy fast bank code into

HL = 0, for code to terminate with RET

HL <> 0, for code to terminate with JP (HL)

C = segment for bank switching (MS_Sx)

Fc = 0

A = code size

..BCDEHL/IXIY same

AF....../.... different

A = bank to bind to segment specified by OS_FC initialization

AFBCDEHL/IXIY/afbcdehl same

......../..../........ different

OS_FC has only been implemented with the above bank switching

routine.

%CO:C,12,56%

RC_BAD ($04), incorrect reason code or parameters

RC_UNK ($03), unknown request

Application enquiry (system use only)

read keyboard handle (use ":INP" device instead)

read screen handle (use ":SCR" device instead)

read printer indirected handle

read null handle (use ":NUL" device instead)

Who am I? (system use only)

read comms handle (use ":COM" device instead)

read IN handle (use ":INP" device instead)

read OUT handle (use ":OUT" device instead)

read direct printer handle

read free space information (system use only)

read slot type information (system use only)

fetch current device

fetch current directory

fetch current filename match string

fetch Director special memory handle (system use only)

read std. input handle

read std. output handle

read printer stream handle

read input-T handle

read output-T handle

read printer-T stream handle

read comms handle

RC_BAD ($04), incorrect reason code or parameter

RC_UNK ($03), unknown request

%H4%Size%H4%

0

1

1

1

1

1

%H4%Size%H4%

1

1

1

1

12

16

%H4%Size%H4%

2 (or 1)

2 (or 1)

1

1

%H4%Size%H4%

9

1

31 (max)

31 (max)

31 (max)

31 (max)

31 (max)

31 (max)

31 (max)

31 (max)

1

1

%H4%Size%H4%

31 (max)

31 (max)

%H4%Size%H4%

RC_UNK ($03), unknown request

RC_BAD ($04), bad arguments

RC_HAND ($08), bad handle

RC_ROOM ($07), no room

RC_SUSP ($69), with SR_PWT

RC_DRAW ($66), with SR_PWT

RC_QUIT ($67), with SR_PWT

Save user screen

Restore user screen

Free user screen

Write parameter data (mailbox)

Read parameter data (mailbox)

Page wait

Occasionally a random number (system use)

%CO:D,12,44%

%H4%Meaning%H4%

Install changed settings

Timeout in minutes

Repeat rate

Keyclick 'Y' or 'N'

Sound 'Y' or 'N'

Default bad process size in K

%H4%Meaning%H4%

Insert/Overtype 'I' or 'O'

Date format 'E' or 'A' (European, American)

PipeDream map 'Y' or 'N'

Map size in pixels

Default directory

Default device

%H4%Meaning%H4%

Transmit baud rate - binary

Receive baud rate - binary

Xon/Xoff 'Y' or 'N'

Parity 'O', 'E', 'M', 'S' or 'N'

%H4%Meaning%H4%

Printer name

Allow Linefeed, 'Y' or 'N'

Printer On sequence

Printer Off sequence

End of page sequence

HMI prefix sequence

HMI suffix sequence

HMI offset sequence

Highlight ON sequence

Highlight OFF sequence

OFF at CR 'Y' or 'N'

Highlight n special character

eg. ?, BS, "_"

special highlight character

Underline

Bold

Extended sequence

Italics

Subscript

Superscript

Alternate font

User Defined

%H4%Meaning%H4%

Translate from character

Translate to sequence

%H4%Meaning%H4%

Row 1, Entry A

B

C

Row 2, Entry A

B

C

Row 3, Entry A

B

C

Set current device

Set current directory

Set current name match

Save user screen

Restore user screen

Write parameter data (mailbox)

Read parameter data (mailbox)

Free user screen

Remove card (system use only)

Insert card (system use only)

Page wait

Occasionally a random number (system use)

%CO:E,12,32%%CO:F,12,20%